| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 coding for synchronization control messages of multimode operation over IEEE 802.16.1a |  |
| Date Submitted | 2012-05-07 |  |
| Source(s) | Seokjoo Shin Chosun University Won-Ik Kim, Seokki Kim, Eunkyung Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI | E-mail: sjshin@chosun.ac.kr woniks@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16-12-271,” in response to Letter Ballot Recirc #38a on P802.16.1a/D2 |  |
| Abstract | This provides AWD text proposals for ASN.1 coding of AAI-MM-RS-SYN-REQ and AAI-MM-RS-SYN-RSP on multimode operation in IEEE p802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 coding for synchronization control messages of multimode operation over IEEE 802.16.1a

Seokjoo Shin

Chosun University

Won-Ik Kim, Seokki Kim, Eunkyung Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Introduction

This document provides ASN.1 coding of AAI-MM-RS-SYN-REQ/RSP message for HR-Network.

References

[1] IEEE P802.16nTM/D2, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[2] IEEE P802.16.1aTM/D2, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, April 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” April 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, April 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Annex A

...

A.2 MAC control message definitions (normative)

Change Annex A.2 as indicated:

WirelessMAN-Advanced-Air-Interface DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type,

...

}

[Remedy1: Add the following codes below Line# 60, Page#218 in IEEE P802.16.1a/D2.]

MAC-Control-Msg-Type ::= CHOICE {

...

-- Misc

aaiL2Xfer			AAI-L2-XFER,

aaiMsgAck			AAI-MSG-ACK,

aaiResCmd			AAI-RES-CMD,

-- HR Multimode operation

aaiMmRsSynReq			AAI-MM-RS-SYN-REQ,

aaiMmRsSynRsp			AAI-MM-RS-SYN-RSP,

-- HR Multicast message

aaiHrMgInd			AAI-HR-MG-IND,

aaiHrMtInd			AAI-HR-MT-IND,

...

}

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Common type definitions

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

...

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- MAC control messages for HR-Networks

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

...

[Remedy2: Add the following codes below Line# 17, Page#228 in IEEE P802.16.1a/D2.]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode RS Synchronization Request message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MM-RS-SYN-REQ ::=			SEQUENCE {

scanDurationControl			ENUMERATED {

increaseTimeDuration,

decreaseTimeDuration

},

timeIntervalControl			ENUMERATED {

increaseTimeInterval,

decreaseTimeInterval

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode RS Synchronization Response message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

RelaySyncInfo ::=			SEQUENCE {

scanDuration				INTEGER (0..255),

startSuperFrame			SuperframeNumberLSB,

startFrame				INTEGER (0..3),

interleavingInterval			INTEGER (0..255)

}

AAI-MM-RS-SYN-RSP ::=			SEQUENCE {

scanDuration				CHOICE {

zeroScanDuration			NULL,

nonzeroScanDuration			RelaySyncInfo

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multicast Messages

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...