| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MAC control messages for multimode operation in IEEE 802.16.1a |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16-12-400-00-Gdoc,” in response to Letter Ballot Recirc #38b on P802.16.1a/D3 |  |
| Abstract | This provides AWD text proposals for clarification of message fields for multimode operation over IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MAC control messages for multimode operation in IEEE 802.16.1a

Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Seokjoo Shin

Chosun University

Introduction

In this contribution, we suggest the corrections of typos as well as the modifications of the ambiguous text in message fields for multimode operation over IEEE P802.16.1a/D3. The table numbers for messages are 106b, 106c, 100, 106f, and 106g.

References

[1] IEEE P802.16nTM/D3, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[2] IEEE P802.16.1aTM/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” June 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, June 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Correct the typos or errors in Table 106b in IEEE P802.16.1a/D3.]

[Page# 57, Line# 6]

6.2.3.65.2 AAI-MM-RS-REQ

To establish relay link between a multimode station and superordinate HR-BS, AAI-MM-RS-REQ message is transmitted by the multimode station or the superordinate HR-BS.

Table 106b – AAI-MM-RS-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Request Relay mode | 1 | 0b0: TTR relay mode 0b1: STR relay mode | Always present |
| If(this request is subordinate station initiated request) { |  |  | Shall be present when subordinate station initiates AAI-MM-RS-RSPREQ |
| … | … | … | … |

[Remedy2: Correct the typos or errors in Table 106c in IEEE P802.16.1a/D3.]

[Page# 58, Page# 59]

6.2.3.65.3 AAI-MM-RS-RSP

An AAI-MM-RS-RSP message is transmitted by multimode station or superordinate HR-BS in response to AAI-MM-RS-REQ message.

Table 106c – AAI-MM-RS-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If(the response is transmitted by superordinate HR-BS) { |  |  | Present when superordinate HR-BS responds the subordinate station initiated request |
| Response code | 2 | 0b00: in response to the AAI-MM-RS-REQ message to accept the request 0b01: in response to the AAI-MM-RS-REQ message to allow to transmit subordinate station initiated AAI-ARE-REQ AAI-MM-RS-REQ after action time expires 0b10: in response to the AAI-MM-RS-REQ message to reject the request 0b11: reserved |  |
| If(Response code == 0b00 and the request was sent by an HR-BS wishing to establish TTR relay link to establish TTR relay mode){ |  |  |  |
| Ta | 11 | Confirmed value of timing advance Ta , in units of 0.1 μs |  |
| Tbs | 5 | Confirmed duration of the BS Operation mode, in units of frames |  |
| Trs | 5 | Confirmed duration of the RS Operation mode, in units of frames |  |
| } |  |  |  |
| If(Response code == 0b01) { |  |  |  |
| Action time | 4 | LSBs of the superframe number when the subordinate station transmits AAI-MM-RS-REQ message. | Always present |
| } |  |  |  |
| } else { |  |  | Present when subordinate station responds to the superordinate HR-BS initiated request |
| If(received request relay mode == 0b0) { |  |  | // TTR mode |
| ST-TTG | 6 | Transmit-to-receive turnaround gap of subordinate station, i.e., HR-MS or HR-BS, in units of μs. It shall be less than 50 μs. | Shall be present if action code request relay mode == 0b0 in AAI-MM-RS-REQ. |
| ST-RTG | 6 | Receive-to-transmit turnaround gap of subordinate station, i.e., HR-MS or HR-BS, in units of μs. It shall be less than 50 μs. | Shall be present if action code request relay mode == 0b0 in AAI-MM-RS-REQ. |
| If (requested subordinate station is HR-BS) { |  |  | Shall be present if the subordinate station is HR-BS and action code request relay mode == 0b0 in AAI-MM-RS-REQ. |
| Ta | 11 | Confirmed value of timing advance Ta , in units of 0.1 μs |  |
| Tbs | 5 | Confirmed duration of the BS Operation mode, in units of frames |  |
| Trs | 5 | Confirmed duration of the RS Operation mode, in units of frames |  |
| } |  |  |  |
| … | … | … | … |

[Remedy3: Correct the typos or errors in Table 130 in IEEE P802.16.1a/D3.]

[Page# 52] Note: Our suggestion is for the ‘Superframe Number Action’ field in page 52.

6.2.3.57 AAI-ARS-CONFIG-CMD message format

…

Table 100 – AAI-ARS-CONFIG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| If (subordinate HR-MS is multimode MS acting as HR-RS in HR-Network) { |  |  |  |
| … | … | … | … |
| Superframe Number Action | 4 | LSBs of the superframe number when HR-RS start HR-MS starts RS operation and apply applies the PHY operational parameters. | Always present |
| … | … | … | … |

[Remedy4: Correct the typos or errors in Table 106f in IEEE P802.16.1a/D3.]

[Page# 63]

6.2.3.65.6 AAI-MM-BS-REQ

…

Table 106f – AAI-MM-BS-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| If (SFH encoding format == 0b00) { |  |  |  |
| SFH Subpkt 1 | variable | See Table 182 in IEEE Std 802.16m-2011. |  |
| SFH Subpkt 2 | variable | See Table 183 in IEEE Std 802.16m-2011. |  |
| SFH Subpkt 3 | variable | See Table 184 in IEEE Std 802.16m-2011. |  |
| … | … | … | … |

[Remedy5: Correct the typos or errors in Table 106g in IEEE P802.16.1a/D3.]

[Page# 63]

6.2.3.65.7 AAI-MM-BS-RSP

…

Table 106g – AAI-MM-BS-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Response code | 2 | This indicates response type of AAI-MM-BS-RSP message. 0b00: Request by multimode HR-MS in Unsolicited manner 0b01: Approval of AAI-MM-BS-REQ 0b10: Rejection of AAI-MM-BS-REQ 0b11: Reserved |  |
| If (Response code == 0b10) { |  |  |  |
| Operation | 2 | 0b00: Retransmit AAI-MM-BS-REQ message (HR-BS mode of multimode HR-MS is not yet completely configured.) 0b01: Retransmit AAI-MM-BS-REQ message after applying the system configuration change count updating the system configuration parameters (System configuration change count of multimode HR-MS is out of date.) 0b10: Cancel BS configuration request 0b11: Reserved |  |
| } |  |  |  |