| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on relay function of HR-BS over IEEE 802.16.1a |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Seokjoo Shin Chosun University Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI | E-mail: sjshin@chosun.ac.kr woniks@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16-12-400-00-Gdoc,” in response to Letter Ballot Recirc #38b on P802.16.1a/D3 |  |
| Abstract | This provides AWD text proposals for clarification on relay function of HR-BS over IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on relay function of HR-BS over IEEE 802.16.1a

Seokjoo Shin

Chosun University

Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

# Introduction

In this contribution, we suggest the corrections of typos and modification of the sentences in Section 6.12.1.1 Relay function for HR-BS over IEEE P802.16.1a/D3. The major suggestions are listed in below.

- Technical corrections: Action type should be 4 bits

- Grammar errors : verb tense, article, etc

- Remove ambiguities in some sentences: suggest clear terminologies such as an affected HR-BS and a superordinate HR-BS

# References

[1] IEEE P802.16nTM/D3, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[2] IEEE P802.16.1aTM/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” June 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, June 2012.

# Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Insert the sentence in Section 3 in IEEE P802.16.1a/D3.]

[Page# 3, Line# 29]

3. Definitions

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary: Glossary of Terms & Definitions should be consulted for terms not defined in this clause.

…

3.71 self-coexistence mode: An operation mode of HR network, in which multiple HR cells share the same frequency channel in time. See also: high reliability network (HR-Network).

3.72 affected HR-BS: An HR-BS which is experiencing a failure of its backhaul connection to the backbone network. See also: high reliability base station (HR-BS)..

…

6.12 Support for HR-Networks

6.12.1 Multi-mode operation

6.12.1.1 Relay function for HR-BS

…

[Remedy2: Modify the sentences in Section 6.12.1.1.1 in IEEE P802.16.1a/D3.]

[Page# 108, Line# 24]

6.12.1.1.1 Relay link establishment

The HR-BS having no connection to backhaul transmits AAI-MM-ADV message with action type = 0b100 0b0100 described in 6.2.3.65.1 including expected time of backhaul link available. Based on the expected time, HR-MS handovers performs handover to neighbor infrastructure station or staying stays in the affected HR-BS until restarting service with an available backhaul link.

To establish relay link with a serving superordinate HR-BS, the affected HR-BS having no connection to backhaul follows network entry and initialization for relay link described in 6.2.15 and 6.6.2.10. In addition, the affected HR-BS shall perform the relay link establishment procedure as follows:

a) Scan for DL channel and establish synchronization with the HR-BS having connection to backhaul

b) Obtain DL/UL parameters (from SuperFrameHeader)

c) Perform ranging

d) Basic capability negotiation, if needed

e) Authorization, authentication, and key exchange, if needed

f) Registration with the HR-BS, if needed

g) Configuration operational parameters including initiating relay link using for establishing relay link using control messages such as AAI-MM-RS-REQ/RSP and AAI-ARS-CONFIG-CMD messages.

To establish relay link with another HR-BS (serving HR-BS) After performing the network entry to the superordinate HR-BS, the affected HR-BS having no connection to backhaul transmits AAI-MM-RS-REQ message described in 6.2.3.65.2 including relay mode, i.e., either TTR or STR mode. In response to AAI-MM-RS-REQ, the serving superordinate HR-BS transmits AAI-MM-RS-RSP message described in 6.2.3.65.3 to inform whether the request is accepted or rejected. Upon receiving the AAI-MM-RS-RSP message, the affected HR-BS starts establishing the relay link with serving superordinate HR-BS immediately or retransmits AAI-MM-RS-REQ message at after expiring the action time expires. If the serving superordinate HR-BS rejects the request, the serving superordinate HR-BS informs the HR-BS having no connection to backhaul the rejection of the request. Upon receiving the AAI-MM-RS-RSP message with rejection information, the HR-BS either tries to establish relay link with another HR-BS or follows standalone network operation described in 6.12.4.

…

[Remedy3: Modify the sentences in Section 6.12.1.1.3 in IEEE P802.16.1a/D3.]

[Page# 112, Line# 21]

6.12.1.1.3 Relay link configuration

During establishing relay link, serving the superordinate HR-BS transmits AAI-ARS-CONFIG-CMD message described in 6.2.3.57 to configure PHY layer operational parameter set including superframe number indicating the time to establish relay link start acting as HR-RS.

While the superordinate HR-BS is maintaining relay link, the serving superordinate HR-BS shall send AAI-ARS-ESI message described in 6.2.3.58 in the DL relay zone when the essential system information in SFH is changed. The superordinate HR-BS also shall send AAI-ARS-CONFIG-CMD message in the DL relay zone when PHY layer operational parameter needs to be reconfigured.

The HR-BS acting as relay may transmit AAI-MM-ADV message with action type = 0b000 0b0000 described in 6.2.3.65.1 for its subordinate MSs to update PHY/MAC layer operational parameter after receiving AAI-ARS-ESI or AAI-ARS-CONFIG-CMD message.

[Remedy4: Modify the sentences in Section 6.12.1.1.4 in IEEE P802.16.1a/D3.]

[Page# 112, Line# 33]

6.12.1.1.4 Relay link release

If the HR-BS recovers from failure of backhaul, it may inform network or notify the current serving HR-BS of the HR-BS having recovered backhaul link through the backhaul network interface. When the affected HR-BS recovers the backhaul link, the notification of recovery is announced to the backhaul network or its superordinate HR-BS through the backhaul interface. The superordinate serving HR-BS may then initiate HR-MS handover back to the recovered HR-BS in which the recovered HR-BS should be listed in the first priority. The recovered HR-BS having recovered backhaul may have been storing store MAC context information of the serving MSs (basic capabilities, security capabilities, etc.). Such context information allows HR-MS to perform optimized network reentry when returning back to the HR-BS upon its recovery.

The affected HR-BS transmits AAI-MM-ADV message with action type = 0b101 0b0101 described in 6.2.3.65.1 including expected time of backhaul link up. When receiving the AAI-MM-ADV message, HR-MS performs either handover to neighbor infrastructure station and returns to the HR-BS at the expected time or waiting in the HR-BS until restarting service with available backhaul link.

If the HR-BS acting as relay receives the request of relay link 1 release from superordinate serving HR-BS but the HR-BS acting as relay does not recover from failure of backhaul the backhaul link, the HR-BS either tries to establish relay link with another HR-BS having the backhaul link as described in 6.12.1.1.2 6.12.1.1.1 or follows standalone network operation described in 6.12.4.