| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of AAI-MM-ADV message in IEEE 802.16.1a |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Seokjoo Shin Chosun University Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI | E-mail: sjshin@chosun.ac.kr woniks@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16-12-400-00-Gdoc,” in response to Letter Ballot Recirc #38b on P802.16.1a/D3 |  |
| Abstract | This provides AWD text proposals for clarification of message fields for multimode operation over IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of AAI-MM-ADV message in IEEE 802.16.1a

Seokjoo Shin

Chosun University

Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Introduction

According to Section 6.12.1.3.1 Proactive Operation of IEEE P802.16.1a/D3, multimode HR-MSs can transmit their status information to the superordinate HR-BS after receiving AAI-MM-ADV message. In the current standard, however, AAI-MM-ADV message does not have an Action Type for requesting HR-MS’s status information response.

In this contribution, we suggest to insert an additional value of Action Type in AAI-MM-ADV for requesting multimode HR-MS’s status information.

References

[1] IEEE P802.16nTM/D3, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[2] IEEE P802.16.1aTM/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” June 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, June 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Insert the sentence and type in Section 6.2.3.65.1 in IEEE P802.16.1a/D3.]

[Page# 54, Line# 18]

6.2.3.65.1 AAI-MM-ADV message

Infrastructure stations and HR-MS acting as HR-BS or HR-RS may transmit AAI-MM-ADV message to support multimode operation in the case as follows:

- When the backhaul link is down or up

- During maintaining relay link due to unavailable backhaul link, PHY/MAC layer parameters need be reconfigured such as

  - Power down

  - Power reduction

  - FA change

- Multimode service establish/release/change to inform subordinate stations to perform handover

- Scanning operation for HR-MS acting as relay to maintain synchronization with the serving HR-BS

- Neighbor cell scanning operation

- Report status information of multimode station

Table 106a – AAI-MM-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Type | 4 | Used to indicate the purpose of this message 0b0000: Reconfiguration of HR-BS/RS including multimode BS/RS 0b0001: Restart of HR-BS/RS including multimode BS/RS 0b0010: Power down (including FA down) of HR-BS/RS including multimode BS/RS 0b0011: Power reduction of HR-BS/RS including multimode BS/RS 0b0100: Backhaul link down of HR-BS 0b0101: Backhaul link up of HRBS 0b0110: FA change of HR-BS/RS including multimode BS/RS 0b0111: Multimode service end of HR-MS 0b1000: Scanning operation for HR-MS acting as relay to maintain synchronization with the serving HR-BS 0b1001: Neighbor cell scanning 0b1010 : Handover of multimode station with access link reconfiguration 0b1011: Report status information of multimode station 0b1100 – 0b1111: reserved 0b1011 – 0b1111: reserved | Mandatory |
| … | … | … | … |