| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on relay function of HR-MS over IEEE 802.16.1a |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16-12-400-00-Gdoc,” in response to Letter Ballot Recirc #38b on P802.16.1a/D3 |  |
| Abstract | This provides AWD text proposals for clarification on relay function of HR-MS over IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on relay function of HR-MS over IEEE 802.16.1a

Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Seokjoo Shin

Chosun University

Introduction

In this contribution, we suggest the corrections of typos and modification of the sentences in Section 6.12.1.2 Relay function for HR-MS over IEEE P802.16.1a/D3. The major suggestions are listed in below.

- Grammar errors : verb tense, article, etc.

- Move a paragraph from subsection to subsection

- Remove a paragraph

- Reordering of subsections

- Modifications of sentences

References

[1] IEEE P802.16nTM/D3, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[2] IEEE P802.16.1aTM/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” June 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, June 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

6.12 Support for HR-Networks

6.12.1 Multi-mode operation

…

6.12.1.2 Relay function for HR-MS

[Remedy1: Modify the sentences in Section 6.12.1.2 in IEEE P802.16.1a/D3.]

[Page# 113, Line# 8]

An HR-MS may operate as an HR-RS to provide connectivity for multiple out-of-coverage HR-MSs. During basic capability negotiation at network entry, an HR-MS that is capable of role change to HR-RS shall report such capability to the super-ordinate superordinate HR-BS/HR-RS. A mode switch to HR-RS shall be commanded by its superordinate HR-BS.

While operating as HR-RS, the station maintains MS functionalities by performing dual-role MS/RS operation described in 6.12.1.2.5.

[Remedy2: Move a paragraph in Section 6.12.1.2.1 to Section 6.12.1.2.2 in IEEE P802.16.1a/D3.

[Page# 113, Line# 30]

6.12.1.2.1 Relay link establishment

…

During establishing relay link, HR-BS transmits AAI-ARS-CONFIG-CMD message described in 6.2.3.57 to configure PHY layer parameter set including superframe number indicating the time to start acting as HR-RS.

To support handover as a part of robustness against SPOF as described in 6.12.7.2, an indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

6.12.1.2.2 Relay link configuration

During establishing relay link, the HR-BS transmits AAI-ARS-CONFIG-CMD message described in 6.2.3.57 to configure PHY operational parameter set including superframe number indicating the time to start acting as HR-RS.

While HR-MS is acting as relay mode, the superordinate HR-BS shall send AAI-ARS-ESI message described in 6.2.3.58 in the DL relay zone when the essential system information in SFH is changed. The HR-BS also shall send AAI-1 ARS-CONFIG-CMD message in the DL relay zone when PHY layer parameter needs to be reconfigured.

[Remedy3: Remove the sentence in Section 6.12.1.2.2 in IEEE P802.16.1a/D3.]

[Page# 114, Line# 3]

While an HR-MS operating as HR-RS, any communication is performing with superordinate HR-BS in DL/UL relay zone to maintain HR-MS functionalities.

HR-MS acting as relay mode may transmit AAI-MM-ADV message described in 6.2.3.65.1 to update PHY/MAC layer parameter after receiving AAI-ARS-ESI or AAI-ARS-CONFIG-CMD message.

[Remedy4: Modify the sentences in Section 6.12.1.2.3.1 in IEEE P802.16.1a/D3.]

[Page# 114, Line# 9]

6.12.1.2.3 Relay link maintenance

6.12.1.2.3.1 Synchronization control for HR-MS acting as relay

Since HR-MS acting as relay has mobility, preambles from the superordinate HR-BS shall be received by HR-MS acting as relay periodically in TTR mode. A superordinate HR-BS may allocate time intervals to a relay mode HR-MS in order for an HR-MS acting as relay to receive preambles from the superordinate HR-BS in the purpose of synchronization between a superordinate HR-BS and an HR-MS acting as relay. Such a time interval during which for the HR-MS acting as relay scans scanning preambles from the superordinate HR-BS while not available to serve its MSs is referred to as a an A-preamble scanning interval. Initial parameter values for synchronization time the A-preamble scanning interval shall be given by the superordinate HR-BS through AAI-MM-RS-SYN-RSP message when an HR-MS acting as relay performs network entry to the superordinate HR-BS. During the relay mode operation, a superordinate HR-BS may transmit unsolicited AAI-MM-RS-SYN-RSP message to an HR-MS acting as relay or shall respond upon receiving AAI-MM-RS-SYN-REQ message transmitted by HR-MS acting as relay to chase the timing offset. Allocated time interval information shall be transmitted to the subordinate MSs within the coverage of the HR-MS acting as relay through AAI-MM-ADV message. During the scanning interval HR-MS acting as relay may communicate with its serving HR-BS but not with its subordinate MSs.

[Remedy5: Modify the sentences in Section 6.12.1.2.3.2.1 in IEEE P802.16.1a/D3.]

[Page# 115, Line# 6]

6.12.1.2.3.2 HO procedure for HR-MS acting as RS

6.12.1.2.3.2.1 HR-MS acting as RS scanning of neighbor HR-BSs

…

In STR mode, two cases are defined for neighbor cell scanning.

- Case 1: the neighbor cells using same FA with the superordinate HR-BS. In this case the HR-MS acting as HR-RS may use any interval to perform performs autonomous scanning.

- Case 2: the neighbor cells using different FAs with the superordinate HR-BS. In this case the scanning procedure of the HR-MS acting as HR-RS is identical to that of in TTR mode.

…

[Remedy6: Modify the sentences in Section 6.12.1.2.4 in IEEE P802.16.1a/D3.]

[Page# 116, Line# 9]

[Exchange the position between Relay link release and Dual-role operation of HR-MS]

6.12.1.2.4 6.12.1.2.5 Relay link release

An HR-MS acting as RS may end its relay service and remove the relay link from the HR-BS. During the HR-MS’ relay mode release process, all subordinate HR-MSs of the HR-MS acting as RS shall be transferred to another infrastructure station prior to HR-MS’ relay mode release. The HR-MS acting as RS sets Cell Bar bit to 1 in order to prevent HR-MS (re)entry and transmits AAI-MM-ADV message to transfer all subordinate HR-MSs to another infrastructure station. An HR-MS acting as RS may transmit an AAI-MM-RL-REQ message described in 6.2.3.65.4 in UL relay zone to an HR-BS so that it initiates the release procedure and requests handover of all its subordinate HR-MSs. Upon receiving the AAI-MM-RL-REQ message, the HR-BS decides whether it allows the HR-MS’ relay mode release. If the request is accepted, the HR-BS may transmit the AAI-MM-RL-RSP message described in 6.2.3.65.5 in DL relay zone to inform the acceptance and start BS-initiated handover process for the requested HR-MSs. After handover procedures between the HR-BS and HR-MS acting as RS’ subordinate HR-MSs are completed, the HR-BS informs the HR-MS acting as RS that handover is completed by transmitting an AAI-MM-RL-RSP message in DL relay zone.

…

[Remedy7: Modify the sentences in Section 6.12.1.2.5 in IEEE P802.16.1a/D3.]

[Page# 116, Line# 28]

6.12.1.2.5 6.12.1.2.4 Dual-role operation of HR-MS

An HR-MS acting as RS may maintain MS functionalities in company with RS functionalities. When an HR-MS in TTR relay mode receives AAI-ARS-CONFIG-CMD message with MS functionality maintenance indication that is the value 0b1, it performs dual-role MS/RS operation. At the time instance specified by “Superframe Number Action” in the AAI-ARS-CONFIG-CMD message, the following procedures shall be performed for dual-role operation of the HR-MS acting as RS.

- The dual-role HR-MS starts TTR relay mode and establishes relay link with the superordinate HR-BS. The HR-MS establishes relay link with the superordinate HR-BS and starts the RS mode operation.

- The MS mode entity in the dual-role HR-MS is behaving as a subordinate station of the RS mode entity. For connecting the MS’s service flow(s), the MS mode entity shall communicate with the RS mode entity through internal interfaces in the dual-role HR-MS. The details of MS’s reentry process with the RS through the internal interface is are out of scope of this standard.

- The RS mode entity in the dual-role HR-MS requests to the superordinate HR-BS for switching the data path of the MS mode entity by sending the AAI-L2-XFER message, as if the MS’ handover process had been performed.