| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on relay function of HR-MS over IEEE 802.16n |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16-12-400-00-Gdoc,” in response to Letter Ballot Recirc #37b on P802.16n/D3 |  |
| Abstract | This provides AWD text proposals for clarification on relay function of HR-MS over IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on relay function of HR-MS over IEEE 802.16n

Won-Ik Kim, Eunkyung Kim, Miyoung Yun, Seokki Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Seokjoo Shin

Chosun University

Introduction

In this contribution, we suggest the corrections of typos and modification of the sentences in Section 16.1.2 Relay function for HR-MS over IEEE P802.16n/D3. The major suggestions are listed in below.

- Grammar errors : verb tense, article, etc.

- Move a paragraph from subsection to subsection

- Remove a paragraph

- Reordering of subsections

- Modifications of sentences

References

[1] IEEE P802.16nTM/D3, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[2] IEEE P802.16.1aTM/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[3] EEE P802.16Rev3/D6, IEEE Draft Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” June 2012.

[4] IEEE P802.16.1TM/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, June 2012.

Proposed Text for the 802.16n AWD

Note:

The text in BLACK color: the existing text in the 802.16n AWD

The text in RED color: the removal of existing 802.16n AWD

The text in BLUE color: the new text added to the 802.16n AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16. Support for HR-Networks

16.1 Multi-mode operation

16.1.2 Relay function for HR-MS

[Remedy1: Modify the sentences in Section 16.1.2 in IEEE P802.16n/D3.]

[Page# 72, Line# 2]

An HR-MS may operate as an HR-RS to provide connectivity for multiple out-of-coverage HR-MSs. During basic capability negotiation at network entry, an HR-MS that is capable of role change to HR-RS shall report such capability to the super-ordinate superordinate HR-BS/HR-RS.

While operating as HR-RS, the station may maintain certain HR-MS functionalities MS functionalities by performing dual-role operation described in 16.1.2.4. A mode switch to HR-RS shall be commanded by its superordinate HR-BS.

[Remedy2: Remove the sentence in Section 16.1.2.2 in IEEE P802.16n/D3.]

[Page# 72, Line# 32]

16.1.2.2 Relay link configuration

While HR-MS is acting as relay mode, the superordinate HR-BS may send an RCD message to configure the Relay operation parameters as specified in 6.3.9.18. The HR-BS also shall send RS-Config-CMD message in the DL relay zone when PHY layer parameter needs to be reconfigured.

While an HR-MS operating as HR-BS, any communication is performing with superordinate HR-BS in DL/UL relay zone to maintain HR-MS functionalities.

HR-MS acting as relay mode may transmit MM-ADV message described in 6.3.2.3.99.1 to update PHY/MAC layer parameter after receiving RCD or RS-Config-CMD message.

[Remedy3: Modify the sentences in Section 16.1.2.3 in IEEE P802.16n/D3.]

[Page# 73, Line# 10]

[Exchange the position between Relay link release and Dual-role operation of HR-MS]

16.1.2.3 16.1.2.4 Relay link release

An HR-MS acting as RS may end its relay service and remove the relay link from the HR-BS. During the HR-MS’ relay mode release process, all subordinate HR-MSs of the HR-MS acting as RS shall be transferred to another infrastructure station prior to HR-MS’ relay mode release. The HR-MS acting as RS prevents HR-MS (re)entry and transmits MM-ADV message to transfer all subordinate HR-MSs to another infrastructure station. An HR-MS acting as RS may transmit an MM-RL-REQ message described in 6.3.2.3.99.7 in UL relay zone to an HR-BS so that it initiates the release procedure and requests handover of all its subordinate HR-MSs. Upon receiving the MM-RL-REQ message, the HR-BS decides whether it allows the HR-MS’ relay mode release. If the request is accepted, the HR-BS may transmit the MM-RL-RSP message described in 6.3.2.3.99.8 in DL relay zone to inform the acceptance and start BS-initiated handover process for the requested HR-MSs. After handover procedures between the HR-BS and HR-MS acting as RS’ subordinate HR-MSs are completed, the HR-BS informs the HR-MS acting as RS that handover is completed by transmitting an MM-RL-RSP message in DL relay zone. Upon receiving the MM-RL-RSP message, the HR-MS acting as RS starts relay mode release process immediately or at action time expires. If the HR-BS rejects the request, the HR-BS informs the HR-MS acting as RS the rejection of the request by transmitting the MM-RL-RSP message in DL relay zone. Upon receiving the MM-RL-RSP message with rejection information, the HR-MS acting as RS continues operating in relay mode. After action time expires, the HR-MS acting as RS retransmits an MM-RL-REQ message in UL relay zone to the HR-BS.

…

[Remedy4: Change the section number in IEEE P802.16n/D3.]

[Page# 73, Line# 27]

16.1.2.4 16.1.2.3 Dual-role operation of HR-MS

…