| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of TDC resource allocation |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Seokki Kim, Sungcheol Channg, Miyoung Yun, Won-Ik Kim, Hyun Lee, Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | Voice: +82-42-860-0626 E-mail: kimsk0729@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #38b (IEEE P802.16.1a/D3) |  |
| Abstract | This contribution proposes change to TDC resource allocation |  |
| Purpose | To be discussed and adopted by TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of TDC resource allocation

Seokki Kim, Sungcheol Chang, Miyoung Yun, Won-Ik Kim, Hyun Lee, Sungkyung Kim, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

This contribution proposes change to TDC resource allocation. There are two problems regarding legacy support and DL control as following.

- When the frame structure is supporting the WirelessMAN-OFDMA R1 MS in PUSC zone by FDM manner as Figure 1, UL subchannelization scheme is different from Advanced Air Interface as Figure 2. So TDC frame structure cannot be supported in UL FDM case.

- The DL Subframe which is used for transmitting A-preamble and midamble cannot be used for talk-around direct communication.

Figure 1 TDD frame configuration to support WirelessMAN-OFDMA UL FDM operation

Figure 2 Example of subchannelization for FDM base UL PUSC zone support

# References

[1] IEEE P802.16.1a/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, June. 2012.

# Proposed Text

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

# [Remedy1: Modify the following figure and text in section 6.12.2.3.2.1.1 in the IEEE P802.16.1a/D3]

6.12.2.3.2.1.1 Resource for talk-around direct communication

…

Figure 245-An example of talk-around direct communication resource allocation

The highest four PRUs of uplink resources are assigned for Common Direct Mode Zone (CDMZ). For FFT size = 512, PRU 20, 21, 22, 23 are assigned for CDMZ, for FFT size = 1024, PRU 44, 45, 46, 47 are assigned for CDMZ, and for FFT size = 2048, PRU 92, 93, 94, 95 are assigned for CDMZ. The highest four PRUs of downlink resources are assigned for Common Direct Mode Zone Extended(CDMZ-E). For FFT size = 512, PRU 20, 21, 22, 23 are assigned for CDMZ-E, for FFT size = 1024, PRU 44, 45, 46, 47 are assigned for CDMZ-E, and for FFT 1 size = 2048, PRU 92, 93, 94, 95 are assigned for CDMZ-E. The resources for Cell Specific Direct Mode Zone (CSDMZ) are multiple of four PRUs, and determined by each HR-BS independently and the assignment information is transmitted in the CDMZ. The DL Subframe which is used for transmitting A-preamble and midamble shall not be assigned for talk-around direct communication. When the frame structure is supporting the WirelessMAN-OFDMA R1 reference system, talk-around direct communication is supported in UL TDM mode only.

[-------------------------------------------------End of Text Proposal---------------------------------------------------]