| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 coding for control messages of multimode operation over IEEE 802.16.1a |  |
| Date Submitted | 2012-09-13 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Seokki Kim, Miyoung Yun, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | In response to Sponsor Ballot on P802.16.1a |  |
| Abstract | Comments on ASN.1 coding for multimode operation in IEEE P802.16.1a/D5 |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 coding for control messages of multimode operation over IEEE 802.16.1a

Won-Ik Kim, Eunkyung Kim, Seokki Kim, Miyoung Yun, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Sungcheol Chang

ETRI

Seokjoo Shin

Chosun University

Introduction

This document provides ASN.1 code for control messages of multimode operation over HR-Network.

References

[1] IEEE P802.16nTM/D5, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, 2012.

[2] IEEE P802.16.1aTM/D5, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, 2012.

[3] IEEE Std 802.16™-2012, IEEE Standard for Air Interface for Broadband Wireless Access Systems,” 2012.

[4] IEEE P802.16.1™/D6, IEEE Draft for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, 2012.

Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the 802.16.1a AWD

The text in RED color: the removal of existing 802.16.1a AWD

The text in BLUE color: the new text added to the 802.16.1a AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Annex A

...

A.2 MAC control message definitions (normative)

Change Annex A.2 as indicated:

WirelessMAN-Advanced-Air-Interface DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type,

...

}

[Remedy1: Add the following codes below Line# 61, Page#235 in IEEE P802.16.1a/D5.]

MAC-Control-Msg-Type ::= CHOICE {

...

-- HR Multimode operation

aaiMmAdv			AAI-MM-ADV,

aaiMmRsReq			AAI-MM-RS-REQ,

aaiMmRsRsp			AAI-MM-RS-RSP,

aaiMmRlReq			AAI-MM-RL-REQ,

aaiMmRlRsp			AAI-MM-RL-RSP,

aaiMmBsReq			AAI-MM-BS-REQ,

aaiMmBsRsp			AAI-MM-BS-RSP,

aaiMmBsCmd			AAI-MM-BS-CMD,

aaiMmRsSynReq			AAI-MM-RS-SYN-REQ,

aaiMmRsSynRsp			AAI-MM-RS-SYN-RSP,

-- HR-BS controlled direct communication

...

}

...

[Remedy2: Add the following codes below Line# 52, Page#249 in IEEE P802.16.1a/D2.]

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- MAC control messages for HR-Networks

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode Advertisement message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

ReconfBsRs ::=				SEQUENCE {

newIDCell				IDCell			OPTIONAL,

frameConfigurationIndex		INTEGER (0..63)	OPTIONAL,

-- Start of unavailable time in unit of frame

unavailableStartTime			INTEGER (0..255),

-- Interval of unavailable time in unit of superframe

unavailableTimeInterval		INTEGER (0..255)

}

RestartBsRs ::=			SEQUENCE {

-- Start of unavailable time in unit of frame

unavailableStartTime			INTEGER (0..255),

-- Interval of unavailable time in unit of superframe

unavailableTimeInterval		INTEGER (0..255)

}

PowerDownBsRs ::=			SEQUENCE {

--powered down in units of frame

timeOfPowerDown			INTEGER (0..255),

-- Expected uptime of BS in units of superframe

expectedUptimeOfBs			INTEGER (0..255)	OPTIONAL

}

PowerReductionBsRs ::=			SEQUENCE {

-- dB value of Tx power reduction

txPowerReduction			INTEGER (0..1023),

-- Expected resource adjustment time in units of frame

expectedTimeOfPowerReduction		INTEGER (0..255)

}

BackhaulLinkDown ::=			SEQUENCE {

--backhaul link down in units of superframe

timeOfBackhaulLinkDown			INTEGER (0..255)	OPTIONAL,

-- Expected uptime of BS in units of superframe

expectedTimeOfBackhaulLinkAvailalbe	INTEGER (0..255)	OPTIONAL

}

BackhaulLinkUp ::=			SEQUENCE {

--backhaul link up in units of superframe

timeOfBackhaulLinkUp			INTEGER (0..255)	OPTIONAL

}

FaChangeBsRs ::=			SEQUENCE {

-- FA index

fAIndex					FAIndex,

-- Expected current FA downtime in units of frame

expectedDowntimeOfCurrentFa		INTEGER (0..255),

-- Expected uptime of new FA in units of superframe

expectedUptimeOfNewFa			INTEGER (0..255)	OPTIONAL

}

MmServiceEndMs ::=			SEQUENCE {

--backhaul link up in units of superframe

timeOfBackhaulLinkUp			INTEGER (0..255)	OPTIONAL

}

ScanningOperationRsModeMs ::=			SEQUENCE {

scanDuration				INTEGER (0..255),

interleavingInterval			INTEGER (0..255),

recommendStartSuperFrame		SuperframeNumberLSB,

recommendedStartFrame			INTEGER (0..3)

}

NeighborCellScanning ::=			SEQUENCE {

scanDuration				INTEGER (0..255),

interleaving				INTEGER (0..255),

scanIteration				INTEGER (0..63),

recommendStartSuperFrame		SuperframeNumberLSB,

recommendedStartFrame			INTEGER (0..3)

}

HandoverMmStation ::=			SEQUENCE {

-- Start of unavailable time in unit of frame

unavailableStartTime			INTEGER (0..255),

-- Interval of unavailable time in unit of superframe

unavailableTimeInterval		INTEGER (0..255)

}

AAI-MM-ADV ::=				SEQUENCE {

actionType				CHOICE {

reconfBsRs				ReconfBsRs,			-- 0b0000

restartBsRs				RestartBsRs, 			-- 0b0001

powerDownBsRs				PowerDownBsRs, 			-- 0b0010

powerReductionBsRs			PowerReductionBsRs, 		-- 0b0011

backhaulLinkDown			BackhaulLinkDown, 		-- 0b0100

backhaulLinkUp				BackhaulLinkUp, 		-- 0b0101

faChangeBsRs				FaChangeBsRs, 			-- 0b0110

mmServiceEndMs				MmServiceEndMs, 		-- 0b0111

scanningOperationRsModeMs		ScanningOperationRsModeMs, 	-- 0b1000

neighborCellScanning			NeighborCellScanning, 		-- 0b1001

handoverMmStation			HandoverMmStation, 		-- 0b1010

reportStatusInfoMmStation		NULL,			 	-- 0b1011

...

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode RS Request message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MM-RS-REQ ::=				SEQUENCE {

requestRelayMode				ENUMERATED {

ttrRelayMode,

strRelayMode

},

subordinateStationInitReq			SEQUENCE {

--present if request relay mode == 0

sTTTG						INTEGER (0..49)	OPTIONAL,

sTRTG						INTEGER (0..49)	OPTIONAL,

--present if request relay mode == 0

subordinateStationIsBs				SEQUENCE {

tA						INTEGER (0..2047),

tBs						INTEGER (0..31),

tRs						INTEGER (0..31)

} OPTIONAL,

--present if request relay mode == 1

duplexModeSupportIndication			BIT STRING {

fDDSupport (0),

tDDSupport (1)

} (SIZE (2))	OPTIONAL,

--present if request relay mode == 1

numOfAvailableFrequency			SEQUENCE (SIZE (1..16)) OF SEQUENCE {

carrierFrequency				INTEGER (0..1023)

} OPTIONAL

} OPTIONAL,

subordinateStationInitReqAndRcvByBs		SEQUENCE {

--present if request relay mode == 0

tA						INTEGER (0..2047),

tBs						INTEGER (0..31),

tRs						INTEGER (0..31)

} OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode RS Response message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

SuperordinateBsTxMMRSRSP ::=			SEQUENCE {

responseCode					ENUMERATED {

acceptRequest,

retxAfterActionTimeExpires,

rejectRequest,

spare1

},

acceptAndSubordinateStationIsBsForTTR	SEQUENCE {

tA					INTEGER (0..2047),

tBs					INTEGER (0..31),

tRs					INTEGER (0..31)

} OPTIONAL,

retxAfterActionTime				SEQUENCE {

actionTime					INTEGER (0..31)

} OPTIONAL

}

TTRRevMMRSREQ ::=				SEQUENCE {

sTTTG							INTEGER (0..49),

sTRTG							INTEGER (0..49),

subordinateStationIsBs				SEQUENCE {

tA						INTEGER (0..2047),

tBs						INTEGER (0..31),

tRs						INTEGER (0..31)

} OPTIONAL

}

STRRevMMRSREQ ::=				SEQUENCE {

duplexModeSupportIndication				BIT STRING {

fDDSupport (0),

tDDSupport (1)

} (SIZE (2)) OPTIONAL,

--present if request relay mode == 1

numOfAvailableFrequency				SEQUENCE (SIZE (1..16)) OF SEQUENCE {

carrierFrequency					INTEGER (0..1023)

}

}

SubordinateStTxMMRSREQ ::=			SEQUENCE {

recRelayMode					CHOICE {

ttrRevMMRSREQ					TTRRevMMRSREQ,

strRevMMRSREQ					STRRevMMRSREQ

}

}

AAI-MM-RS-RSP ::=				SEQUENCE {

responseStation 				CHOICE {

superordinateBS				SuperordinateBsTxMMRSRSP,

subordinateStation				SubordinateStTxMMRSREQ

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode Release Request message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MM-RL-REQ ::=				SEQUENCE {

releaseRequestCode				ENUMERATED {

multimodeRelease,

responseUnsolicitReleaseRspMsgByBs,

rejectUnsolicitReleaseRspMsgByBs,

spare1

},

releaseReasonCode			ENUMERATED {

insufficientBatteryLevel,

impermissibleInterferenceLevel,

powerOffInitiation,

spare1

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode Release Response message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

ActionTime ::=					SEQUENCE {

actionTime					INTEGER (0..15)

}

AAI-MM-RL-RSP ::=				SEQUENCE {

actionCode					CHOICE {

terminateMultimodeServiceImmediately		NULL,

terminateMultimodeServiceAtActionTime		ActionTime,

responseForRetransmissionAtActionTime		ActionTime,

rejectReleaseRequest				NULL

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- HR Multimode BS Request message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

...

[-------------------------------------------------End of Text Proposal---------------------------------------------------]