| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Contribution towards an Architecture and Requirements Document for Small Cell Backhaul |  |
| Date Submitted | 2013-05-08 |  |
| Source(s) | Paul Trubridge, Paul Senior and David Castelow Airspan Communications Ltd Capital Point, 33 Bath Road Slough, SL3 1UF, UK | Voice: +44 1895 467261 E-mail: ptrubrid at airspan dot com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions 16-13-0074 |  |
| Abstract | The objective of this document is to outline use cases and establish requirements associated with NLOS Licensed sub5GHz Small Cell Backhaul applications. This will form a basis for the IEEE802.16 amendments to address this application space. |  |
| Purpose | For incorporation into the 16r Requirements Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Contribution towards an Architecture and Requirements Document for Small Cell Backhaul

Paul Trubridge, Paul Senior and David Castelow

Airspan Communications Ltd

Table of Contents

1	Introduction  (Informative)	4

2	Objective and Scope	5

3	Abbreviations, Definitions, and Conventions (Informative)	5

3.1	Conventions (Informative)	5

3.2	Abbreviations and Acronyms (Informative)	5

3.3	Definitions (Informative)	5

3.3.1	Definition 1	5

4	References	5

5	Use Cases (Informative)	5

5.1	Use Case 1 – Mobile Carrier, Backhaul for Outdoor LTE Pico-cell Deployment, POP at existing macro-sites.	5

5.1.1	Short Description	5

5.1.2	Actors	6

5.1.3	Pre-conditions	6

5.1.4	Post-conditions	6

5.1.5	Normal Flow	6

5.1.6	Alternative Flow	6

5.2	Use Case  2 – Mobile Carrier, Backhaul for Outdoor LTE Pico-cell Deployment, POP through street-based fibre network	7

5.2.1	Short Description	7

5.2.2	Actors	7

5.2.3	Pre-conditions	7

5.2.4	Post-conditions	7

5.2.5	Alternative Flow	8

5.3	Use Case  3 – Mobile Carrier, Backhaul for Indoor LTE Enterprise femto-cell Deployment.	8

5.3.1	Short Description	8

5.3.2	Actors	8

5.3.3	Pre-conditions	8

5.3.4	Post-conditions	8

5.3.5	Normal Flow	9

5.3.6	Alternative Flow	9

6	Requirements (Conditional Normative)	9

6.1	Mandatory Requirements	9

6.2	Optional Requirements	9

6.3	Network Requirements	9

6.4	Air Interface Requirements	10

6.4.1	Overview	10

6.4.2	Spectrum and Licensing	11

6.5	Security Requirements	11

6.6	Management Requirements	11

6.7	Performance Requirements	11

6.7.1	Capacity	11

6.7.2	Latency	12

6.7.3	Coverage	12

7	Architecture	13

7.1	Convergence Sublayer	13

7.2	Bridged Ethernet Support	13

8	Guidance and Recommendation to other Standards Organizations	13

Annex A	Document History (Informative)	14

Introduction  (Informative)

As the spectral efficiency of wireless links approaches its theoretical limits, and with the data traffic requirements continuing to grow rapidly, cell density and cooperation among base stations must increase in order to further improve network capacity and efficiently manage radio resources. Multi-tier access network architecture consisting of macrocells and a variety of overlaid smaller cells provides an approach towards solving the problem, allowing low cost per bit and efficiently utilizing all spectral resources in the system. Some such systems will be deployed using radio access technology outside the realm of IEEE 802.16. In such cases, IEEE Std 802.16, as enhanced, can provide out-of-band, or in-band (adjacent channel) wireless backhaul to the small cells, allowing those cells them to be positioned for optimal performance without regard to the local availability of high-capacity wired backhaul. The resulting system design will offer improvements in spectral efficiency needed to support the rapidly expanding demand for mobile broadband access.

Objective and Scope

The objective of this document is to outline use cases and establish requirements associated with NLOS Licensed sub5GHz Small Cell Backhaul applications. This will form a basis for the IEEE802.16 amendments to address this application space.

Abbreviations, Definitions, and Conventions (Informative)

Conventions (Informative)

TBC

Abbreviations and Acronyms (Informative)

TBC

Definitions (Informative)

### Definition 1

TBC

References

Use Cases (Informative)

Use Case 1 – Mobile Carrier, Backhaul for Outdoor LTE Pico-cell Deployment, POP at existing macro-sites.

### Short Description

As part of an LTE heterogeneous network. Providing backhaul for a layer of cells deployed outdoors on street poles such as lamp posts or utility poles, advertising billboard, or on building in non-traditional locations. The small cell layer is deployed to increase overall capacity of the network by providing data offload from the macro-cell in areas of high usage.

Network connectivity is made via existing macro-cells, where the wireless backhaul feeder is located on a high site along with macro LTE eNBs. Small cell locations are generally non-LOS from the macro-sites, which inherently increases the capacity gains in the LTE access network.

The network is owned and operated by the Mobile Carrier. The mobile carrier is responsible for the installation, commissioning and ongoing maintenance of the small cell backhaul transport network.

The small cell transport network is an extension of the Macro RAN transport network and should be managed and configured in a compatible way.

Typical Macro-hosted POP Small Cell Backhaul Scenario

### Actors

Outdoor pico eNB, 3G Pico NodeB or Wi-Fi Access Point

Small cell location – eg. Lamp post or utility pole, advertising billboard, or non-traditional building locations, minimum channel size of 3 MHz TDD.

Existing mobile carrier macro-cell site.

### Pre-conditions

Macro-cell site able to accommodate 802.16 backhaul feeder node.

Sub-5GHz (NLOS) Licensed spectrum available for backhaul.

Traffic from Macro network is overloaded and requires off-load or capacity augmentation from Small Cell Pico network

### Post-conditions

Data offload in high foot-fall locations providing LTE network capacity uplift.

### Normal Flow

Small Cells are integrated into Macro 3G/4G RAN and mobile users can connect to Macro or Small Cell based on the network design and policy decisions of the Mobile carrier. The small cell increase overall network capacity and/or fill coverage gaps in Macro Network. The 802.16 Small Cell backhaul transport network provides adequate backhaul capacity so that the QoS and QoE of the 3G, 4G or Wi-Fi services are equal to or better than the Macro based RAN..

### Alternative Flow

TBD.

Use Case  2 – Mobile Carrier, Backhaul for Outdoor LTE Pico-cell Deployment, POP through street-based fibre network

### Short Description

As part of an LTE heterogeneous network. Providing backhaul for a layer of cells deployed outdoors on street poles such as lamp posts or utility poles. The small cell layer is deployed to increase overall capacity of the network by providing data offload from the macro-cell in areas of high usage.

Network connectivity is made through a fibre network, where the wireless backhaul feeder is located at convenient street-pole locations where fibre is available. Small cell locations are a mix of non-LOS and LOS from the fibre connected sites.

Fiber

Relay

Relay

Typical Street-based POP Small Cell Backhaul Scenario

### Actors

Outdoor pico eNB, 3G Pico NodeB or Wi-Fi Access Point

Small cell location – eg. Lamp post or utility pole, advertising billboard, or non-traditional building locations

A proportion of small cell sites with fibre.

### Pre-conditions

Proportion of small cell sites able to accommodate backhaul feeder node.

Sub-5GHz (NLOS) Licensed spectrum available for backhaul, minimum channel size of 3 MHz TDD.

Traffic from Macro network is overloaded and requires off-load or capacity augmentation from Small Cell Pico network

### Post-conditions

Data offload in high foot-fall locations providing LTE network capacity uplift.

Normal Flow

Small Cells are integrated into Macro 3G/4G RAN and mobile users can connect to Macro or Small Cell based on the network design and policy decisions of the Mobile carrier. The small cell increase overall network capacity and/or fill coverage gaps in Macro Network. The 802.16 Small Cell backhaul transport network provides adequate backhaul capacity so that the QoS and QoE of the 3G, 4G or Wi-Fi services are equal to or better than the Macro based RAN.

### Alternative Flow

TBD.

Use Case  3 – Mobile Carrier, Backhaul for Indoor LTE Enterprise femto-cell Deployment.

### Short Description

As part of an LTE heterogeneous network. Providing backhaul for a layer of cells deployed indoors in shopping malls, stadiums, SME facilities etc. The small cell layer is deployed to increase coverage in these locations and provide dedicated capacity in these extreme high usage locations.

Wireless backhaul is provided to the building direct from the mobile carriers network by feeding from existing macro-sites. Final distribution to indoor eNBs is made through Ethernet or Fibre LAN within the building.

Typical Enterprise Small Cell Backhaul Scenario

### Actors

Indoor pico eNB, 3G Pico NodeB or Wi-Fi Access Point

Building to be covered using indoor eNB, or Wi-Fi Access Points

Existing mobile carrier macro-cell site.

### Pre-conditions

Macro cell sites able to accommodate backhaul feeder node.

Sub-5GHz (NLOS) Licensed spectrum available for backhaul.

### Post-conditions

Improved in-building coverage and dedicated LTE capacity for target locations.

### Normal Flow

TBD.

### Alternative Flow

TBD.

Requirements (Conditional Normative)

Mandatory Requirements

TBC

Optional Requirements

TBC

Network Requirements

The reference architecture to support an LTE air interface as defined in the 3GPP specifications is shown below.

The communication paths and associated protocols between the LTE access nodes (eNodeBs) and core network are shown in greater detail below.

Small cell backhaul shall support communication between eNodeBs using the X2 protocol, as well as communication to the core network. A summary is provided below.

	S1-MME; control plane interface between eNodeB & MME, using SCTP over IP.

	S1-U; user plane interface between eNodeB & S-GW, using GTP over UDP.

	X2-U; user plane interface between eNodeB, using GTP over UDP

	X2-C; control plane interface between eNodeB, using SCTP over IP

	MGMT; management plane interface between eNodeB & EMS, as specified in Section 13

Air Interface Requirements

### Overview

When considering small cell backhaul, operators will generally opt for licensed spectrum for the following key reasons:

- Maintaining CoS / QoS from core to user equipment. Licensed spectrum provides the means to deliver against an SLA by means of a controlled RF interference environment.

- Avoiding external interference to allow a scalable high capacity backhaul network.

Sub-5GHz has been used primarily for wireless access services where it’s ability to operate in harsh mobile non-line of sight (NLOS) propagation environments make it an ideal choice over higher frequency bands. The same characteristics also make it well suited to the backhaul of small cells deployed in locations where high capacity NLOS connectivity is required. Licensed frequency bands in the sub-5GHz range vary by geography. Particular allocations are fully occupied for mobile access services, however there are many allocations currently under utilised. These include small fragmented unpaired allocations, as well as frequency ranges above 3GHz which, due to higher propagation losses, are sub optimal for providing mobile connectivity to handsets. These spectrum allocations are ideally suited to NLOS small-cell backhaul (SCB) applications. The use of beam-switching / beam-forming, network wide synchronisation and highly directional antennas with sophisticated algorithms, delivers large spectral efficiency gains and enables n=1 frequency re-use, typically doubling or tripling the real-world capacities. This means that a small amount of licensed spectrum goes a long way.

### Spectrum and Licensing

Frequency ranges can be broadly considered as follows:

| Sub1GHz | Mostly fully utilized for mobile access using paired allocations. TVWS provides lightly licensed unpaired spectrum suitable for SCB in rural locations. |
| --- | --- |
| 1GHz to 2GHz | Largely used for mobile access using paired allocations. However small unpaired allocations, held by some operators, are unused and ideally suited to SCB. |
| 2GHz to 3GHz | Paired allocations recently or currently being auctioned primarily for mobile access. However, small unpaired allocations are available and ideally suited to SCB. |
| 3GHz to 5GHz | Largely under utilised and licensed for paired or unpaired operation. Due to propagation losses, this frequency range is more suited to fixed applications such as SCB than mobile access applications. Lightly licensed allocations are available also in this frequency range in some countries (eg. 3.65GHz in the USA) |

In some cases, >40MHz allocations are available, however this is unusual in the sub-5GHz bands. Generally allocations range from 5MHz to 20MHz.

Available licensed spectrum in the sub 5GHz bands varies from country to country. However it is generally licensed on a regional or nationwide basis allowing an operator to deploy large scale networks with contiguous coverage. An area, regional, or nationwide licensing regime allows an operator to roll out a network most rapidly. This is clearly advantageous when deploying a small-cell network, and the ability to lay down SCB coverage across a target area significantly speeds up the deployment of small-cell nodes.

Most spectrum regulators limit the power (EiRP) limits permitted by unlicensed spectrum. Sub 5GHz licensed bands allow much higher EiRP limits. Typically EiRPs in sub 5GHz license exempt bands are limited to +33dBm or +36dBm, whereas licensed sub 5GHz bands typically permit up to +60dBm or at least +50dBm. This gives a 14-30dB link budget advantage which can be practically leveraged for SCB using symmetric links.

Security Requirements

Management Requirements

Performance Requirements

### Capacity

The capacity of a wireless link is determined by it’s spectral efficiency multiplied by the channel size. Further to this, the capacity of a wireless system is determined by the “link” capacity multiplied by the system’s ability to re-use frequency channels.

Licensed sub-5GHz spectrum offers the opportunity to manage interference levels to leverage high order modulation and coding schemes such as 256QAM (and later 1024QAM). In addition, it’s NLOS propagation characteristics allow multi-path techniques such as MIMO to be used. These both increase the “link” spectral efficiency. Today’s SCB solutions use 256QAM and 2x2 MIMO to yield a net spectral efficiency in the order of 14b/s/Hz over the air. Future extensions are expected to use 1024QAM and 4x4 MIMO increasing net spectral efficiency to 35b/s/Hz. These typically equate to net TCP/IP channel capacities of 170Mbps and 425Mbps respectively when operating a 20MHz TDD channel which can be delivered on a point to multi-point or point to point basis.

OFDMA based frequency re-use schemes applied to downlink and uplink, combined with interference aware scheduling algorithms and smart steerable antenna technology promise the ability to deploy a SCB network in sub-5GHz licensed spectrum using a single channel allocation.

A point to multipoint SCB network running in a 20MHz sub-5GHz licensed spectrum allocation consisting of 50 backhaul “hub” nodes, is expected to provide a peak capacity of 9Gbps using today’s technology. With 1024QAM and 4x4 MIMO extensions, this can potentially be increased to 22Gbps.

Network-wide optimization and interference aware algorithms are required to achieve these capacities, and the ability to schedule traffic in frequency, time and space are fundamental to maximizing the spectral efficiencies. OFDMA and smart steerable antennas are being adopted by SCB vendors to achieve this.

It should be noted that the dimensioning of a point to multi-point backhaul solution is calculated differently to a traditional uncontended point to point solution. The ability of the system’s QoS to deliver bursty data rates to multiple locations (depending on instantaneous demand) allows for network dimensioning to be based on statistical multiplexing.

### Latency

SCB systems designed for sub-5GHz Licensed spectrum are mostly PtMP, where traffic scheduling is centrally coordinated. In such systems, the latency is dominated by air interface frame structures and scheduling algorithms. 15ms one way latency is typical of today’s solutions and sub 10ms one way latency is targeted with technology evolution. However, it should be noted that latency and QoS are closely coupled. Not all traffic requires the minimum latency and overall network capacity increases if real-time services are handled differently to traffic that is not latency sensitive. Latency aware scheduling is a key feature for systems deploying contended backhaul. A close integration of the backhaul with the access QoS is critical to ensuring delivery of real time services.

### Coverage

Sub-5GHz Licensed spectrum provides the means to achieve reliable NLOS coverage as well as LOS between network nodes. SCB systems using this spectrum are expected to be deployed in order to cover a target area rather than on a node by node basis.

Two basic deployment models are possible depending on whether network POPs are at street level or high-sites such as existing macro-sites. With street level POPs, NLOS backhaul is used between adjacent street poles where small cells are located. In this case, NLOS technology is advantageous in order to accommodate local clutter such as trees. Where high-site POPs are available, a point to multi-point “cellular” coverage of small cell backhaul can be deployed. Both of these models result in very different backhaul link distances, or POP spacing.

The link budget of the SCB solution is critical to achieving maximum coverage. A typical solution operating in 3.5GHz would have a system link budget greater than 160dB. In the interest of form factor and MIMO performance, unlike microwave point to point solutions, antenna gains cannot be simply increased to achieve these link budgets.

Typical POP spacings are shown below:

| Environment | Coverage Type | Typical POP Spacing |
| --- | --- | --- |
| Urban | “Cellular Coverage” | 3km |
| Sub-urban | “Cellular Coverage” | 5km |
| Rural | “Spotty Coverage” | 15-20km |

Architecture

Convergence Sublayer

TBC

Bridged Ethernet Support

TBC

Guidance and Recommendation to other Standards Organizations

TBC

- Annex A	Document History (Informative)

| Date | Subject History | Version |
| --- | --- | --- |
| 2013-05-08 | Draft | A |