IEEE P802.19
Wireless Coexistence

| Temporary resource release proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-04-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Päivi Ruuska | Nokia Research Center | Visiokatu 1, 33720 Tampere, Finland | +358-718035433 | paivi.m.ruuska@nokia.com |
| Jari Junell | Nokia Research Center | Itämerenkatu 11-13, 00180 Helsinki, Finland | +358-718036575 | jari.junell@nokia.com |
| Mika Kasslin | Nokia Research Center | Itämerenkatu 11-13, 00180 Helsinki, Finland | +358-718036294 | mika.kasslin@nokia.com |

Abstract

This document includes a proposal of a section which defines a procedure with which a CE releases resources in a temporary manner to DF 2.08.

The temporary resource release is initiated by a WSO that temporarily uses fewer resources than allocated to it. Temporarily released resources may be reallocated temporarily to the other WSOs in the same coexistence set. The resource need of a WSO may vary due to changes in number of connected client devices, active services per client devices, etc. In temporary resource release the CE communicates the release and its temporary nature to the serving CM. The CM informs other CMs serving WSOs in the same coexistence set on temporary released resources. The CMs may allocate the resources temporarily to other WSOs in the same coexistence set. Once the CE requires the resources back it communicates the reclaim of resources to the serving CM. The CM commands the WSOs and other CMs to stop using the temporarily released resources upon release expiry. The temporary resource release only applies to WSOs subscribing the management service.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Editorial instructions: The following section is proposed to be added in an appropriate place.

## 9.x Resource release

A WSO shall inform the coexistence system of the actual resource need or use, depending on coexistence service to which the WSO is subscribed. If the resource need or use changes permanently the WSO shall release the resources.

The WSO that is subscribed to the management service may release part of its resources temporarily if it estimates that the change in resource need is short term.

### 9.x.1 Temporary resource release

WSO that is subscribed to the management service may release a portion of the resources that are allocated to it in temporary manner.

### 9.x.1.1 Releasing resources

A CE may send a ResourceReconfiguration_request to the serving CM to indicate temporary release of resources. This procedure is described in 5.2.11.3 “Sending resource reconfiguration request from CE to CM”. The request contains operating parameters of the WSO after the temporary release, and indication of the temporary release activation. The temporary release shall be active for at least a 60 s time after sending the ResourceReconfiguration_request.

Upon receiving information on temporary resource release from a CE, the CM may inform the other CMs serving WSOs in the coexistence set of the releasing WSO. This is described in section 5.2.7.3 “Sharing coexistence set information procedure”. The CM indicates that the resource release is temporary.

Any CM which is aware of temporarily released resources may allocate any portion of the temporarily released resources to its WSOs belonging to the coexistence set of temporarily releasing WSO. The CM shall not change allocations of any other WSO in the coexistence set of the WSO for which temporarily released resources are allocated. The CM may send Reconfiguration_Request to the CE of the WSO that is subscribed to the management service. This is described in section 5.2.11.1 “Sending reconfiguration request from CM to CE”. The request indicates that the resources are temporary. The CM which reallocates temporary resources to a WSO, shall inform the other CMs serving WSOs in the same coexistence set on the changed resource allocation of the WSO, as defined in section 5.2.7.3 “Sharing coexistence set information procedure”.

### 9.x.1.2 Reclaiming the released resources

A CE shall send a ResourceReconfiguration_request to the serving CM to reclaim the released resources. This procedure is described in 5.2.11.3 “Sending resource reconfiguration request from CE to CM”. The request contains the indication of terminating the temporary release. The CE shall not send this ResourceReconfiguration_request before the temporary release has been active for at least TBD time.

Upon receiving a request to return the released resources from a CE that has temporary resource release active, the CM shall inform the other CMs that are aware of the temporary resources. This is described in section 5.2.7.3 “Sharing coexistence set information procedure”. The CM indicates the termination of the temporary resources.

If temporary resources were allocated to another WSO, the CM serving the WSO shall send a Reconfiguration_Request to the CE of the WSO. This is described in section 5.2.11.1 “Sending reconfiguration request from CM to CE.” The request indicates termination of the temporary resources as the reason for the reconfiguration. The CM shall also inform the other CMs serving the WSOs in the same coexistence set on the changed resource allocation of the WSO, as defined in section 5.2.7.3 “Sharing coexistence set information procedure”.

## 5.3 Messages

Editorial instructions: Add parameters to messages according to the proposal 12/0051r0.

## 5.4 Data types

Editorial instructions: Add parameters to data types according to the proposal 12/0051r0.