| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | LB#1 Issue 13 (Comments: 178,185, 190, 191, 192, 219, 222) |  |
| Date Submitted | September 2006 |  |
| Source(s) | Peretz Feder, Ajay Rajkumar, Andrea Francini. |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for the issues. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Change text on page 29, section 5.6 of draft D01.09 as follows:

# “The specification of the MIHF includes the definition of SAPs that are media-independent (i.e., insensitive to the interface types that the mobile node may support) and recommendations to define or extend other SAPs that are media-dependent. Media-independent SAPs allow the MIHF to provide services to the upper layers of the mobility-management protocol stack, the network management plane, and the data bearer plane. The MIH_SAP and associated primitives provide the interface from MIHF to the upper layers of the mobility-management stack. Upper layers need to register with MIHF as users to receive MIHF generated events and also for link layer events that originate at layers below the MIHF but may be passed on to upper layers through MIHF. Upper layers may directly send commands to MIHF. Similarly MIHF entity may also send transport these commands and events to another remote (peer) MIHF entity. Primitives corresponding to all these services described above are within the scope of MIH_SAP. The L2 transport functions are provided by media specific SAPs. The L3 transport functions may be provided by other higher layer services.”

## Replace the above with the following:

“The specification of the MIHF includes the definition of SAPs that are media-independent (i.e., transparent to the interface types that the mobile node may support) and recommendations to define or extend other SAPs that are media-dependent. Media-independent SAPs allow the MIHF to provide services to the upper layers of the mobility-management protocol stack, the network management plane, and the data bearer plane. The MIH_SAP and associated primitives provide the interface from MIHF to the upper layers of the mobility-management stack. Upper layers need to register with MIHF as users to receive MIHF generated events and also for link layer events that originate at layers below the MIHF but may be passed on to upper layers through MIHF. Upper layers may directly send commands to the local MIHF using the service primitives of the MIH_SAP. For communication between two remote MIHF, the propagation from the local MIHF to the remote MIHF relies on MIH protocol messages.”