| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21a: Security <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Security Related Information Elements |  |
| Date Submitted | June 26, 2009 |  |
| Source(s) | Antonio Izquierdo, David Cypher, Nada Golmie, Lily Chen (NIST) |  |
| Re: | IEEE 802.21 Session #33 in San Francisco, CA |  |
| Abstract | This document introduces a set of security related information elements to facilitate fast handovers. |  |
| Purpose | Task Group Discussion and Acceptance |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

The information service specified in IEEE 802.21-2008 uses information elements (IEs) to carry necessary information to facilitate fast handover.  These information elements are described in Section 6.5.4, defined in Section 6.5.5. Their representation and query methods are specified in Section 6.5.6.

Those information elements defined in IEEE 802.21-2008 do not include security related information about the potential candidate networks for a MN to handover to. However, security related information is crucial to make a handover decision and to prepare a fast handover such as fast authentication options (e.g. pre-authentication, proactive re-authentication, etc) supported by the network, which the candidate PoA is associated with.  This proposal introduces a set of security related information elements which may be included in 802.21a. The proposal also presents two options in defining the security related IEs for discussion.

At this stage, the proposal is not intent to provide a complete list for the information elements to be included in 800-21a. Additional security related information elements, introduced and needed for the security mechanisms and features defined in 802-21a, may be further included.

# Security Related Information Service

Before a handover decision is made and a handover is executed, the following information may be needed for both the MN and the relevant network entities.

- Authentication options:

  - Authentication protocols

    - Extensible Authentication Protocol (EAP)

      - EAP-TLS

      - EAP-TTLS

      - EAP-GPSK

      - EAP-AKA

      - etc

    - 3GPP AKA

    - etc

  - Authenticator’s information – If EAP is used as the authentication protocol, then each PoA must be associated with an authenticator

    - IP address;

    - L2 address (If the authenticator is collocated with the PoA, then it may have the same L2 address as the PoA.);

    - etc

  - Fast authentication options – In order to make fast handover, it may execute a fast authentication through the following options:

    - Pre-authentication

      - Direct pre-authentication

      - Indirect pre- authentication

    - Re-authentication

    - etc

- Layer 3 authentication and key establishment options

  - MOBIKE (IKEv2 Mobility and Multihoming Protocol)

  - etc

- Security policies and infrastructure

  - Identity of CAs

  - Identity of EAP server

  - Support of tunnel based authentication

  - Allow password based authentication

  - Etc.

The information will be represented by information elements. These information elements should be included in the 802-21a as additional IEs.

# Define Additional Security Related IEs in 802.21a

There are at least two options to define the aforementioned security related IEs.

- Add the IEs to existing containers;

- Define a security container for the new IEs.

In order to explore the pros and cons for each option and to make a decision in the future, we will provide the tentative text for each option. Please consider the listed information elements as place holders. The inclusion, representation, and completion will be finalized in the future.

## Add the IEs to Existing Containers

6.5.4 Information elements

Insert the following Information Elements in Table 10:

| Name of information element | Description | Data type |
| --- | --- | --- |
| General information elements |  |  |
| … |  |  |
| Access network specific information elements |  |  |
| IE_SEC_OPEN_AUTH | Whether the security policy allows open authentication. | BOOLEAN |
| IE_SEC_PASSWORD | Whether the security policy allows password based authentication. | Tbd |
| IE_SEC_CA | Certificate authority ID | Tbd |
| PoA-specific information elements |  |  |
| IE_SEC_AUTH_PROTOCOL | Which authentication protocol is supported for access authentication | Tbd |
| IE_SEC_EAP_METHODS | If it is EAP, then which EAP methods are supported | Tbd |
| IE_SEC_EAP_REAUTH | Whether to support re-authentication | BOOLEAN |
| IE_SEC_EAP_PREAUTH | Whether to support pre-authentication | BOOLEAN |
| PoA-specific higher layer service information elements |  |  |
| IE_SEC_MOBIKE | Whether to support MOBIKE | BOOLEAN |

## Define a Security Container

6.5.4 Information elements

Change the following text as indicated:

The Information Service elements are classified into the following three groups:

- General Information and Access Network Specific Information: These information elements give a general overview of the different networks providing coverage within an area. For example, a list of available networks and their associated operators, roaming agreements between different operators, cost of connecting to the network and network security and quality of service capabilities.

- PoA Specific Information: These information elements provide information about different PoAs for each of the available access networks. These IEs include PoA addressing information, PoA location, data rates supported, the type of PHY and MAC layers and any channel parameters to optimize link-layer connectivity. This also includes higher layer services and individual capabilities of different PoAs.

- Other information that is access network specific, service specific, or vendor/network specific.

- Security information: These information elements provide information on authentication protocols, authenticator information associated with a specific PoA, fast authentication options, EAP methods, security policies, and other security related information.

Insert the following Information Elements in Table 10:

| Name of information element | Description | Data type |
| --- | --- | --- |
| Security information elements |  |  |
| IE_SEC_OPEN_AUTH | Whether the security policy allows open authentication. | BOOLEAN |
| IE_SEC_PASSWORD | Whether the security policy allows password based authentication. | Tbd |
| IE_SEC_CA | Certificate authority ID | Tbd |
| IE_SEC_AUTH_PROTOCOL | Which authentication protocol is supported for access authentication | Tbd |
| IE_SEC_EAP_METHODS | If it is EAP, then which EAP methods are supported | Tbd |
| IE_SEC_EAP_REAUTH | Whether to support re-authentication | BOOLEAN |
| IE_SEC_EAP_PREAUTH | Whether to support pre-authentication | BOOLEAN |
| IE_SEC_MOBIKE | Whether to support MOBIKE | BOOLEAN |

6.5.6.2.1 IE Containers

Insert the following bullet and table at the end of 6.5.6.2.1:

- IE_CONTAINER_SEC – contains security related information as shown in Table 15.

Table 15 – IE_CONTAINER_SEC definition

| Information element ID = (see Table G.1) | Length = variable |
| --- | --- |
| IE_SEC_OPEN_AUTH |  |
| IE_SEC_PASSWORD |  |
| IE_SEC_CA |  |
| IE_SEC_AUTH_PROTOCOL |  |
| IE_SEC_EAP_METHODS |  |
| IE_SEC_EAP_REAUTH |  |
| IE_SEC_EAP_PREAUTH |  |
| IE_SEC_MOBIKE |  |

# Decisions to Make and Next Steps

In order to move forward, we will need to make the following decisions.

- Whether to add the security related IEs to the existing containers as demonstrated in Section 3.1 or define a new container as demonstrated in Section 3.2.  What are the pros and cons for each of the options?

- Besides introducing the related terminologies in Clause 3 and 4, shall we add one sub-clause in Clause 5.1 about security in the handover?

- What other security IEs are needed?

- Any other changes are needed for introducing the new IEs?

Here, the original defined information elements in each container are omitted. In 802.21a, it will replace table 10 with added information elements.