| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21a: Security <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Candidate authenticator discovery mechanism |  |
| Date Submitted | July 5, 2009 |  |
| Source(s) | Dapeng Liu (China Moible) |  |
| Re: | IEEE 802.21 Session #33 in San Francisco, CA |  |
| Abstract | This document introduces a mechanism used for 802.21a candidate authenticator discovery |  |
| Purpose | Task Group Discussion and Acceptance |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

The applicability of EAP pre-authentication and pro-active re-authentication is limited to the scenarios where the candidate authenticator(s) can be discovered prior to mobile node movement to the candidate network.

This proposal introduces the method of extending 802.21 standards to define new Event and Information Elements for candidate authenticator discovery.

# Overview of proposed candidate authenticator discovery mechanism

The basic procedure of candidate authenticator discovery is as follows:

- The peer is connected to a serving network and authenticated by the serving authenticator.

- The peer or the serving network discovers one or more candidate network(s) in its neighborhood.

- When the peer or serving network discovers one or more candidate network(s), a candidate network detected MIH Event which is specified by this proposal is generated by the peer or network.

- The peer’s MIH user receives the candidate network detected Event and this Event will trigger the peer to initiate a MIIS query. The query will be transmitted to the MIH Information Server which contains the candidate authenticator Information Element that specified by this proposal.

- The peer receives the query response which contains the candidate authenticator’s information, such as its L2 and IP address.

- The peer or the network then initiates pre-authentication or pre-active re-authentication which is performed between the peer and the discovered candidate authenticator.

# Define new MIH Event for candidate authenticator discovery

6.3.4 Link event

In table 4-Link events, insert following item:

| Link event name | Link event type | Description | Defined in |
| --- | --- | --- | --- |
| Link_Candidate_PoA_Detected | State change | A candidate PoA has been detected. This event is typically generated when the MN or network detects a new candidate PoA. | TBD |

6.3.5 MIH event

In table 5-MIH events, insert the following item:

| MIH event name | (L)ocal (R)emote | Description | Defined in |
| --- | --- | --- | --- |
| MIH_Candidate_PoA_Detected | L,R | A candidate PoA has been detected. This event is typically generated when the MN or network detects a new candidate PoA. | TBD |

If the PoA and the MIH PoS are co-located, then the Candidate PoA Detected Event can be generated by either the network (MIH PoS) or the MN; otherwise the Event can only be generated by the MN.

# Define new MIH Information Element for candidate authenticator discovery

New MIH Information Element should be defined that contains the candidate authenticator’s information such as its L2 or IP address, supported proactive authentication method etc.

    - Information elements

In table 10, insert the following item:

| Name of information element | Description | Data type |
| --- | --- | --- |
| PoA Specific Information Elements |  |  |
| IE_POA_AUTHENTICATOR_ADDR | The L2 address of the authenticator which serves the PoA. | LINK_ADDR |
| IE_POA_AUTHENTICATOR_METHOD_INFO | The supported proactive authentication method information of the authenticator which serves the PoA. (eg. EAP pre-authentication or re-authentication) | SYSTEM_INFO |
| PoA Specific Higher Layer Service Information Elements |  |  |
| IE_POA_AUTHENTICATOR_IP_ADDR | The IP address of the authenticator which serves the PoA. | IP_ADDR |