| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Change Request to 21-08-0172-01-0sec-21-08-0012-02-0sec-mih-security-technical-report |  |
| DCN | 21-08-xxxx-00-0000 |  |
| Date Submitted | 29th October, 2008 |  |
| Source(s) | Da-Peng Liu, Hui Deng |  |
| Re: | IEEE 802.21 Security Study Group |  |
| Abstract | Change request to MIH Security Technical Report |  |
| Purpose | This document provides potential approach for the authenticator discovery mechanism used in pre-authentication and pro-active re-authentication. The suggested changes are in section 2.3.4 and kept under “track changes”. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

IEEE 802.21 Security Study Group

Technical Report

1	Introduction	4

1.1	Scope	4

1.1.1	Security signaling optimization during handovers	4

1.1.2	MIH Level security	5

1.2	References	5

1.3	Definitions and abbreviations	6

1.3.1	Definitions	6

1.3.2	Abbreviations	6

2	Security Signaling Optimization during Handovers	7

2.1	Main handover scenarios	7

2.2	Use Cases	8

2.2.1	Applicable Handover Scenarios	8

2.2.2	General Requirements	9

2.2.3	General Assumptions	9

2.2.4	Use Case 1	10

2.2.4.1	Assumptions	10

2.2.4.2	Requirements	10

2.2.5	Use Case 2	10

2.2.5.1	Assumptions	11

2.2.5.2	Requirements	11

2.2.6	Use Case 3	11

2.2.6.1	Assumptions	12

2.2.6.2	Requirements	12

2.3	Potential Approaches	12

2.3.1	Potential Approaches Matrix	12

2.3.2	EAP Pre-Authentication	13

2.3.2.1	High-Level Architecture	15

2.3.2.2	Applicability of EAP Pre-Authentication	15

2.3.2.3	High-Level Requirements for EAP Pre-Authentication	16

2.3.3	Pro-active EAP Re-Authentication	16

2.3.3.1	Key Hierarchy	17

2.3.3.2	Protocol Interfaces	18

2.3.3.3	Applicability of Pro-active EAP Re-authentication	18

2.3.3.4	Requirements for Pro-active EAP Re-authentication	19

2.4	Example Call Flows	20

2.4.1	Example Call Flow for Pro-active EAP Re-Authentication	20

3	MIH Protocol Security	21

3.1	Terminologies	21

3.2	Creating a security profile for the MIH protocol	22

3.3	Assumptions on trust relationships	23

3.4	Threat Analysis	23

3.5	General Assumptions	30

3.6	Use cases	31

3.6.1	Use Case 1:  Access Control is applied	31

3.6.1.1	Use Case 1.1	31

3.6.1.2	Use Case 1.2	33

3.6.2	Use Case 2: Access Control is not applied	35

3.6.2.1	Use Case 2.1	35

3.6.2.2	Use Case 2.2	37

3.6.3	Use Case 3: Visited Domain access	38

3.6.3.1	Use Case 3.1	38

3.6.3.2	Use Case 3.2	40

# Introduction

The IEEE 802.21 draft provides a media independent solution for handovers but does not address the security signaling required between heterogeneous networks. To perform a secure handover, the authorization and authentication between the MN and the target network needs to be addressed. However, security-related signaling can add significant delays to handover efforts. In many cases, service continuity can no longer be met, which affects time sensitive applications in particular, and ultimately impacts the user experience.

The other aspect is that the IEEE 802.21 draft does not address security at the MIH level. Because 802.21 provides services that affect network resources, cost and user experience, MIH level security will be an important factor to network providers that want to  deploy these MIH services in their network. MIH level security may comprise of the following aspects: mutual or unilateral authentication of MIH peer nodes, authentication-based access control to MIH services, MIH protocol messages confidentiality, integrity and replay protection.

## Scope

The scope of this document is to describe usage scenarios, requirements and potential high-level approaches for the following two 802.21 Security Study Group topics:

  - Security signaling optimization during handovers

  - MIH-level security

### Security signaling optimization during handovers

This report considers mobile devices (MN) that have multiple radios to access 802-based networks (e.g. 802.11 and 802.16). Devices with multiple radios which cannot operate concurrently have certainly more stringent requirements on security signalling delays during handovers, compared to devices that have multiple radios available that can operate in an overlapping fashion during the handovers.

Each access network may have its own authentication scheme and key hierarchy. This report specifically addresses a security transition scheme between 802-based networks that are based on EAP authentication mechanisms.

The document outlines use cases whereby:

- A mobile device transitions between two 802-based access networks of different media types within the same AAA domain, e.g., 802.16 and 802.11

- A mobile device transitions between two 802-based access networks of the same media type and deployed in different AAA domains, e.g., 802.16

- A mobile device transitions between two 802-based access networks of different media types and deployed in different AAA domains, e.g., 802.16 and 802.11

The document also examines the requirements for and implications of:

- Network-specific aspects: changes required in access network or core network infrastructure for seamless transition between networks without comprising security

- Mobile client aspects: Client authentication credentials, security advertisements and discovery, cryptographic algorithms, and key-usage scope and requirements.  (e.g. 802.16e/m, 802.11r)

- IETF HOKEY and other standard group aspects:  investigate if HOKEY covers all the targeted transition scenarios

### MIH Level security

With regards to MIH level security, the following security characteristics are being considered and use cases are provided:

- One-way authentication or mutual authentication of MIH peer nodes

- Access control to MIH services

- MIH protocol data confidentiality, integrity and replay protection

## References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

[1]	IEEE Draft Standard 802.21-D9.0: "Media Independent Handovers"

[2]	RFC 3748 – Extensible Authentication Protocol (EAP)

[3]	I-D: draft-ietf-hokey-preauth-ps-01.txt – EAP Pre-authentication Problem Statement

[4] 	IEEE Draft Standard 802.11r D9.0: ”Fast BSS Transition”

[5]	21-07-0127-00-0000-Hokey-802.21.ppt, ”Handover security in a heterogeneous Access Environment IETF HOKEY-IEEE 802.21 Integration”

[6] 	21-07-0122-04-0000-Security_proposal.ppt, “Security Optimization During Handovers: 802.21 SG Proposal”.

[7] I-D: draft-ietf-hokey-emsk-hierarchy-01.txt

[8] I-D: draft-ietf-hokey-erx-05.txt

[9]  RFC 1136 – Administrative Domains and Routing Domains: A model for routing in the Internet

[10] Draft-melia-mipshop-mstp-solution-01: “Mobility Services Transport Protocol Design”

[11] Draft-ietf-mipshop-mis-ps-05: “Mobility Services Transport: Problem Statement”

[14] IEEE Draft Amendment to Standard P802.11u/D2.0: Interworking with External Networks

[15] IEEE Draft Std 802.16g-2007 September 2007

[16] RFC 4907:  Architectural Implications of Link Indications, June 2007

## Definitions and abbreviations

### Definitions

AAA domain: See RFC2903

Authenticator: EAP Authenticator [2]

Candidate Authenticator: The authenticator on a candidate Point of Attachment (PoA) with respect to the mobile node.

EAP Pre-authentication:  The utilization of EAP to pre-establish EAP keying material on an authenticator prior to attaching the peer to the access network managed by that authenticator.

Peer: EAP Peer [2]

Serving Authenticator: The authenticator on the serving PoA/PoS

Target Authenticator: The authenticator on the target PoA/PoS

### Abbreviations

In addition to the abbreviations that are defined in the 802.21 draft specification, the following abbreviations are defined:

.

BS		Base station in 802.16 networks

CA		Candidate Authenticator

CS		Command service of the media independent handover services

ERS	EAP Re-authentication Server

ES		Event service of the media independent handover services

IAB	Internet architecture board

IS		Information service of the media independent handover services

FQDN 	Fully qualified domain name

MIH	Media independent handover services

MIHF	Media independent handover services function

MN	Mobile node

MoS	Mobility services (as defined in [10]) - includes the MIH IS, CS and ES)

MoSh	Mobility service in home network

MoS3	Mobility service in third party network

MoSv	Mobility service in visited network

MS		Mobile station in 802.16 networks

MSTP	Mobility services transport protocol

ND&S	Network detection and selection

PFS	Perfect forward secrecy

PoA	Point of attachment

PoS	Point of service

SA		Serving Authenticator

TA		Target Authenticator

MIHID	MIH Identifier e.g. FQDN

NN		Network Node

# Security Signaling Optimization during Handovers

## Main handover scenarios

As an example, let us consider a device with 2 radios doing a handover from an 802.11 access network to an 802.16 access network. Both access networks are in the same AAA domain.

- Both radios can operate concurrently:

In this case, both the source radio (accessing the 802.11 network) and the target radio (to access the 802.16 network) can be operating before, during and after the session transfer/handover. The mobile device can establish the target network resource reservation; perform network authentication and context establishment using the target radio to prepare for a smooth handover to that target network. In effect this is a ‘make-before-break’ handover as service disruption can be avoided. Once the session has completely transferred to the target network, the source radio can be turned off if so desired.

- Only one radio can operate at a given time:

In this case, the target radio (for accessing the 802.16 network) cannot operate concurrently with the source radio (accessing the 802.11 network). The reason for this mobile device limitation may be radio interference, insufficient power, or a regulatory reason. The mobile device will have to do resource reservation, (some form of) pre-authentication and context establishment on the target network using the source radio and network. Once the preparation on the target network has been done, the source radio will be deactivated and the target radio will become active and will start the attachment process with the target network. The more the MN can prepare on the target network using the source radio and network, the more the MN will be able to cut down the time it takes to do a handover from the source to the target network. In effect, this is a ‘break-before-make’ handover.

In order for the MN to be able to prepare the target network for a handover, there must be a level of coupling between the source and target network.

Figure 1: 802.11 and 802.16 network coupling

## Use Cases

### Applicable Handover Scenarios

Figure 1 below depicts which handover scenarios are addressed in this report.

Intra-technology

Intra-AAA domain

Inter-AAA domain

Inter-technology

EAP to non-EAP

Yes

No (for

future study)

Inter-technology

EAP to EAP

No

Yes

Yes

No (for

future study)

Figure 2: Applicable handover scenarios

### General Requirements

The following requirements are common for all use cases outlined in Section 2.1:

R0.1 The subscriber possesses a MN which gives access to 802 access networks. For example; the MN can have 802.11 and 802.16 radios which can either operate concurrently or only one radio can operate at any given time.

R0.2 Transition between networks shall be automatic and shall not require the   manual intervention of the user. The Network and MN should work in tandem to provide the most optimal handover experience.

R0.3 It shall conduct an authentication prior to handover to a target network. That is, a transition to a different network shall be authorized based on an authentication.

R0.4 The handover procedure shall establish a security context in the target network

R0.5 The resource consumption (including network traffic and power consumption) of the authentication and key establishment for handovers should be minimized with respect to a full EAP authentication.

R0.6 The delay caused by the authentication and key establishment for handovers should be minimized.

### General Assumptions

The following assumptions are common for all use cases outlined in Section 2.1:

A0.1 EAP ([2]) is used as the access authentication protocol for each of the media types. The EAP methods provide mutual authentication and required key material.

A0.2 A MN is authenticated with the serving authenticator through an EAP method.

A0.3 MN shall be able to discover candidate authenticators.

A0.4 Furthermore, HOKEY key hierarchy ([5] and [7]) may be supported by the employed EAP methods.

### Use Case 1

A mobile device transitions between two 802-based networks of different media types (e.g., 802.11 and 802.16) within the same AAA domain.

Figure 3: Handover scenario between 802.11 and 802.16 networks (single AAA domain)

#### Assumptions

See section 2.1.3 General Assumptions.

#### Requirements

See section 2.1.2 General Requirements.

### Use Case 2

A mobile device transitions between two networks of different media types (e.g., 802.16 and 802.11) and deployed in different AAA domains.

Figure 4: Handover scenario between 802.11 and 802.16 networks (different AAA domains)

#### Assumptions

See section 2.1.3 General Assumptions.

#### Requirements

See section 2.1.2 General Requirements.

### Use Case 3

A mobile device transitions between two networks with the same media types (e.g., 802.16) and deployed in different administrative domains.

Figure 5: Handover scenario between two 802.16 networks (different AAA domains)

#### Assumptions

See section 2.1.3 General Assumptions.

#### Requirements

See section 2.1.2 General Requirements.

## Potential Approaches

### Potential Approaches Matrix

Intra-technology

Intra-AAA domain

Inter-AAA domain

Inter-technology

EAP to non-EAP

As defined in the technology specific key hierarchy, like 802.11r or 3GPP

Key hierarchy based solution

May need to establish mapping between different authentication mechanisms

(For future study)

EAP Pre-authentication based solution

Inter-technology

EAP to EAP

May need to establish mapping between different authentication mechanisms

(For future study)

Figure 6: Potential approaches matrix

### EAP Pre-Authentication

The following usage scenario is considered for security signaling optimization during handover:

- The peer is connected to the serving network.

- The peer or the serving network discovers one or more candidate network(s) in its neighborhood. The candidate network(s) and the serving network may have different link-layer technologies or may be in different IP subnets or administrative domains.

- The peer or the serving network chooses one or more candidate authenticator(s) among the discovered candidate network(s) and initiates EAP pre-authentication to be performed between the peer and authenticator on the candidate network(s).

- A successful EAP pre-authentication for candidate authenticator(s) will generate EAP keying material that is delivered from the EAP server to the authenticator(s) of candidate network(s).

- After EAP pre-authentication, the peer or the serving network chooses one target authenticator in one of the candidate network(s), and a handover procedure will take place to switch from the serving authenticator to the target authenticator.

- In the handover procedure, the EAP keying material generated during the EAP pre-authentication process will be used for establishing a session key (or session keys) to be used for protecting link-layer frames exchanged between the peer and the authenticator of the target access network.

There are two modes of operations to establish EAP pre-authentication depending on whether the pre-authentication signaling is transparent to the serving authenticator or not. Direct pre-authentication is the mode in which the pre-authentication signaling is transparent to the serving authenticator (see Fig.6). Indirect pre-authentication is the mode in which the authenticator participates in the pre-authentication signaling (see Fig.7).

Serving PoA

M

I

H

MN

R1

M

I

H

P

oS

MIH PoS

R5

Candidate PoA

Home AAA

Server

MN-CA Signaling

EAP over L2 or

higher layers (HL)

EAP over AAA

R2

Figure 7: Direct pre-authentication model

Serving PoA

M

I

H

MN

R1

R2

MIH PoS

R5

Candidate PoA

Home AAA

Server

MN-SA Signaling

EAP over L2/HL

EAP over AAA

SA-CA Signaling

EAP over L2/HL

SA  Serving Access Network

CA ( Candidate Access Network

MIH PoS

Figure 8: Indirect pre-authentication model

#### High-Level Architecture

Figure 9: Reference model

The following reference points defined in 802.21 are reused to support EAP pre-authentication, with additional functionalities as described below.

R1: MN-SA signaling messages are exchanged over this reference point.

R2: MN-CA signaling messages are exchanged over this reference point.

R5: SA-CA signaling messages are exchanged over this reference point.

#### Applicability of EAP Pre-Authentication

EAP pre-authentication has general applicability to various deployment scenarios in which pro-active signaling can take effect.  In other words, applicability of EAP pre-authentication is limited to the scenarios where candidate authenticators can be easily discovered and an accurate prediction of movement can be easily made.

The effectiveness of EAP pre-authentication is more significant for intra-technology and inter-subnet handover scenarios, and particular inter-technology handover scenarios where simultaneous use of multiple technologies is a major concern or where there is not sufficient radio-coverage overlap among different technologies.

#### High-Level Requirements for EAP Pre-Authentication

Requirement 1: MIH PoS shall support the functionalities of authenticator for EAP pre-authentication

Requirement 2: MN shall support the functionality of peer for EAP pre-authentication

Requirement 3: An authenticator discovery mechanism shall be defined. The authenticator discovery mechanism must provide a mapping between a link-layer address and an IP address of an authenticator.

Requirement 4: A context binding mechanism shall be defined so that a link-layer specific security context is bound to the EAP keying material generated as a result of EAP pre-authentication. The link-layer specific security context includes link-layer addresses of a peer and an authenticator.

Requirement 5:  Higher-layer transport shall be supported for carrying EAP pre-authentication messages between MN and CA, between MN and SA and between SA and CA

Requirement 6: Link-layer transport shall be supported for carrying EAP pre-authentication messages between MN and SA

Requirement 7: The EAP pre-authentication process shall define a ‘lifetime’ parameter (pre-authentication validity time-out)

### Pro-active EAP Re-Authentication

The following usage scenario is considered for security signaling optimization during handover:

- The MN is connected to the serving network.

- The MN or the serving network discovers one or more candidate network(s) in its neighborhood. The candidate network(s) and the serving network have different link-layer technologies.

- The MN or the serving network chooses one or more candidate authenticator(s) among the discovered candidate network(s) and initiates EAP re-authentication ([8]). It is to be performed between the MN (as the peer) and an authenticator on the candidate network(s).

- Upon a successful EAP re-authentication, the authenticator will receive an rMSK that is delivered from the EAP server (or a local server).

- After EAP re-authentication, a handover procedure will take place to switch from the serving authenticator to the target authenticator.

- In the handover procedure, rMSK may be used for establishing intra-technology handover keys for different PoAs or session keys to be used for protecting link-layer frames exchanged between the MN and a PoA, depending on media specific handover.

The following items are for further study in 802.21 Security Study Group:

- Due to the optimization in security signaling by using EAP re-authentication, MN may or may not need to re-authenticate with multiple candidate authenticators.

- For the same reason MN may or may not use early initiated re-authentication with the target authenticator.

- Similar to EAP pre-authentication, MN either directly authenticates to the target or candidate authenticator or indirectly through the serving authenticator.

#### Key Hierarchy

The key hierarchy is derived using EMSK by the EAP authentication server or using DSRK by a local authentication server.  The re-authentication root keys (rRKs) are derived from EMSK or DSRK.

A re-authentication integrity key (rIK) is derived from rRK to provide integrity protection in the re-authentication between a peer (MN) and an EAP re-authentication server (ERS). MN and ERS communicate through the target authenticator, where ERS may be the EAP server / home server or a local server which has previously obtained an rRK.

Upon a successful re-authentication, an rMSK is delivered to the target authenticator (TA).  Fig. 2 illustrates the HOKEY key hierarchy.

rRK-1

rRK-2

rMSK-2

rMSK-1

EMSK or DSRK

rIK-1

rIK-2

Figure 10: HOKEY key hierarchy

#### Protocol Interfaces

The protocol flow is described by Fig.11

MN

SA

TA

ERS

1. Current connection

2. Re-authentication

3.rMSK

4. New connection

Figure 11: Protocol flow

#### Applicability of Pro-active EAP Re-authentication

EAP re-authentication has general applicability to handovers within the same domain between different media types which use EAP as an access authentication. Similar to EAP pre-authentication, it applies to the scenarios where candidate authenticators can be easily discovered and an accurate prediction of movement can be easily made.

The efficiency of EAP re-authentication applies to the scenario where an AAA domain provides service to different media types. Re-authentication for optimized security signaling enables handovers with minimum delay and minimized communication with the home server so that the end users can maintain their quality of service level without sacrificing the scarce resources (such as battery power and bandwidth).

#### Requirements for Pro-active EAP Re-authentication

Requirement 1: MIH PoS shall support the functionalities of authenticator for EAP re-authentication.

Requirement 2: MN shall support the functionality of peer for EAP re-authentication.

Requirement 3: An authenticator discovery mechanism shall be defined. The authenticator discovery mechanism must provide a mapping between a link-layer address and an IP address of an authenticator.

Requirement 4:  A context binding mechanism shall be defined so that a link-layer specific security context is bound to the EAP keying material generated as a result of EAP re-authentication. The link-layer specific security context includes link-layer addresses of a peer and an authenticator.

Requirement 5:   Higher-layer transport shall be supported for carrying EAP re-authentication messages between MN and CA/TA and between CA/TA and ERS.

Requirement 6: Link-layer transport shall be supported for carrying EAP re-authentication messages between MN and CA/TA.

Requirement 7:  There shall be a trust relationship between each CA/TA and ERS, which may be established via mutual authentication.

Requirement 8:  If an ERS is a local authentication server, then there shall be a trust relationship between the ERS and home EAP server, which may be established via mutual authentication.

Requirement 9: There shall be a protected channel for confidentiality and integrity between each CA/TA and the ERS for the rMSK delivery.

### Potential approach for candidate authenticator discovery

The applicability of EAP pre-authentication and pro-active re-authentication is limited to the scenarios where candidate authenticators can be easily discovered. The candidate authenticator discovery mechanism can be provided by extending 802.21 standards to define new Event and Information Element used for candidate authenticator discovery.

The Event is typically generated when the MN or serving network detects a new candidate network. It can be used to trigger the MN or serving network to query the candidate authenticator’s information. The Information Element provides authenticator’s information, such as its IP and L2 address, supported proactive authentication method, etc.

## Example Call Flows

### Example Call Flow for Pro-active EAP Re-Authentication

MN

TA

ERS

[EAP Request/identity]*

[EAP Initiate /Re-auth Start]

rMSK, EAP Finish/Re-auth

EAP Initiate/Re-auth

EAP Initiate/Re-auth

EAP Finish/Re-auth

Figure 12: EAP Re-authentication message flow

The first two messages are optional. It is also possible that the re-authentication is initiated by MN (peer) since it may indicate its re-authentication capability.

# MIH Protocol Security

This section discusses and analyses MIH protocol security threats and lists the requirements and assumptions related to MIH protocol security such as MIH access control, MIH entities authentication, MIH message protection. These requirements and assumptions are either general in nature or based on and derived from specific use cases such as whether access control is available and needed or not. Also, section 3.1 provides MIH protocol security specific terminologies and also those terminologies that are used in different use cases.

Some of the general considerations while analyzing and listing different requirements, assumptions and use cases for MIH protocol security are listed below:

- Whether access to MIH services is controlled by the MIH service controller/provider or not.

- Whether AAA services are available for MIH services or not. It might not make much difference if there is a dedicated AAA service for MIH or shared AAA service with media/network.

- Whether it is possible to use any infrastructure e.g. PKI or not.

- Which transport protocol used for MIH protocol message exchange

- Whether the transport protocol used for MIH protocol message exchange are protected.

## Terminologies

Access control policy – The set of rules that define the conditions under which an access may take place.

Access control policy rules – Security policy rules concerning the provision of access control services.

Access request – The operations and operands that form part of an attempted access.

Home subscriber network - Network managed by an operator with whom the subscriber has a business relationship

Visited network - A network managed by an operator other than the subscriber’s home operator which the subscriber is receiving services

MIH Service provider - A business entity which provides MIH services.

MIH Home Service Provider - A MIH Service Provider, with whom the MN has a subscription for MIH services

MIH Access control - To limit MIH service access to the subscribers of MIH services.

MIH service access controller - An entity which executes MIH access control.

MIH service home domain - A network domain consisting of MIH PoSs which belong to the MN’s MIH Home Service Provider.

MIH service visited domain - A network domain consisting of MIH PoSs which belong to a MIH service provider, which is different from the MIH home service provider.

AAA services – Authentication, authorization and accounting services for network access, MIH access, or both access.

AAA server – A server that provides AAA services.

Trusted third party - An entity trusted by MIHF peer to provide authentication support, for example, issuing certificate for the public keys. AAA server is a trusted third party to MN and PoS when the AAA services are available.

MIH specific authentication - An entity authentication where the entity is identified as an MIH entity with a MIHF-ID.

Transport protocol for MIH services - The protocol which transports MIH messages.

MIH specific protection - To provide authenticity/integrity and confidentiality for MIH messages so that the protection is independent of the transport protocols for MIH messages. MIH specific protection is applied end-to-end between two MIHFs.

## Creating a security profile for the MIH protocol

Before providing the detail threat analysis, in section 3.4, the areas where the MIH protocol is most vulnerable is identified in this section.

|  | Vulnerabilities |
| --- | --- |
| Authentication | Lack of ways to verify the claimed identity (MIHID) of communicating MIH entities, would lead to unauthenticated access. Lack of ways for an MIH entity to authenticate the source of a given MIH message would potentially lead to issues related to data origin authentication. |
| Authorization/Access Control | Not having access control mechanism could lead to unauthorized accessing of information from the IS. Role based and/or identity based access control policies would be needed. |
| Confidentiality | The MIH protocol messages can be accessed by any entity. |
| Data Integrity | The MIH protocol messages can be modified by a malicious entity. |

## Assumptions on trust relationships

- The NN MIHF entities trust each other.

- The MN may or may not trust the PoS. The PoS may or may not trust the MN.

## Threat Analysis

Threats are identified by classifying them according to the below actions involved in vertical handover using the MIH protocol.

    - Information Query.

    - Resource availability check.

    - Resource preparation.

    - Resource release.

For each of the above 4 actions, the following threats can be identified from the vulnerabilities in Section 3.2:

- Identity Spoofing: Attempting to gain access to a system by using a false identity.

- Tampering: Unauthorized modification of data.

- Information disclosure: Unwanted exposure of private data.

- Denial of Service: The process of making a service unavailable to a user.

- Information Query:

Threat to MN

| Threats | Details of threat | Severity of threat | Countermeasures |
| --- | --- | --- | --- |
| Identity Spoofing | The IS MIH entity cannot be trusted unless its MIHID has been authenticated by the MN MIH entity An MIHF entity can also spoof an identity (masquerade attack) | Impact: High Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Not easy unless attacker had inside information or had access to network. Origin of attack: Would have to be internal. Too many layers of security for external attack. | An authentication mechanism is required to authenticate the MN and IS identities. (See Note 1) Mechanisms to provide authentication must also eliminate the threats of masquerade. |
| Tampering | Information request messages from the MN, can be modified by an attacker. (Man-in-the-middle attack). This may lead to an unexpected response from the IS. The identity of the source of a given message cannot be verified by the MIHF entity. | Impact: Denial/disruption of Service. Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external | Data origin authentication and data integrity is required between the MN and IS. |
| Information Disclosure | A particular MN MIHF may get access to unauthorized information from the IS | Impact:Potentially significant as it could be passed to potential attackers. Frequency of attack:Potentially frequent. Ease of attack: Difficult. Would need detail knowledge of network/systems/IS. Origin of attack: External or internal | A confidentiality mechanism is required between the MN and IS (either using encipherment or access control). Access control mechanisms may be used to control access to different sets of information based on the subscription. |
| Denial of Service/availability | No threat identified. | Impact: Frequency of attack: Ease of attack: Origin of attack: | - |

Threat to IS

| Threat | Details of threat | Severity of threat | Countermeasures |
| --- | --- | --- | --- |
| Identity Spoofing | The claimed MN MIHID cannot be verified by the IS. Any entity can then access information from IS meant for a particular MN. | Impact: High Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Not easy unless attacker had inside information or had access to network. Origin of attack: Would have to be internal. Too many layers of security for external attack. | An authentication mechanism is required to authenticate the MN and IS identities. (See Note 1) Mechanisms to provide authentication must also eliminate the threats of masquerade. |
| Tampering | The information sent by the IS to the MN may be modified by an attacker. | Impact: Denial/disruption of Service. Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external | Data origin authentication and data integrity is required between the MN and IS. |
| Information Disclosure | IS MIHF information may be exposed to unauthorized entities. A MN MIHF may access information designated in the IS for a particular set of subscribers. | Impact:Potentially significant as it could be passed to potential attackers. Frequency of attack:Potentially frequent. Ease of attack: Difficult. Would need detail knowledge of network/systems/IS. Origin of attack: External or internal | A confidentiality mechanism is required between the MN and IS (either using encipherment or access control). Access control mechanisms may be used to control access to different sets of information based on the subscription. |
| Denial of Service/availability | The IS is susceptible to a flooding attack by a malicious MN, leading to the IS not being able to provide information services to legitimate MNs. If the MN should be able to access some information from the IS before authenticating to it, the IS is susceptible to a DoS attack. The MN may flood the IS with requests for information not meant for un-authenticated entities. | Impact: High. DOS Attack. Frequency of attack:In-frequent Ease of attack: Difficult. Would need detail knowledge of network/systems/IS. Origin of attack: External | Access control mechanisms are needed to protect against DoS attacks. (See Notes 2, 3 and 4) The access control mechanism may or may not require authenticating the identity of the MIH entities. |

Note 1: Whether a mutual authentication or a unilateral authentication is required between two MIH entities depends on the trust relationships between them. Also, entity authentication only provides assurance of an identity at an instant of time. A way of assuring the continuity of authentication would be needed (for example by linking the authentication service with a data integrity service).

Note 2: If the MN should be able to access the IS before authenticating with it, appropriate access control policies are needed for the MN to request only information meant for it in an un-authenticated state.

Note 3:  If it is possible for the IS, based on MNs subscription, to define different sets of information available to MNs,then identity based access control policies may be defined and used.

Note 4: If the access control mechanism requires authentication of the communicating MIH entities, then, the MN cannot access IS information before authenticating to the IS. There is no threat of Denial-of-Service attack on the IS in this case. However, if there is no access control mechanism, an authentication mechanism (between MN and IS) alone cannot prevent a DoS attack on the IS. The DoS attack threat on the IS remains whether or not the MN has authenticated with the network.

- The threats and countermeasures for Resource Availability Check, Resource Preparation and Resource Release are common and can be identified as follows:

Threats to MN

| Threat | Details of threat | Severity of threat | Countermeasures |
| --- | --- | --- | --- |
| Identity Spoofing | The PoS MIH entity cannot be trusted unless its MIHID has been authenticated by the MN MIH. | Impact: DoS/Disruption of service to MN. Frequency of attack: In-frequent Ease of attack: Difficult. Attacker needs to have detailed knowledge of network/.system plus access to network? Origin of attack: Internal ? | An authentication mechanism between MN and PoS is required. This mechanism must also protect against masquerade. |
| Tampering | The events/commands sent by the PoS to the MN may be modified by an attacker. This can lead to an unexpected response at the MN. The identity of the source of a given message cannot be verified by an MIH entity. | Impact: Potential DoS/Disruption of service Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external | Data origin authentication and message integrity is required between the MN and PoS. |
| Information Disclosure | The events/commands sent by the PoS to the MN may be intercepted; but not modified by MITM attacker. | Impact:Low. The disclosure of PoS Events/Commands does not pose a critical threat. Frequency of attack: Frequent Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external | No countermeasures are needed. |
| Denial of Service/availability | Once MN has registered itself, a malicious PoS may originate events or commands thus misleading the MN. | Impact: Potential DoS/Disruption of service Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external: | Access control mechanisms are required between the MN and PoS entities to prevent DoS attacks. |

Threat to PoS(ES/CS)

| Threat | Details of threat | Severity of threat | Countermeasures |
| --- | --- | --- | --- |
| Identity Spoofing | The claimed MN MIHID cannot be verified by the PoS. Any entity can then access the PoS. An MIHF entity can also spoof an identity (masquerade attack) | Impact: High Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Not easy unless attacker had inside information or had access to network. Origin of attack: Would have to be internal. Too many layers of security for external attack. | An authentication mechanism between MN and PoS is required. This mechanism must also protect against masquerade. |
| Tampering | Messages from the MN, can be modified by an attacker. (MITM attack). This may lead to an unexpected response from the PoS | Impact: Denial/disruption of Service. Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external | Data origin authentication and message integrity is required between the MN and PoS. |
| Information Disclosure | The events/commands sent by the MN to PoS may be intercepted but not modified by Man in the middle attacker. | Impact:Low. The disclosure of MN/PoS Events/Commands does not pose a critical threat. Frequency of attack: Frequent Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: Internal or external | No countermeasures are needed. |
| Denial of Service/availability | Once MN has registered itself, a malicious MN may initiate DoS attacks. A malicious MN may send multiple registration request to PoS. | Impact: Potential DoS/Disruption of service to other MN using the PoS. Frequency of attack: Potentially quite frequent if vulnerability was publicized. Ease of attack: Difficult even if attacker had knowledge of system. Origin of attack: External: | Access control mechanisms are required between the MN and PoS entities to prevent DoS attacks. |

## General Assumptions

GA1 MIH protocol messages can use either L2 (e.g. Mobile Node to Point of

attachment) or MSTP transport (e.g. communication across different sub-network)

GA2 MIHF entity and its capabilities discovery are done either by media specific or

higher layer mechanism.

GA3 IS, ES and CS may be located in the serving, candidate or home network or it can

even be managed by a third party authority.

GA4 For the MIH protocol security perspective it would not matter whether Point of

Service (PoS) and Point of Attachment (PoA) are collocated or not.

GA5 MIH access control may or may not be applicable, e.g.

  - MIH access control is conducted through AAA server, which is

    - the same as the network access AAA server; or

    - different from the network access AAA server.

GA6   MIH specific mutual authentication may or may not be applicable

  - MIH specific mutual authentication between MN and PoS is conducted through a MIH specific centralized database, e.g. AAA server managed by MIH service provider;

  - MIH specific mutual authentication between MN and PoS is conducted through a trusted third party, e.g. a CA.

GA7   MIH specific protections may or may not be applicable

  - The mutual authentication leads to a key establishment to protect MIH messages.

  - The MIH messages are protected by transport protocols, which may or may not in place.

Assumptions GA5, GA6 and GA7 are summarized in the below flowchart:

MIH Access control?

Access Authentication (maybe mutual through access controller, e.g. service provider)

yes

no

Mutual Authentication (through a Trusted Third Party, e.g. PKI)

Key establishment  (MN and PoS)

MIH Authenticity/integrity and confidentiality

MIH specific auth?

yes

Transport Authenticity/integrity and confidentiality

MIH specific protection?

no

no

yes

The transport protections may or may not be in the place

Fig 13 Use Case Framework

## Use cases

### Use Case 1:  Access Control is applied

In this case, access control is applied through the access controller. The access control is applied through an access authentication with the MIH service provider through an authentication server, e.g. an EAP Server or an AAA server. Upon a successful authentication, the MN is authorized to access the MIH services through PoSs. Following two sub-use cases are further elaborated.

#### Use Case 1.1

MIH Access control?

Access Authentication (maybe mutual through access controller, e.g. service provider)

yes

no

Mutual Authentication (through a Trusted Third Party, e.g. PKI)

Key establishment  (MN and PoS)

MIH Authenticity/integrity and confidentiality

MIH specific auth?

yes

Transport Authenticity/integrity and confidentiality

MIH specific protection?

no

no

yes

The transport protections may or may not in the place

The access authentication includes a key establishment procedure so that keys are established between the MN and the authentication server. At least one MIH specific key will be delivered to a requesting PoS to derive MIH specific session keys to protect MIH messages. The procedure is described in Figure 14 (these are the main steps not message flows).

MN

PoS

Authentication Server

1. Request service

2. Access Authentication and Key Establishment

3. If success, Key Delivery

4. Protected MIH Protocol

Figure 14. Enable MIH Specific Protections

Notes:

- Use Case 1.1 can provide MIH level protection independent to media and network access and protection. However, it is also possible to share the authentication server and even access authentication with media and network access. In that case, the access authentication establishes keys not only for MIH but also for media or network protection.

- If EAP is used as an authentication, then the authenticator can be the PoS where MN requests for access. But when MIH access control is integrated with media access, then the authenticator can be a PoA. But in this case, after a successful authentication, PoS and PoA obtain different keys for different purposes.

- One access authentication may establish different keys to be used for multiple PoSs.

- Since MIH protection is end to end between the MN and PoS, it is independent of the transport protocol for MIH.

- Use Case 1.1 is suitable for Information Service (IS) since the PoS for IS is more centralized.

- It requires an Authentication Server (AS) and an interface between AS and PoS.

#### Use Case 1.2

MIH Access control?

Access Authentication (maybe mutual through access controller, e.g. service provider)

yes

no

Mutual Authentication (through a Trusted Third Party, e.g. PKI)

Key establishment  (MN and PoS)

MIH Authenticity/integrity and confidentiality

MIH specific auth?

yes

Transport Authenticity/integrity and confidentiality

MIH specific protection?

no

no

yes

The access authentication does not include a key establishment procedure so that keys are established between the MN and the authentication server. It depends on the transport protocol security for MIH protections.  The procedure is described in Figure 15. (These are the main steps not message flows).

MN

PoS

Authentication Server

1. Request service

2. Access Authentication

3. Success or Failure

4. MIH Protocol (protected by the transport protocol)

Figure 15. No MIH Specific Protection

Notes:

- Use Case 1.2 provide access authentication through the access controller. However, there is no MIH specific key and protection. MIH messages may be protected through the transport protocol security.

- If MIH messages are transported through a layer 2 protocol, then the protection is applied between MN and PoA. When the PoS is not a PoA, then the protection is not end to end as indicated in Figure 16.

- If MIH messages are transported through a layer 3 protocol, then the protection between MN and PoS may not be end to end but hop by hop as indicated in Figure 17.

- If the transport protocol protections are not available, then MIH messages are not protected.

Use Case Specific Threats:

- MN may communicate with a rogue PoS.

- A MN which is not eligible for the service may use the service after the authentication is completed.

- If the transport protocol protections are not available, then attackers can forge MIH messages.

MN

PoA

PoS

MIH Protocols

Layer 2 Protections

Figure 16. MIH Protocol Depends on Layer 2 Protocol for Protection

MN

L3+ Node

PoS

MIH Protocols

Layer 3 Protections

Figure 17. MIH Protocol Depends on Layer 3+ Protocol for Protection

### Use Case 2: Access Control is not applied

In this group of use cases, access control is not applied through any access controller where access controller could be MIH service provider controller or media/network service provider controller.

#### Use Case 2.1

MIH Access control?

Access Authentication (maybe mutual through access controller, e.g. service provider)

yes

no

Mutual Authentication (through a Trusted Third Party, e.g. PKI)

Key establishment  (MN and PoS)

MIH Authenticity/integrity and confidentiality

MIH specific auth?

yes

Transport Authenticity/integrity and confidentiality

MIH specific protection?

no

no

yes

The transport protections may or may not in the place

The MN and the PoS will conduct a mutual authentication and key establishment. The mutual authentication may be based on a pre-shared key or a trusted third party like certificate authority. The authentication is MIH specific. That is, the mutual authentication will assure the MIHF identity of one party to another. The keys established will bind to a pair of MIHF-IDs as well. They are used to protect MIH Messages. The procedure is described in Figure 18.

MN

PoS

1. Mutual Authentication and Key Establishment

2. Protected MIIH Protocols

Figure 18.  Pair wise MIH Level Mutual Authentication and Protections

Notes:

- Use Case 2.1 allows pair wise MIH level mutual authentication and protection. They are independent to media and network access and protection.

- Since MIH protection is end to end between the MN and PoS, it is independent of the transport protocol.

- Use Case 2.1 can treat IS, ES, and CS equally since no centralized server is involved.

- Use Case 2.1 requires a trusted third party, for example, a CA to support mutual authentication.

- Pre-shared key authentication is not scalable for large number of PoSs.

#### Use Case 2.2

MIH Access control?

Access Authentication (maybe mutual through access controller, e.g. service provider)

yes

no

Mutual Authentication (through a Trusted Third Party, e.g. PKI)

Key establishment  (MN and PoS)

MIH Authenticity/integrity and confidentiality

MIH specific auth?

yes

Transport Authenticity/integrity and confidentiality

MIH specific protection?

no

no

yes

The transport protections may or may not in the place

The MN and the PoS will not conduct mutual authentication and will not establish MIH specific keys. It depends on transport protocol to protect MIH messages.

Notes:

- If MIH messages are transported through a layer 2 protocol, then the protection applies between MN and PoA. When a PoS is not a PoA, then the protection is not end to end as indicated in Figure 16. .

- If MIH messages are transported through a layer 3 protocol, then the protection between MN and PoS may be hop by hop but not end to end as indicated in Figure 17.

- If the transport protocol protections are not available, then MIH messages are not protected.

Use Case Specific Threats:

- MN may communicate with a rogue PoS.

- If the transport protocol protections are not available, then attackers can forge MIH messages.

### Use Case 3: Visited Domain access

In this group of use cases, a MN access MIH service through a visited MIH service provider.  We assume a MIH service provider owns a set of PoSs.  However, there is no assumption with regard to whether and how different PoSs communicate.

For a MN in a visited MIH service, there are the following two situations

- The MIH protocol security policies are same as that of the home domain

- The MIH protocol security policies are different from those of home domain

We discuss them in the following two use cases.

#### Use Case 3.1

This use case covers the scenario where visited MIH services have the same security policies as those of the home domain. We will consider a few examples.

- Both home and visited MIH service have access control and may or may not generate MIH specific keys. That is, both have the situation as described in Use Case 1.1 or Use Case 1.2. In this case, in order to provide the same level of MIH security, one of the following may be true.

  - The authentication server in the visited MIH service has an interface with the authentication server in the home MIH service as pictured in Figure 19.

  - The visited PoS has an interface directly with home authentication server as pictured in Figure 20.

Both home and visited MIH service have no access control. They may or may not conduct mutual authentication and may or may not establish MIH specific keys. That is, both have the situation as described in Use Case 2.1 or Use Case 2.2. If both have MIH specific mutual authentication and key establishment, then the visited MIH service and the home MIH service must share the same trusted third party.

MN

PoSV

ASV

ASH

Figure 19: Visited PoS interfaces with Home AS through Visited AS

MN

PoSV

ASH

Figure 20: Visited PoS interfaces with Home AS Directly

Notes:

- If both home and visited MIH service have the same security policies as described in Use Case 1.1 then they may use different authentication mechanisms. However, it may require that both use the same protection algorithms so that MN can operate in the same way in a visited PoS as in the home PoS.

- If both home and visited MIH service have the same security policies as described in Use Case 1.2 then they may use different authentication mechanisms. Since the protection depends on transport protocols, different transport protocols may provide different level of security protections.

- If both home and visited MIH service have the same security policies as described in Use Case 2.1 then they shall use the same authentication mechanisms and the same protection algorithms. Since the authentication is pair wise, the MN need to interface with a visited PoS in the same way as interface with a home PoS.

- If both home and visited MIH service have the same security policies as described in Use Case 2.2, different transport protocols may provide different level of security protections since the protection depends on transport protocols.

#### Use Case 3.2

This use case was intent to cover the situation where a MN request service from a visited PoS which has different security policies with its home MIH service. Based on the discussion, it is impossible that they can have a roaming agreement for MIH servers, if two service providers have different security policies.  For the time being, we will not consider this is a valid use case until different opinions are received.