IEEE P802.22
Wireless RANs

| 802.22 WG comments on 802.11 TVWS PAR/5C |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Wendong Hu | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 | 1-408-467-8410 | Wendong.hu@st.com |

Abstract

This document contains 802.22 WG comments on 802.11 TVWS PAR/5C, Document 11-09-934-03.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Wendong Hu> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

802.22 WG comments on 802.11 TVWS PAR/5C

5.2 Scope of Proposed Standard

Comments:

- The scope should indicate that proposed TVWS operation also adopt mechanisms to prevent harmful interference to incumbent communication services. Add a state “and provide mechanisms to prevent harmful interference to incumbent communication services” at the end of the scope.

5.4 Purpose of Proposed Standard

Comments:

- The current 802.11 technology should be extended for use in the TVWS. Change the statement to be “The purpose of this amendment is to extend the 802.11 wireless network technologies for operations in the TV white space.”

5.5 Need for the Project

Comments:

- Modify the last sentence to be “This project will make the necessary MAC and PHY changes, including spectrum sensing, geo-location, and white-space database access, to enable 802.11 products to take advantage of this additional spectrum while protecting TV incumbent as per the requirements in various regulatory domains.”

17.5.1 Broad Market Potential

Comments:

- Of item c), explain the first sentence of this criterion where it is stated that the impact of the FCC requirements will be the same as that in the 5GHz bands where a more sensitive spectrum sensing threshold is required to detect TV band incumbents. As a result, this more stringent regulation will likely require a longer sensing time, a high quality RF front end, and a requirement that other unlicensed systems in the area quiet down.

17.5.3 Distinct Identity

Comments:

- On item a) “Substantially different from other IEEE 802 standards”, the 802.22 project can functionally support the FCC defined personal/portable mode II devices in the TVWS. Just as the 802.22 project specifies, the FCC requires that personal/portable mode II devices receive a new list of available channels from the incumbent database if the devices move or change locations.

- On item b) “One unique solution per problem (not two solutions to a problem)”, note that beyond the database access, being compliant with FCC part 15, subpart H rules also requires spectrum sensing.

17.5.4 Technical Feasibility

Comments:

On “Demonstrated system feasibility”:

- Demonstrate the technical feasibility that spectrum sensing can be performed as the regulatory domains require, for example the capability to sense signals at threshold levels (e.g. -114dBm) as specified by regulations.

- Demonstrate the technical feasibility of meeting the 55dBr adjacent channel rejection as specified in the FCC R&O.

- Note that sensitivity used in 5GHz is -64dBm whereas the threshold specific in the R&O 08-260 is -114dBm for a differential of 50 dB. Please explain the approach that will fill the gap.

17.5.5 Economic Feasibility

Comments on “Known cost factors, reliable data” and “Reasonable cost for performance”:

Adding support for TVWS is not as simple as changing the tuner to make the devices work in the TVWS bands. TVWS operations require additional new functionalities for example spectrum sensing (a high quality RF frontend) as stated above. Please demonstrate the cost factor for operation in the TVWS.

References: