IEEE P802.22
Wireless RANs

| Comments to the ECC Report on White Space Operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains the IEEE P802.22 comments on the ECC Report 159 on white space operation.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 1.

A minimum occupied bandwidth should be specified.  IEEE 802 believes that a 2.5 MHz minimum bandwidth requirement would ensure that WSDs provide broadband services.

Comment 2.

Additional important parameters for sensing are the probability of detection and the probability of false alarm.

Comment 3.

The ‘TV bands database system’ plan and operation should be augmented to allow online, near real time operation.  We believe that the database push functionality, as well as pull functionality, should be a requirement.  Many Internet transaction systems operate continuously, with redundant elements and very high reliability.  Our recommendation is to have each master mode device provide an Internet contact address, and the TV bands database push changes in channel availability to affected master devices. We believe that the 24 hour grace period for continued operation can lead to an extended duration of uncontrollable interference. In our recommendation, the requirement for daily contact with the TV bands database is replaced by the requirement for master mode devices to verify their Internet connectivity hourly or cease operation. There are a broad range of standards-based paging and messaging technologies available that the TV bands database could use to push messages that reflect changes in channel availability to master mode devices for near real time updates. We also suggest that security related issues should be adequately considered in database implementation. The security aspects include ensuring database availability, authorization to access, authentication, encryption, data integrity, non-repudiation, confidentiality and privacy.

Comment 4.

The WSD should be required to supply a certification of identity in order to be granted operational access by the database.  Reciprocally the database should be required to supply a certification of identity in order for the WSD to ascertain that the WSD is being controlled by an officially approved database. This will ensure mutual authentication to avoid spoofing or Denial of Service (DoS) attacks.

Comment 5.

The IEEE 802.22 standard project (www.ieee802.org/22) is designing a PHY / MAC interface to provide cognitive radio (CR) based wireless access in the TV White spaces. The IEEE 802.22 project on cognitive radio based Wireless Regional Area Networks (WRANs) has three projects under its umbrella.

The IEEE 802.22.1-2010 standard on enhanced protection of low power licensed devices was completed and published in Nov. 2010. This standard specifies a beaconing protocol to be transmitted in a bandwidth of 77 kHz to indicate the presence of wireless microphones.

The IEEE 802.22 project has a current Working Group approval ratio of 100% and is on-track to be completed in the 2011 time-frame (IEEE Sponsor Ballot process is expected to begin in Dec. 2010).  The WRANs for which this standard is being developed are expected to provide broadband access to data networks using vacant channels in the VHF and UHF bands allocated to the Television Broadcasting Service in the frequency range between 54 MHz and 862 MHz while avoiding interference to the broadcast incumbents in these bands.  WSDs connect in a point-to-multipoint architecture where the customer premises equipments (CPEs) are controlled by the cetralized base station (BS).  The BS not only controls the operational parameters of all its registered CPEs but also acts as a proxy to the database service.  A CPE reports its location information to the BS and the BS uses this information to query for available TV channels from the database on behalf of the CPE.  The BS manages TV channel selection for its network using an integrated cognitive engine – called the “spectrum manager.” Each of the devices on the network is capable of spectrum sensing.  The exact spectrum sensing capability employed is left open to implementation.  However, the BS keeps track of the statistical performance of the spectrum sensing capabilities.  These statistics along with the individual sensing measurements made by the individual devices on the network can be combined to accomplish distributed sensing.

The IEEE P802.22 project was originally established to address the opportunity of using spectrum available in the TV Whitespaces in one country but the mandate of the project was quickly expanded to pursue worldwide applicability by including the 3 typical TV channel bandwidths: 6, 7, and 8 MHz, fixed and portable operation, thus making the standard potentially suitable for all the countries in the world. A typical WRAN application is a point-to-multipoint broadband access system covering a rural area around a village, within a radius of 10 km to 30 km from the base station depending on its EIRP and antenna height with the PHY implemented in this standard.  With the MAC implemented in this standard, WRAN systems can cover up to a radius of 100 km with proper scheduling of the traffic in the frame. A Base Station (BS) complying with this standard is able to provide high-speed Internet service for up to 512 fixed or portable WSDs or groups of devices per TV channel within its coverage area, while meeting the regulatory requirements for protection of the incumbents. The standard includes cognitive radio techniques to mitigate interference to incumbents, including geolocation capability, provision to access a database of incumbent services, and spectrum-sensing technology to detect the presence of incumbent services, other WRAN systems and IEEE 802.22.1 wireless beacons. Special consideration was given to security for data transmission as well as cognitive functions in the standard.

The IEEE 802.22 standards project has recognized the challenge in protecting wireless microphone usage during its design and compatibility research.  Spectrum sensing is more suitable than database registration to protect wireless microphone usage due to the dynamic and unpredictable nature of on-location news and emergency reporting.  However, the detection range of the low-power signals is unacceptably shorter than the range of interference potential from a 4 W EIRP WSD.  The IEEE 802.22 standards project has accounted for this difficulty by publishing the IEEE 802.22.1-2010 standard for enhanced protection of low-power devices.  IEEE 802.22.1 specifies a beaconing system that indicates the presence of protected wireless microphones in an area.  The IEEE 802.22 protocol detects and reacts to the IEEE 802.22.1 beacon.

Finally, the IEEE 802.22.2 project is developing a recommended practice for the installation and deployment of wireless regional area networks in the TV bands.

References: