IEEE P802.22 Wireless RANs

| [802.22b CPE initialization] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-03-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Changwoo Pyo | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | cwpyo@nict.go.jp |
| Zhang Xin | NICT | 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore |  | amy.xinzhang@ieee.org |
| Chunyi Song | NICT | 3-4 Hikarion-Oka, Yokosuka, Japan |  | songe@ieee.org |
| Keiichi Mizutani | NICT | 3-4 Hikarion-Oka, Yokosuka, Japan |  | mizk@nict.go.jp |
| Pin-Hsun Lin | NICT | 3-4 Hikarion-Oka, Yokosuka, Japan |  | pslin@nict.go.jp |
| Gabriel Porto Villardi | NICT | 3-4 Hikarion-Oka, Yokosuka, Japan |  | gpvillardi@nict.go.jp |
| Masayuki Oodo | NICT | 3-4 Hikarion-Oka, Yokosuka, Japan |  | moodo@nict.go.jp |
| Hiroshi Harada | NICT | 3-4 Hikarion-Oka, Yokosuka, Japan |  | harada@ieee.org |

Abstract

This document is a revision of initialization and network association (7.14) for 802.22b systems and provies definitions related with the revision.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary

R1

Modify terms

Modify initial ranging via centralized scheduling R-CPE

Define messages

7.14 Initialization and network association

7.14.2 CPE initialization

Figure 33 illustrates a scenario where the need for the definition of an incumbent safe CPE initialization can be easily seen. In this figure, consider that CPE 4 is powered down whereas the BS is transmitting in the cell that is under normal operation. Further, assume that the TV station in Figure 33 is powered up and starts transmitting in the same channel (i.e., channel #N in this example) that is being used by the BS for its transmissions in the cell. CPE 4 should be capable of detecting that the BS is operating in a channel that is occupied by an incumbent service. The BS must be capable of determining if CPE 4 is located within interference range of the TV station protected contour (i.e., in the keep-out region). If the CPE4 is already registered with the BS, it will alert the BS. If the CPE4 is not registered with the BS, it shall not transmit. See 10.2.5, policies 5 and 6. In response to the alert from the CPE, the SM at the BS may or may not decide to switch channel to accommodate the CPE (see 10.2.6.6). The purpose of the sensing and geolocation capabilities of the WRAN system shall be to prevent harmful interference to the primary TV service by providing the necessary information to the BS’s SM that generates the list of available channels. The definition of an incumbent safe CPE initialization phase is critical for cognitive radio systems. The SM incorporates algorithms to address this need (see Table 234, policies 5 and 6).

First and foremost, the MAC does not presuppose any preassigned channel where a CPE is able to look for a BS given the time-varying and unpredictable nature of channel occupancy. Hence, the first task a CPE must perform in attempting to join a network is to scan the set of channels for BSs and incumbent services on which the transmissions of the CPE might interfere. Since the BS shall send concentrated OFDM symbols composed of a superframe preamble, a frame preamble and an SCH once every superframe in its operating channel (see 7.3), the CPE will recognize the existence of a BS transmission and, if appropriate, proceed with the CPE initialization procedure with the corresponding BS.

Figure 33a – 802.22b network scenario where a safe bootstrap operation is required to protect incumbents

Figure 33a illustrates an 802.22b network scenario where the need for the definition of an incumbent safe CPE initialization can be easily seen. In this figure, consider that S-CPE 4, which is located outside of a MR-BS’s cell but located within a distributed scheduling R-CPE 2’s local cell, is powered down whereas the MR-BS is transmitting in the cell and the R-CPE 2 being a member of the MR-BS is transmitting in the local cell that are under normal operation. Further, assume that the TV station in Figure 33a is powered up and starts transmitting in the same channel (i.e., channel #N in this example) that is being used by the MR-BS and R-CPE 2 for its transmissions in the cell. S-CPE 4 should be capable of detecting that the R-CPE 2 is operating in a channel that is occupied by an incumbent service. The MR-BS must be capable of determining if S-CPE 4 is located within interference range of the TV station protected contour (i.e., in the keep-out region). If the S-CPE 4 is already registered with the network managed by the R-CPE 2, it will alert the R-CPE 2. If the S-CPE 4 is not registered with the network, it shall not transmit. See 10.2.5, policies 5 and 6. In the response to the alert from the S-CPE 4, the SM at the R-CPE 2 performs to detect TV station and shall sends the nofitification of detecting TV station to the MR-BS. In response to the notification from the R-CPE 2, the SM at the MR-BS may or may not decide to switch channel to accommodate the connected CPEs (see 10.2.6.6). The purpose of the sensing and geolocation capabilities of the WRAN system shall be to prevent harmful interference to the primary TV service by providing the necessary information to the MR-BS’s SM that generates the list of available channels. The definition of an incumbent safe CPE initialization phase is critical for cognitive radio systems. The SM incorporates algorithms to address this need (see Table 234, policies 5 and 6).

First and foremost, the MAC does not presuppose any preassigned channel where a CPE is able to look for an MR-BS or adistributed scheduling R-CPE given the time-varying and unpredictable nature of channel occupancy. Hence, the first task a CPE must perform in attempting to join a network is to scan the set of channels for MR-BSs or R-CPEs and incumbent services on which the transmissions of the CPE might interfere. Since the MR-BS shall send concentrated OFDM symbols composed of a frame preamble and an FCH once every frame in its operating channel and the distributed scheduling R-CPE shall send concentrated OFDM symbols composed of a local frame preamble and a DRZ-FCH in the downstream DRZ subframe within a frame, if available, in its operating channel (see 7.3), the CPE will recognize the existence of an MR-BS or a distributed scheduling R-CPE transmission and, if appropriate, proceed with the CPE initialization procedure with the corresponding MR-BS or distributed scheduling R-CPE. Although a CPE will recognize the existence of an MR-BS, in particular, the CPE may not be initialized with the MR-BS directly since the transmission of the CPE is not able to reach the MR-BS due to the power constraint. In this case, the CPE will make an initialization by relaying on a centralized scheduling R-CPE.

The procedure carried out by the MR-BS, the centralized scheduling R-CPE, the distributed scheduling R-CPE and the CPE to perform CPE network entry and initialization shall be as follows:

- CPE performs self test.

- CPE acquires the antenna gain information.

- CPE senses for and synchronizes to WRAN services. The sensing thread also begins during this step to detect broadcasting incumbents.

- CPE presents sensing results to the higher layers.

- CPE chooses a WRAN service.

- If CPE is capable of geolocation, CPE acquires valid geolocation data from the satellites. If the data acquisition is unsuccessful, CPE initialization should not continue or may continue to operate as an S-CPE mode (FCC Mode I). If CPE is not capable of geolocation, CPE initialization should not continue or may continue to operate as an S-CPE mode (FCC Mode I).

- CPE acquires the downstream and upstream parameters from the selected WRAN service.

- CPE directional antenna azimuth adjustment.

- If channels N and N±1 pass the sensing and timing requirements, CPE perform initial ranging (see 7.15.2.1).

- CPE transmits basic capabilities.

- If all required basic capabilities are present in the CPE, the AAA authenticates the CPE and key exchange is performed; otherwise, the CPE does not proceed to registration and the MR-BS de-registers the CPE.

- Perform Registration (REG-REQ/RSP).

- Upon completing registration, MR-BS transmits channel sets to CPE.

- Establish IP connectivity.

- Establish time of day.

- Transfer operational parameters.

- Establish dynamic service flows.

- CPE reports sensing results and discovered neighboring networks.

Figure 34a summarizes the network entry of the CPE and its initialization procedure. Note that these steps taken by the CPE consist of a set of actions and error verification. In the following subclauses, a more detailed description of these steps and their individual responsibilities are provided.

Figure 34a — CPE initialization procedure

7.14.2.1 CPE performs self test

On initialization or after signal loss, the CPE shall perform a self test.

7.14.2.2 CPE antenna gain information acquisition

The CPE shall determine if its antenna is integrated or not by querying it using the M-ANTENNA- INTEGRATED primitive structure described in 10.7.6.1 and 10.7.6.2. The CPE shall acquire the antenna information including the maximum antenna gain information for the channels that can be used in the regulatory domain of interest. This information is stored in a MIB, wranIfBsCpeAntennaGainTable. If the antenna is integrated to the CPE TRU, this MIB object shall be pre-populated by the manufacturer of the CPE. If the antenna is not integrated into the CPE TRU, the MIB object shall be populated by querying the AU through the interface defined in 9.12.2. The information at the antenna shall be pre-populated by the antenna manufacturer.

7.14.2.3 CPE senses for and identifies WRAN services and incumbents

The CPE shall perform spectrum sensing to detect the BS and to detect and identify legitimate incumbent services that are to be protected on each active WRAN channel in the area and its adjacent channels as described in 10.3.2.

The CPE identifies WRAN services from detecting the BS/MR-BS or the distributed scheduling R-CPE. The CPE shall perform spectrum sensing to detect the BS/MR-BS or the distributed scheduling R-CPE, and may perform spectrum sensing to detect and identify legitimate incumbent services that are to be protected on each active WRAN channel in the area and its adjacent channels as described in 10.3.2.

7.14.2.4 Present sensing results to the higher layers

As a result of spectrum sensing, the available BSs in the area are presented to the application layer program via connection C2 and MIBs through M-SAP as shown in IEEE 802.22 reference architecture (Figure 7). The application may be running on the CPE or on an attached computer. The data presented includes the operating channel of the BS and RSSL in addition to the WRAN service being advertised.

As a result of spectrum sensing, the available BSs/MR-BSs or distributed scheduling R-CPEs in the area are presented to the application layer program via connection C2 and MIBs through M-SAP as shown in IEEE 802.22 reference architecture (Figure 7). The application may be running on the CPE or on an attached computer. The data presented includes the operating channel of the BS/MR-BS and RSSL in addition to the WRAN service being advertised.

7.14.2.5 CPE chooses a WRAN service

A WRAN service is selected at the higher layers of the CPE after preliminary sensing and identification of available BSs and the presence of incumbents in the area as the previous subclauses describe. The CPE SSA shall issue an M-WRAN-SERVICE-REPORT primitive to request the higher layers through the NCMS to select a channel from the available WRAN service list that is included in the primitive, as described in 10.7.4.1. The SSA shall receive an M-WRAN-SERVICE-RESPONSE primitive with the selected channel from the NCMS, as described in 10.7.4.3. Once the channel is selected, it and its adjacent channels are more rigorously sensed in order to detect the presence of a weak incumbent service that might be masked by the selected WRAN service. This procedure is described in more detail in 10.3.2.

A WRAN service is selected at the higher layers of the CPE after preliminary sensing and identification of available BSs/MR-BSs or distributed scheduling R-CPEs and the presence of incumbents in the area as the previous subclauses describe. The CPE SSA shall issue an M-WRAN-SERVICE-REPORT primitive to request the higher layers through the NCMS to select a channel from the available WRAN service list that is included in the primitive, as described in 10.7.4.1. The SSA shall receive an M-WRAN-SERVICE-RESPONSE primitive with the selected channel from the NCMS, as described in 10.7.4.3. Once the channel is selected, it and its adjacent channels are more rigorously sensed in order to detect the presence of a weak incumbent service that might be masked by the selected WRAN service. This procedure is described in more detail in 10.3.2.

7.14.2.6 CPE performs satellite-based geolocation

The CPE shall acquire geolocation data from a satellite-based geolocation receiver. A CPE shall not progress to the next step of initialization until the satellite-based geolocation technology successfully establishes lock and acquires valid geolocation data from the satellites. The CPE sends the NMEA string to the BS during registration (see 7.14.2.11).

The CPE shall acquire geolocation data from a satellite-based geolocation receiver when it will operate as a fixed mode or as a distributed scheduling R-CPE (which is a mode II defined in FCC regulation). A CPE shall not progress to the next step of initialization for the fixed mode operation or the distributed scheduling R-CPE operation until the satellite-based geolocation technology successfully establishes lock and acquires valid geolocation data from the satellites. The CPE sends the NMEA string to the BS/MR-BS during registration (see 7.14.2.11).

|  | Fixed mode | Mode I | Mode II |
| --- | --- | --- | --- |
| BS/MR-BS | 〇 (4W EIRP, Geolocation) | ✖ | ✖ |
| Distributed Scheduling R-CPE | 〇 (4W EIRP, Geolocation) | ✖ | 〇 (100mW EIRP, Geolocation) |
| Centralized Scheduling R-CPE | 〇 (4W EIRP, Geolocation) | 〇 (40mW EIRP) | ✖ |
| S-CPE | 〇 (4W EIRP, Geolocation) | 〇 (40mW EIRP) | ✖ |

7.14.2.7 Acquire downstream and upstream parameters

There are two methods for acquiring downstream and upstream parameters; acquire downstream and upstream parameters from a BS/MR-BS or acquire downstream and upstream parameters from a distributed scheduling R-CPE.

7.14.2.7.1 Obtaining downstream parameters (from BS/MR-BS)

The MAC shall search for the SCH from the BS, which indicates the beginning of the superframe in normal mode, and the first allocated frame of the superframe in self-coexistence mode. To improve the joining latency, the CPE shall use energy detection to help ascertain about the presence/absence of an IEEE 802.22

BS in a particular channel. If the energy detected is below the detection threshold, the CPE can safely move to the next channel.

After having received an SCH in a channel, the CPE shall perform sensing not only in the set of channels indicated in the SCH, but also in all other affected channels. During this sensing, the CPE shall attempt to identify incumbent operation. If incumbents are detected on the operating channel or either first adjacent channel, the MAC shall cause the CPE to cease transmitting application traffic on the channel and, at the first transmit opportunity send a short control message to the BS indicating that it is using a channel occupied by an incumbent. In case the BS receives such notification, it may take numerous actions as described in Figure 96. The aggregate duration of the short control messages shall not exceed the Channel Closing Transmission Time (see Table 276) of transmissions by the WRAN system before remedying the interference condition (changing channels, backing off transmit EIRP, terminating transmissions, etc.).

Provided no incumbents are found, the CPE may proceed to the next step. Here, the MAC shall search for the DS-MAP MAC management messages. The CPE achieves MAC synchronization once it has received at least one DS-MAP message. A CPE MAC remains in synchronization as long as it continues to successfully receive the SCH, DS-MAP, and DCD messages for its channel(s). If the Lost DS-MAP Interval (Table 273) has elapsed without a valid DS-MAP message or the T1 interval (Table 273) has elapsed without a valid DCD message or Lost SCH counts of SCH are missed, a CPE shall try to re- establish synchronization. The process of acquiring synchronization is illustrated in Figure 35. The process of maintaining synchronization is illustrated in Figure 36.

The MAC shall search for the FCH from the BS/MR-BS, which indicates the beginning of the frame in normal mode, and the allocated frame in self-coexistence mode. To improve the joining latency, the CPE shall use energy detection to help ascertain about the presence/absence of a BS/MR-BS in a particular channel. If the energy detected is below the detection threshold, the CPE can safely move to the next channel.

After having received FCH in a channel, the CPE shall perform sensing not only in the detected operating channel, but also in all other affected channels. During this sensing, the CPE shall attempt to identify incumbent operation. If incumbents are detected on the operating channel or either first adjacent channel, the MAC shall cause the CPE to cease transmitting application traffic on the channel and, at the first transmit opportunity send a short control message to the BS/MR-BS indicating that it is using a channel occupied by an incumbent. In case the BS/MR-BS receives such notification, it may take numerous actions as described in Figure 96. The aggregate duration of the short control messages shall not exceed the Channel Closing Transmission Time (see Table 276) of transmissions by the WRAN system before remedying the interference condition (changing channels, backing off transmit EIRP, terminating transmissions, etc.).

Provided no incumbents are found, the CPE may proceed to the next step. Here, the MAC shall search for the DS-MAP MAC management messages. The CPE achieves MAC synchronization once it has received at least one DS-MAP message. A CPE MAC remains in synchronization as long as it continues to successfully receive theFCH, DS-MAP, and DCD messages for its channel(s). If the Lost DS-MAP Interval (Table 273) has elapsed without a valid DS-MAP message or the T1 interval (Table 273) has elapsed without a valid DCD message or Lost FCH counts of FCH are missed, a CPE shall try to re-establish synchronization. The process of acquiring synchronization is illustrated in Figure 35a. The process of maintaining synchronization is illustrated in Figure 36.

7.14.2.7.1a Obtaining downstream parameters (from distributed scheduling R-CPE)

As another method to obtain downstream parameters, the MAC may search for a distributed relay zone FCH (DRZ-FCH) transmitted from a distributed scheduling R-CPE, which indicates the beginning of the distributed relay zone of downstream. To improve the joining latency, the CPE shall use energy detection to help ascertain about the presence/absence of a distributed scheduling R-CPE in a particular channel. If the energy detected is below the detection threshold, the CPE can safely move to the next channel.

After having received a DRZ-FCH in a channel, the CPE shall perform sensing not only in the  detected operating channel but also in all other affected channels. During this sensing, the CPE shall attempt to identify incumbent operation. If incumbents are detected on the operating channel or either first adjacent channel, the MAC shall cause the CPE to cease transmitting application traffic on the channel and, at the first transmit opportunity in a distributed relay zone (DRZ) of upstream send a short control message to the distributed scheduling R-CPE indicating that it is using a channel occupied by an incumbent. In case that the distributed scheduling R-CPE receives such notification, it shall send a short control message to the MR-BS. In case the MR-BS receives such notification, it may take numerous actions as described in Figure 96. The aggregate duration of the short control messages shall not exceed the Channel Closing Transmission Time (see Table 276) of transmissions by the WRAN system before remedying the interference condition (changing channels, backing off transmit EIRP, terminating transmissions, etc.).

Provided no incumbents are found, the CPE may proceed to the next step. Here, the MAC shall search for the DS-MAP MAC management messages, which are transmitted from the distributed scheduling R-CPE, in a DRZ of downstream. The CPE achieves MAC synchronization to the distributed scheduling R-CPE once it has received at least one DS-MAP message. A CPE MAC remains in synchronization as long as it continues to successfully receive the DRZ-FCH,  DS-MAP, and  DCD messages for its channel(s) within a DRZ. If the lost DS-MAP Interval (Table 273) has elapsed without a valid  DS-MAP message or the T1 interval (Table 273) has elapsed without a valid  DCD message or lost DRZ-FCH counts of DRZ-FCH are missed, a CPE shall try to re-establish synchronization. The process of acquiring synchronization is illustrated in Figure 35. The process of maintaining synchronization is illustrated in Figure 36a.

Figure 35a – Obtaining downstream parameters

Figure 36 – Maintaining downstream parameters

Figure 36a – Maintaining downstream parameters in DRZ

7.14.2.7.2 Obtaining upstream parameters (from BS/MR-BS)

After synchronization, the CPE shall wait for a UCD message from the BS in order to retrieve a set of transmission parameters for a possible upstream channel. These messages are transmitted periodically from the BS for all available upstream channels and are addressed to the MAC broadcast address.

If no upstream channel can be found after a suitable timeout period, then the CPE shall continue scanning to find another downstream channel. The process of obtaining upstream parameters is illustrated in Figure

37.

The  CPE  shall  determine  from  the  channel  description  parameters  whether  it  may  use  the  upstream channel. If the channel is not suitable, then the CPE shall continue scanning to find another downstream channel. If the channel is suitable, the CPE shall extract the parameters for this upstream from the UCD. It then shall wait for the next DS-MAP message and extract the time synchronization from this message. Then, the CPE shall wait for a bandwidth allocation map for the selected channel. It may begin transmitting upstream in accordance with the MAC operation and the bandwidth allocation mechanism.

The CPE shall perform initial ranging at least once. If initial ranging is not successful, the procedure is restarted from scanning to find another downstream channel.

The CPE MAC is considered to have valid upstream parameters as long as it continues to successfully receive the SCH, US-MAP, and UCD messages. If at least one of these messages is not received within the time intervals specified in Table 273, the CPE shall not use the upstream. This is illustrated in Figure 38.

After synchronization to the BS/MR-BS, the CPE shall wait for a UCD message from the BS/MR-BS in order to retrieve a set of transmission parameters for a possible upstream channel. These messages are transmitted periodically from the BS/MR-BS for all available upstream channels and are addressed to the MAC broadcast address.

If no upstream channel can be found after a suitable timeout period, then the CPE shall continue scanning to find another downstream channel. The process of obtaining upstream parameters is illustrated in Figure 37.

The CPE shall determine from the channel description parameters whether it may use the upstream channel. If the channel is not suitable, then the CPE shall continue scanning to find another downstream channel. If the channel is suitable, the CPE shall extract the parameters for this upstream from the UCD. It then shall wait for the next DS-MAP message and extract the time synchronization from this message. Then, the CPE shall wait for a bandwidth allocation map for the selected channel. It may begin transmitting upstream in accordance with the MAC operation and the bandwidth allocation mechanism.

The CPE shall perform initial ranging at least once. If initial ranging is not successful, the procedure is restarted from scanning to find another downstream channel.

The CPE MAC is considered to have valid upstream parameters as long as it continues to successfully receive theFCH, US-MAP, and UCD messages. If at least one of these messages is not received within the time intervals specified in Table 273, the CPE shall not use the upstream. This is illustrated in Figure 38.

7.14.2.7.2a Obtaining upstream parameters (from distributed scheduling R-CPE)

After synchronization to the distributed scheduling R-CPE, the CPE shall wait for a UCD message from the distributed scheduling R-CPE in order to retrieve a set of transmission parameters for a possible upstream channel. These messages are transmitted periodically in a DRZ of downstream from the distributed scheduling R-CPE for the available upstream channels and are addressed to the MAC broadcast address.

If no upstream channel can be found after a suitable timeout period, then the CPE shall continue scanning to find another downstream channel. The process of obtaining upstream parameters is illustrated in Figure 37.

The CPE shall determine from the channel description parameters whether it may use the upstream channel. If the channel is not suitable, then the CPE shall continue scanning to find another downstream channel. If the channel is suitable, the CPE shall extract the parameters for this upstream from the UCD. It then shall wait for the next  DS-MAP message and extract the time synchronization from this message. Then, the CPE shall wait for a bandwidth allocation map for the selected channel. It may begin transmitting upstream in accordance with the MAC operation and the bandwidth allocation mechanism.

The CPE shall perform initial ranging to the distributed scheduling R-CPE at least once. If initial ranging is not successful, the procedure is restarted from scanning to find another downstream channel.

The CPE MAC is considered to have valid upstream parameters as long as it continues to successfully receive the DRZ-FCH,  US-MAP, and UCD messages. If at least one of these messages is not received within the time intervals specified in Table 273x, the CPE shall not use the upstream. This is illustrated in Figure 38a.

Figure 38a – Maintaining upstream parameters

7.14.2.8 CPE transmits ranging/CDMA burst

The selected channel is analyzed to determine if it passes the restrictions specified in 10.3.2. If the selected channel does not pass these restrictions, the association with the selected BS is unsuccessful and the selected channel shall be removed from further consideration. Available BSs are again presented to the higher layers for selection if there exist any other BSs with which to associate.

Next the selected channel and the channels that could be harmfully interfered by operation on this selected channel shall be more finely sensed as to determine if there exists a weak protected incumbent signal that was not detected at an earlier stage in the CPE initialization procedure. This process is described in 10.3.2.

Time in this subclause shall be referenced to two positions in space. One position will be that of the BS and the other position will be that of the CPE. Many such CPE positions will exist. Ranging is the process of acquiring the correct timing offset and EIRP adjustments such that the CPE’s transmissions are aligned at the BS position. Ranging also adjusts transmit EIRP of the various CPEs such that the OFDMA signal received at the BS arrives with compatible amplitudes from all the CPEs. The timing delays through the PHY shall be constant to within 25% of the shortest symbol cyclic prefix as indicated in 9.9.1.

From the result of synchronization as described in 7.14.2.7, initial ranging will be performed. There are three methods of CPE transmit initial ranging; initial ranging to a BS/MR-BS, initial ranging to a distributed scheduling R-CPE, and inital ranging to a centralized scheduling R-CPE for relaying.

The selected channel is analyzed to determine if it passes the restrictions specified in 10.3.2. If the selected channel does not pass these restrictions, the association with the selected BS/MR-BS or distributed scheduling R-CPE is unsuccessful and the selected channel shall be removed from further consideration. Available BSs/MR-BSs or distributed scheduling R-CPEs are again presented to the higher layers for selection if there exists any other BSs/MR-BSs or distributed scheduling R-CPEs with which to associate.

Next the selected channel and the channels that could be harmfully interfered by operation on this selected channel shall be more finely sensed as to determine if there exists a weak protected incumbent signal that was not detected at an earlier stage in the CPE initialization procedure. This process is described in 10.3.2.

Time in this subclause shall be referenced to two positions in space. One position will be that of the BS/MR-BS or distributed scheduling R-CPE and the other position will be that of the CPE. Many such CPE positions will exist. Ranging is the process of acquiring the correct timing offset and EIRP adjustments such that the CPE’s transmissions are aligned at the MR-BS or distributed scheduling R-CPE position. Ranging also adjusts transmit EIRP of the various CPEs such that the OFDMA signal received at the BS/MR-BS or distributed scheduling R-CPE arrives with compatible amplitudes from all the CPEs.

Although a CPE successfully obtains downstream parameters from a BS/MR-BS, in particular, ranging to the BS/MR-BS as described in 7.14.2.8.1 may be failed due to the CPE transmitting power constraint. However, the CPE is still able to have an uplink transmission to the BS/MR-BS by relaying on a centralized scheduling R-CPE. In this case, the CPE may perform ranging to a centralized scheduling R-CPE for the relaying operations that acquiring the correct timing offset and EIRP adjustments aligned at the centralized scheduling R-CPE.

The timing delays through the PHY shall be constant to within 25% of the shortest symbol cyclic prefix as indicated in 9.9.1.

7.14.2.8.1 CDMA initial ranging and automatic adjustments (to BS/MR-BS)

First, a CPE shall synchronize to the downstream superframe and frame preamble. At this point, the CPE shall scan the US-MAP message to find an Initial Ranging Interval. The BS may allocate an Initial Ranging Interval consisting of one or more transmission opportunities. The CPE shall extract the number of initial ranging codes (see Table 31, element ID 150) from the UCD MAC management message.

The CPE randomly selects the CDMA code as described in 7.15.2.1 and sends the initial ranging CDMA code on the US allocation dedicated for that purpose. The BS receives the CDMA code. As many CPEs may contend for ranging, the CDMA code received may be the sum of many CPE transmissions. The BS isolates each of these transmissions and computes the ranging adjustments based on the relative time of

arrival of each CPE upstream burst, i.e., the timing offset, so that all these bursts arrive at the BS at the beginning of the symbol period within sufficient tolerance.

Ranging adjusts each CPE’s timing offset such that each CPE appears to be co-located with the BS. The CPE shall set its initial timing offset to “zero advance” as if it was physically co-located with the BS. When the Initial Ranging transmission opportunity occurs, the CPE shall send a CDMA code. After reception and decoding of this CDMA code, the BS will react by sending a RNG-CMD MAC message in a following frame with the same CDMA code and indicate the timing advance that the CPE should use for its upstream transmissions (see Table 46) so that the beginning of its bursts is aligned with the center of the cyclic prefix within the tolerance indicated in 9.9.1.

When the Initial Ranging transmission opportunity occurs, the CPE shall send a CDMA code. Thus, the CPE sends the message as if it were co-located with the BS.

The CPE shall calculate the transmit EIRP per subcarrier for initial ranging, EIRPIR_CPE, from the following equation:

EIRPIR_CPE = EIRPBS + RSSIR_BS_nom – (RSSIR_CPE – GRX_CPE) + 10×log(NIR_sub/1680)

where

RSSIR_BS_nom and EIRPBS are defined in a DCD IE (see Table 23) GRX_CPE is the antenna gain at the CPE RSSIR_CPE  is the RSSL measured by the CPE, which is then corrected by the CPE antenna gain to represent the RSSL for an isotropic antenna NIR_sub is the number of subcarriers used by the CPE for initial ranging

NOTE—The value of RSSIR_BS_nom corresponds to the nominal signal strength that should be measured at the output of a 0 dBi gain receive antenna at the BS for normal operation. The EIRPBS is the equivalent isotropic radiated power of the base station in the direction of the CPE, which would be computed for a simple single-antenna transmitter as PTX_BS

.

+ GTX_BS, where PTX_BS is the transmit power and GTX_BS is the BS transmit antenna gain in the direction of the CPE.14

The CPE shall send a CDMA code with a power level resulting in the EIRPIR_CPE per subcarrier. If the CPE does not receive a response after waiting at least one frame to allow processing at the BS, the CPE shall send a new CDMA code at the next appropriate Initial Ranging transmission opportunity with 1 dB higher power level. The CPE shall, however, stop increasing the power level at the following condition:

EIRPIR_MAX +10×log(NIR_sub) > EIRPCPE_MAX

where

EIRPCPE_MAX        is the upper bound in maximum transmitted EIRP for the CPE on the current operating channel as described in Table 108 of 7.7.11.3.2.1 or 4 Watt whichever is the smallest EIRPIR_CPE_MAX   is the upper bound for the increased EIRPIR_CPE

If the CPE receives a RNG-CMD message containing the parameters of the code it has transmitted and the status “continue,” it shall consider the transmission attempt unsuccessful but implement the corrections specified in the RNG-CMD and issue another CDMA code after the appropriate backoff delay. If the CPE receives an US-MAP containing a CDMA allocation IE with the parameters of the code it has transmitted, it shall consider the RNG-CMD reception successful, and proceed to send a unicast RNG-REQ (on Initial Ranging FID, allocated to Cell SID) on the allocated BW.

Once the BS has successfully received the RNG-REQ message, it shall return a RNG-CMD message using the initial ranging connection (see 12.2). Within the RNG-CMD message shall be the Station ID (SID) assigned to this CPE. The message shall also contain information on the required CPE EIRP level, offset frequency adjustment as well as the proper timing advance when needed. At this point the BS shall start using invited Initial Ranging Intervals addressed to the CPE’s Basic FID to complete the ranging process, unless the status of the RNG-CMD message is “success,” in which case the initial ranging procedure shall end.

If the status of the RNG-CMD message is “continue,” the CPE shall wait for an individual Initial Ranging Interval assigned to its Basic FID. Using this interval, the CPE shall transmit another RNG-REQ message using the Basic FID along with any power level and timing offset corrections.

The BS shall return another RNG-CMD message to the CPE with any additional fine-tuning required. The ranging request/response steps shall be repeated until the response contains a “Ranging Successful” notification or the BS aborts ranging. Once successfully ranged (timing, frequency and EIRP are within tolerance at the BS), the CPE shall join normal data traffic in the upstream. In particular, the retry counts and timer values for the ranging process are defined in Table 273.

NOTE 1—The burst profile to use for any upstream transmission is defined by the Upstream Interval Usage Code

(UIUC). Each UIUC is mapped to a burst profile in the UCD message.

NOTE 2—The BS shall allow the CPE sufficient time to have processed the previous RNG-CMD (i.e., to modify the transmitter parameters) before sending the CPE a specific ranging opportunity. This is defined as CPE Ranging Response Processing Time in Table 273.

On receiving a RNG-CMD instruction to move to a new channel during initial ranging, the CPE shall obtain a new SID via initial ranging and registration.

It is possible that the RNG-CMD may be lost after transmission by the BS. The CPE shall recover by timing out and reissuing its Initial RNG-REQ. Since the CPE is uniquely identified by the source MAC address in the Ranging Request, the BS may immediately reuse the SID previously assigned. If the BS assigns a new SID, it shall immediately age out the old SID and associated CPE.

First, a CPE shall synchronize to the frame preamble in order to perform initial ranging to BS/MR-BS. At this point, the CPE shall scan the US-MAP message to find an Initial Ranging Interval. The BS/MR-BS may allocate an Initial Ranging Interval consisting of one or more transmission opportunities. The CPE shall extract the number of initial ranging codes (see Table 31, element ID 150) from the UCD MAC management message.

The CPE randomly selects the CDMA code as described in 7.15.2.1 and sends the initial ranging CDMA code on the US allocation dedicated for that purpose. The BS/MR-BS receives the CDMA code. As many CPEs may contend for ranging, the CDMA code received may be the sum of many CPE transmissions. The BS/MR-BS isolates each of these transmissions and computes the ranging adjustments based on the relative time of arrival of each CPE upstream burst, i.e., the timing offset, so that all these bursts arrive at the BS/MR-BS at the beginning of the symbol period within sufficient tolerance.

Ranging adjusts each CPE’s timing offset such that each CPE appears to be co-located with the BS/MR-BS. The CPE shall set its initial timing offset to “zero advance” as if it was physically co-located with the MR-BS. When the Initial Ranging transmission opportunity occurs, the CPE shall send a CDMA code. After reception and decoding of this CDMA code, the BS/MR-BS will react by sending a RNG-CMD MAC message in a following frame with the same CDMA code and indicate the timing advance that the CPE should use for its upstream transmissions (see Table 44) so that the beginning of its bursts is aligned with the center of the cyclic prefix within the tolerance indicated in 9.9.1.

When the Initial Ranging transmission opportunity occurs, the CPE shall send a CDMA code. Thus, the CPE sends the message as if it were co-located with the MR-BS.

The CPE shall calculate the transmit EIRP per subcarrier for initial ranging, EIRPIR_CPE, from the following equation:

EIRPIR_CPE = EIRPMR-BS + RSSIR_MR-BS_nom – (RSSIR_CPE – GRX_CPE) + 10×log(NIR_sub/1680)

where

RSSIR_MR-BS_nom and EIRPMR-BS are defined in a DCD IE (see Table 23)

GRX_CPE is the antenna gain at the CPE

RSSIR_CPE is the RSSL measured by the CPE, which is then corrected by the CPE antenna gain to represent the RSSL for an isotropic antenna

NIR_sub is the number of subcarriers used by the CPE for initial ranging

NOTE—The value of RSSIR_MR-BS_nom corresponds to the nominal signal strength that should be measured at the output of a 0 dBi gain receive antenna at the BS for normal operation. The EIRPMR-BS is the equivalent isotropic radiated power of the base station in the direction of the CPE, which would be computed for a simple single-antenna transmitter as PTX_MR-BS

.

+ GTX_MR-BS, where PTX_MR-BS is the transmit power and GTX_MR-BS is the MR-BS transmit antenna gain in the direction of the CPE.

The CPE shall send a CDMA code with a power level resulting in the EIRPIR_CPE per subcarrier. If the CPE does not receive a response after waiting at least one frame to allow processing at the BS/MR-BS, the CPE shall send a new CDMA code at the next appropriate Initial Ranging transmission opportunity with 1 dB higher power level. The CPE shall, however, stop increasing the power level at the following condition:

EIRPIR_MAX +10×log(NIR_sub) > EIRPCPE_MAX

where

EIRPCPE_MAX        is the upper bound in maximum transmitted EIRP for the CPE on the current operating channel as described in Table 108 of 7.7.11.3.2.1 or 4 Watt for the fixed CPE whichever is the smallest

EIRPIR_CPE_MAX   is the upper bound for the increased EIRPIR_CPE

If the CPE receives a RNG-CMD message containing the parameters of the code it has transmitted and the status “continue,” it shall consider the transmission attempt unsuccessful but implement the corrections specified in the RNG-CMD and issue another CDMA code after the appropriate backoff delay. If the CPE receives an US-MAP containing a CDMA allocation IE with the parameters of the code it has transmitted, it shall consider the RNG-CMD reception successful, and proceed to send a unicast RNG-REQ (on Initial Ranging FID, allocated to Cell SID) on the allocated BW.

Once the BS/MR-BS has successfully received the RNG-REQ message, it shall return a RNG-CMD message using the initial ranging connection (see 12.2). Within the RNG-CMD message shall be the Station ID (SID) assigned to this CPE. The message shall also contain information on the required CPE EIRP level, offset frequency adjustment as well as the proper timing advance when needed. At this point the BS/MR-BS shall start using invited Initial Ranging Intervals addressed to the CPE’s Basic FID to complete the ranging process, unless the status of the RNG-CMD message is “success,” in which case the initial ranging procedure shall end.

If the status of the RNG-CMD message is “continue,” the CPE shall wait for an individual Initial Ranging Interval assigned to its Basic FID. Using this interval, the CPE shall transmit another RNG-REQ message using the Basic FID along with any power level and timing offset corrections.

The BS/MR-BS shall return another RNG-CMD message to the CPE with any additional fine-tuning required. The ranging request/response steps shall be repeated until the response contains a “Ranging Successful” notification or the BS/MR-BS aborts ranging. Once successfully ranged (timing, frequency and EIRP are within tolerance at the MR-BS), the CPE shall join normal data traffic in the upstream. In particular, the retry counts and timer values for the ranging process are defined in Table 273.

NOTE 1—The burst profile to use for any upstream transmission is defined by the Upstream Interval Usage Code (UIUC). Each UIUC is mapped to a burst profile in the UCD message.

NOTE 2—The BS/MR-BS shall allow the CPE sufficient time to have processed the previous RNG-CMD (i.e., to modify the transmitter parameters) before sending the CPE a specific ranging opportunity. This is defined as CPE Ranging Response Processing Time in Table 273.

On receiving a RNG-CMD instruction to move to a new channel during initial ranging, the CPE shall obtain a new SID via initial ranging and registration.

It is possible that the RNG-CMD may be lost after transmission by the BS/MR-BS. The CPE shall recover by timing out and reissuing its Initial RNG-REQ. Since the CPE is uniquely identified by the source MAC address in the Ranging Request, the MR-BS may immediately reuse the SID previously assigned. If the BS/MR-BS assigns a new SID, it shall immediately age out the old SID and associated CPE.

7.14.2.8.1a CDMA initial ranging and automatic adjustments (to distributed scheduling R-CPE)

A CPE shall synchronize to the local frame preamble within a distributed relay zone (DRZ) in order to perform initial ranging to the distributed scheduling R-CPE. At this point, the CPE shall scan the US-MAP message to find a DRZ Initial Ranging Interval within a local cell managed by a distributed scheduling R-CPE. The distributed scheduling R-CPE may allocate a DRZ Initial Ranging Interval consisting of one or more transmission opportunities in a DRZ for upstream. The CPE shall extract the number of DRZ initial ranging codes (see Table 31, element ID157) from the UCD MAC management message.

The CPE randomly selects the CDMA code as described in 7.15.2.1 and sends the DRZ initial ranging CDMA code on the DRZ of US allocation dedicated for that purpose. The distributed scheduling R-CPE receives the CDMA code. As many CPEs may contend for ranging, the CDMA code received may be the sum of many CPE transmissions. The distributed scheduling R-CPE isolates each of these transmissions and computes the ranging adjustments based on the relative time of arrival of each CPE upstream burst, i.e., the timing offset, so that all these bursts arrive at the distributed scheduling R-CPE at the beginning of the symbol period within sufficient tolerance.

Ranging adjusts each CPE’s timing offset such that each CPE appears to be co-located with the distributed scheduling R-CPE. The CPE shall set its initial timing offset to “zero advance” as if it was physically co-located with the dirstibuted scheduling R-CPE. When the DRZ Initial Ranging transmission opportunity occurs, the CPE shall send a CDMA code. After reception and decoding of this CDMA code, the distributed scheduling R-CPE will react by sending a RNG-CMD MAC message in a following frame with the same CDMA code and indicate the timing advance that the CPE should use for its upstream transmissions (see Table 44) so that the beginning of its bursts is aligned with the center of the cyclic prefix within the tolerance indicated in 9.9.1.

When the DRZ Initial Ranging transmission opportunity occurs, the CPE shall send a DRZ initial ranging CDMA code. Thus, the CPE sends the message as if it were co-located with the distributed scheduling R-CPE.

The CPE shall calculate the transmit EIRP per subcarrier for initial ranging, EIRPIR_CPE, from the following equation:

EIRPIR_CPE = EIRPR-CPE + RSSIR_R-CPE_nom – (RSSIR_CPE – GRX_CPE) + 10×log(NIR_sub/1680)

where

RSSIR_R-CPE_nom and EIRPR-CPE are defined in a DCD IE (see Table 23x)

GRX_CPE is the antenna gain at the CPE

RSSIR_CPE is the RSSL measured by the CPE, which is then corrected by the CPE antenna gain to represent the RSSL for an isotropic antenna

NIR_sub is the number of subcarriers used by the CPE for initial ranging

The CPE shall send a CDMA code with a power level resulting in the EIRPIR_CPE per subcarrier. If the CPE does not receive a response after waiting at least one frame to allow processing at the distributed scheduling R-CPE, the CPE shall send a new CDMA code at the next appropriate Initial Ranging transmission opportunity with 1 dB higher power level. The CPE shall, however, stop increasing the power level at the following condition:

EIRPIR_MAX +10×log(NIR_sub) > EIRPCPE_MAX

where

EIRPCPE_MAX        is the upper bound in maximum transmitted EIRP for the CPE on the current operating channel as described in Table 108 of 7.7.11.3.2.1 or 4 Watt for the fixed CPE whichever is the smallest

EIRPIR_CPE_MAX   is the upper bound for the increased EIRPIR_CPE

If the CPE receives a RNG-CMD message containing the parameters of the code it has transmitted and the status “continue,” it shall consider the transmission attempt unsuccessful but implement the corrections specified in the RNG-CMD and issue another CDMA code after the appropriate backoff delay. If the CPE receives an US-MAP containing a CDMA allocation IE with the parameters of the code it has transmitted from the distributed scheduling R-CPE, it shall consider the RNG-CMD reception successful, and proceed to send a unicast RNG-REQ (on Initial Ranging FID, allocated to Cell SID) on the allocated BW.

Once the distributed scheduling R-CPE has successfully received the RNG-REQ message, it shall return a RNG-CMD message using the initial ranging connection (see 12.2). Within the RNG-CMD message it shall be the Station ID, which is selected into one ofthe Local SID Group (7.7.7.3.6), assigned to this CPE. Note that a distributed scheduling R-CPE shall obtain the Local SID Group (7.7.7.3.6) used in a local cell from the MR-BS at registration. The RNG-CMD message shall also contain information on the required CPE EIRP level, offset frequency adjustment as well as the proper timing advance when needed. At this point the distributed scheduling R-CPE shall start using DRZ invited Initial Ranging Intervals addressed to the CPE’s Basic FID to complete the ranging process, unless the status of the RNG-CMD message is “success,” in which case the initial ranging procedure shall end.

If the status of the RNG-CMD message is “continue,” the CPE shall wait for a DRZ individual Initial Ranging Interval assigned to its Basic FID. Using this interval, the CPE shall transmit another RNG-REQ message using the Basic FID along with any power level and timing offset corrections.

The distributed scheduling R-CPE shall return another RNG-CMD message to the CPE with any additional fine-tuning required. The ranging request/response steps shall be repeated until the response contains a “Ranging Successful” notification or the R-CPE aborts ranging. Once successfully ranged (timing, frequency and EIRP are within tolerance at the R-CPE), the CPE shall join normal data traffic in the upstream. In particular, the retry counts and timer values for the ranging process are defined in Table 273.

On receiving a RNG-CMD instruction to move to a new channel during initial ranging, the CPE shall obtain a new SID via initial ranging and registration.

It is possible that the RNG-CMD may be lost after transmission by the distributed scheduling R-CPE. The CPE shall recover by timing out and reissuing its Initial RNG-REQ. Since the CPE is uniquely identified by the source MAC address in the Ranging Request, the distributed scheduling R-CPE may immediately reuse the SID previously assigned. If the distributed scheduling R-CPE assigns a new SID, it shall immediately age out the old SID and associated CPE.

7.14.2.8.1a CDMA initial ranging and automatic adjustments (by relaying on centralized scheduling R-CPE)

Although a CPE successfully obtains downstream parameters from a BS/MR-BS, CDMA initial ranging to the BS/MR-BS as described in 7.14.2.8.1 may be failed due to the CPE transmitting power constraint. However, a CPE is still able to have an uplink to MR-BS by relaying on  a centralized scheduling R-CPE.

A CPE shall synchronize to the frame preamble in order to perform initial ranging to a BS/MR-BS. At this point, the CPE shall scan the US-MAP message to find an Initial Ranging Interval and CRZ Initial Ranging Interval if available. The BS/MR-BS may allocate a CRZ Initial Ranging Interval consisting of one or more transmission opportunities within a CRZ of US subframe. The CPE shall extract the number of initial ranging codes and may extract the number of CRZ initial ranging codes (see Table 31, element ID 155) from the UCD MAC management message.

The CPE randomly selects the CDMA code as described in 7.15.2.1 and sends the initial ranging CDMA code to the BS/MR-BS on the Initial Ranaing Interval, and sends the CRZ inital ranging CDMA code to the centralized scheduling R-CPE on the CRZ Initial Ranging Interval as well in US allocation dedicated for that purpose. The initial ranging between the CPE and the MR-BS shall be following as described in 7.14.2.8.1. The following section describes the case that the CRZ initial ranging between the CPE and the centralized scheduling R-CPE.

The centralized scheduling R-CPE may receive the CRZ Initial Ranging CDMA code within the CRZ Initial Ranging Interval in a CRZ of US subframe. As many CPEs may contend for ranging, the CDMA code received may be the sum of many CPE transmissions. The centralized scheduling R-CPE isolates each of these transmissions and computes the ranging adjustments based on the relative time of arrival of each CPE upstream burst, i.e., the timing offset, so that all these bursts arrive at the centralized scheduling R-CPE at the beginning of the symbol period within sufficient tolerance.

Ranging adjusts each CPE’s timing offset such that each CPE appears to be co-located with the centralized scheduling R-CPE. The CPE shall set its initial timing offset to “zero advance” as if it was physically co-located with the centralized scheduling R-CPE. When the CRZ Initial Ranging transmission opportunity occurs, the CPE may send a CRZ CDMA code. After reception and decoding of this CDMA code, the centralized scheduling R-CPE will react by sending a RNG-CMD MAC message in a following frame with the same CDMA code and indicate the timing advance that the CPE should use for its upstream transmissions (see Table44) so that the beginning of its bursts is aligned with the center of the cyclic prefix within the tolerance indicated in 9.9.1. For the transmission of RNG-CMD to the CPE, a centralized scheduling R-CPE shall request bandwidth to a BS/MR-BS by using an Extended Bandwidth Request Subheader (7.6.1.2.1a).

When the CPE receives the RNG-CMD MAC message, CRZ intial ranging will start for the centralized scheduling CPE. The CPE randomly selects the CDMA code as described in 7.15.2.1 and sends the CRZ inital ranging CDMA code to the centralized scheduling R-CPE on the CRZ Initial Ranging Interval. Thus, the CPE sends the message as if it were co-located with the centralized scheduling R-CPE.

The CPE shall calculate the transmit EIRP per subcarrier for initial ranging, EIRPIR_CPE, from the following equation:

EIRPIR_CPE = EIRPR-CPE + RSSIR_R-CPE_nom – (RSSIR_CPE – GRX_CPE) + 10×log(NIR_sub/1680)

where

RSSIR_R-CPE_nom and EIRPR-CPE are defined in a DCD IE (see Table 23x)

GRX_CPE is the antenna gain at the CPE

RSSIR_CPE is the RSSL measured by the CPE, which is then corrected by the CPE antenna gain to represent the RSSL for an isotropic antenna

NIR_sub is the number of subcarriers used by the CPE for initial ranging

The CPE shall send a CDMA code with a power level resulting in the EIRPIR_CPE per subcarrier. If the CPE does not receive a response after waiting at least one frame to allow processing at the centralized scheduling R-CPE, the CPE shall send a new CDMA code at the next appropriate CRZ Initial Ranging transmission opportunity with 1 dB higher power level. The CPE shall, however, stop increasing the power level at the following condition:

EIRPIR_MAX +10×log(NIR_sub) > EIRPCPE_MAX

where

EIRPCPE_MAX        is the upper bound in maximum transmitted EIRP for the CPE on the current operating channel as described in Table 108 of 7.7.11.3.2.1 or 4 Watt for the fixed CPE whichever is the smallest

EIRPIR_CPE_MAX   is the upper bound for the increased EIRPIR_CPE

If the CPE receives a RNG-CMD message containing the parameters of the code it has transmitted and the status “continue,” it shall consider the transmission attempt unsuccessful but implement the corrections specified in the RNG-CMD and issue another CDMA code after the appropriate backoff delay. If the CPE receives a RNG-CMD message containing the parameters of the code it has transmitted and the status “success”, it shall proceed to send a unicast RNG-REQ (on Initial Ranging FID, allocated to Cell SID) to the centralized scheduling R-CPE on the allocated BW, which shall be required by the centralized scheduling R-CPE to the MR-BS by using the Extended Bandwidth Request Subheader (7.6.1.2.1a). If the centralized scheduling R-CPE receives RNG-REQ from the CPE, the centralized scheduling R-CPE relays the RNG-REQ messages, which will be conveyed on a Container message (7.7.26), to the MR-BS. The container message may contain several management messages, which are scheduled to transmit from a centralized scheduling R-CPE to the BS/MR-BS.

Once the BS/MR-BS has successfully received the RNG-REQ message by encoding the received Container message it shall return a Container ACK message with a confirmation code for the received messages to the centralized scheduling R-CPE. If the confirmation code for a certain management message is not “success”, the centralized scheduling R-CPE shall retransmit the indicated management message to the BS/MR-BS. After correctly receiving RNG-REQ, the BS/MR-BS shall return a RNG-CMD message to the CPE. Within the RNG-CMD message shall be the Station ID (SID) assigned to this CPE.

Moreover, a CPE can successfully perform CDMA initial ranging to the several devices including a BS/MR-BS and centralized scheduling R-CPEs. In this case, the CPE shall select one of those.

7.14.2.8.2 Ranging parameter adjustment

Adjustment of local parameters (e.g., transmit EIRP) in a CPE as a result of the receipt or non-receipt of a RNG-CMD message is considered to be implementation-dependent with the following restrictions:

a)    All parameters shall be within the approved range at all times.

b)   EIRP adjustment shall start from the initial value selected with the algorithm described in 7.14.2.8.1 unless a valid EIRP setting is available from non-volatile storage, in which case this value may be used a the starting point.

c)	EIRP adjustment shall be capable of being reduced or increased by the specified amount in response to the RNG-CMD messages.

d)   If, during initialization, EIRP is increased to the maximum value as determined in 7.14.2.8.1 without a response from the BS, it shall go back to the minimum EIRP and ramp up to its maximum EIRP four (4) times before aborting the ranging process with this base station.

On receiving a RNG-CMD message, the CPE shall not transmit until the RF signal has been adjusted in accordance with the RNG-CMD and has stabilized.

7.14.2.9 CPE transmit basic capabilities

Immediately following the completion of initial ranging, the CPE informs the BS of its basic capabilities by transmitting a CBC-REQ message (see Table 105) with its capabilities set to “on” (see Figure 39). Note that T18 is a timer used to wait for CBC-RSP timeout and the default value is indicated in Table 272.

The BS responds with a CBC-RSP message (see Table 106) with the intersection of the CPE’s and BS’s capabilities set to “on” (see Figure 40 and Figure 41, respectively). The timer T9 refers to the time allowed between the BS sending a RNG-CMD to a CPE, and receiving a CBC-REQ from that same CPE, and the minimum value  is  specified  in  Table  272.  Note  that  the  CPE  capability  information  is  presented  in 7.7.7.3.4. When T9 expires, the SID assigned during ranging shall be aged out and the CPE shall have to attempt ranging process over again while not exceeding the maximum number of CDMA ranging retries indicated in Table 273.

Immediately following the completion of initial ranging, the CPE informs the BS/MR-BS of its basic capabilities by transmitting a CBC-REQ message (see Table 105) with its capabilities set to “on” (see Figure 39). Note that T18 is a timer used to wait for CBC-RSP timeout and the default value is indicated in Table 272.

The BS/MR-BS responds with a CBC-RSP message (see Table 106) with the intersection of the CPE’s and BS’s/MR-BS’s capabilities set to “on” (see Figure 40 and Figure 41, respectively). The timer T9 refers to the time allowed between the BS/MR-BS sending a RNG-CMD to a CPE, and receiving a CBC-REQ from that same CPE, and the minimum value is specified in Table 272. Note that the CPE capability information is presented in 7.7.7.3.4. When T9 expires, the SID assigned during ranging shall be aged out and the CPE shall have to attempt ranging process over again while not exceeding the maximum number of CDMA ranging retries indicated in Table 273.

7.14.2.9.1 CPE transmit basic capabilities (to distributed scheduling R-CPE)

Immediately following the completion of DRZ initial ranging, the CPE informs the distraibuted scheduling R-CPE of its basic capabilities by transmitting a CBC-REQ message (see Table 105) with its capabilities set to “on” (see Figure 39). Note that T18 is a timer used to wait for CBC-RSP timeout and the default value is indicated in Table 272.

The distributed scheduling R-CPE responds with a CBC-RSP message (see Table 106) with the intersection of the CPE’s and distributed scheduling R-CPE’s capabilities set to “on” (see Figure 40 and Figure 41a, respectively). The timer T9 refers to the time allowed between the distributed scheduling R-CPE sending a RNG-CMD to a CPE, and receiving a CBC-REQ from that same CPE, and the minimum value is specified in Table 272. Note that the CPE capability information is presented in 7.7.7.3.4. When T9 expires, the SID assigned during ranging shall be aged out and the CPE shall have to attempt ranging process over again while not exceeding the maximum number of CDMA ranging retries indicated in Table 273.

Figure 41a – Negotiate basic capabilities at distributed scheduling R-CPE

7.14.2.9.2 CPE transmit basic capabilities (relaying on centralized scheduling R-CPE)

Immediately following the completion of CRZ initial ranging, the CPE informs the centralized scheduling R-CPE of its basic capabilities by transmitting a CBC-REQ message (see Table 105) with its capabilities set to “on” (see Figure 39). Note that T18 is a timer used to wait for CBC-RSP timeout and the default value is indicated in Table 272. When the centralized scheduling R-CPE receives the CBC-REQ messages from the CPEs, the centralized scheduling R-CPE transmits a Container message containing the received CBC-REQ messages to the MR-BS (see Figure 39a). Note that the Container messages may contain not only CBC-REQ messages but also other management messages. Note that Txx is a timer used to wait for Container ACK timeout, which is indicated in Table 272.

When the BS/MR-BS has successfully received the CBC-REQ message by encoding the received Container message, it shall return a Container ACK message with a confirmation code for the CBC-REQ to the centralized scheduling R-CPE. If the confirmation code for a certain management message is not “success”, the centralized scheduling R-CPE shall retransmit the indicated management message to the BS/MR-BS (see Figure 39b). After correctly receiving CBC-REQ, the MR-BS responds with a CBC-RSP message (see Table 106) to the CPE, with the intersection of the CPE’s and MR-BS’s capabilities set to “on”.The timer T9 refers to the time allowed between the MR-BS sending a RNG-CMD to a CPE, and receiving a CBC-REQ from that same CPE, and the minimum value is specified in Table 272. Note that the CPE capability information is presented in 7.7.7.3.4. When T9 expires, the SID assigned during ranging shall be aged out and the CPE shall have to attempt ranging process over again while not exceeding the maximum number of CDMA ranging retries indicated in Table 273.

Figure 39a – Wait for CBC-REQ and Sending Container Message including CBC-REQ at a centralized scheduling CPE

Figure 39b – Wait for Container ACK at a centralized scheduling CPE

7.14.2.10 CPE authentication and key exchange

Once configuration of the required basic capabilities is completed, the CPE and AAA continue with performing authentication and exchanging keys, as described in Clause 8. If the AAA and CPE can not authenticate each other, authentication of the CPE fails for the selected WRAN service. This WRAN service on the selected channel is removed from further consideration. If there are any other BSs available with which to associate, the updated list of available WRAN services is presented to the higher user layers.

Once configuration of the required basic capabilities is completed, the CPE and AAA continue with performing authentication and exchanging keys, as described in Clause 8. If the AAA and CPE can not authenticate each other, authentication of the CPE fails for the selected WRAN service. This WRAN service on the selected channel is removed from further consideration. If there are any other BSs available with which to associate, the updated list of available WRAN services is presented to the higher user layers.

7.14.2.11 Registration

Registration is the process by which the CPE verifies its configuration with the BS. If the CPE supports a configuration that is set by the BS, it is allowed entry into the network and thus becomes manageable. To register with a BS, the CPE shall send a REG-REQ message to the BS. The REG-REQ message shall include a CPE NMEA Location string IE.

During registration, the CPE’s NMEA Location String and various operational parameters are configured (see 7.7.7.3). The CPE sends its location data string (see 7.6.1.3.1.6) upon initial registration and re- registration. When the IP Address Allocation Information Element (see 7.7.7.3.4.11) is present in the REG- REQ message, the BS shall include this IP address allocation parameter in the REG-RSP message to command the CPE to use the indicated version of IP on the secondary management connection. The BS shall command the use of exactly one of the IP versions supported by the CPE.

The BS shall determine the location of the antenna of each associated CPE with the accuracy as specified in Table A.9 for the specific regulatory domain. The BS’s SM shall receive the generated NMEA string and validate its contents.

The BS’s SM shall provide the geolocation data to the database service. The BS shall refuse to serve theCPE if

The geographic location of the CPE has not been successfully determined as indicated by a failed validation of the data in the NMEA string. Validation shall fail if

a) The NMEA string contains data that is outside the allowable range of values or;

b) The distance between the initializing CPE and the BS or other associated CPEs is outside the allowable range of values.

The database service has indicated that the CPE cannot operate on the channel on which the WRAN network intends to operate.

In the first case, validation of the NMEA string fails and CPE initialization fails, in the second case, the CPE initialization fails on the current channel and shall proceed to the next channel on its available WRAN services list.

The BS shall respond with a REG-RSP message. The REG-RSP message shall include the Permanent Station ID (see Table 61), if CPE Privacy (see 8.7) is enabled. Figure 42 shows the procedure that shall be followed by the CPE to initiate registration.

Once the CPE has sent a REG-REQ to the BS, it shall wait for a REG-RSP to authorize it to forward traffic to the network. Figure 43 shows the waiting procedure that shall be followed by the CPE.

The BS shall perform the operations shown in Figure 44. Note that the Timer T13 represents the time allowed for a CPE, following receipt of a REG-RSP message, to send a TFTP-CPLT message to the BS, and its minimum time is specified in Table 272. In addition, the Timer T28 is the time allowed for the BS to complete the transmission of channel sets; its default value is specified in Table 272.

IEEE 802.22 CPEs are managed devices. Network entry is not considered complete until after the TFTP- CPLT/RSP (see 7.7.19). When the BS and CPE complete the TFTP-CPLT/RSP exchange, timer T30 is scheduled for the value set in CPE Registration Timer ( 7.7.7.3.5) IE. When T30 expires the BS and CPE shall  delete  all  information  pertaining  to  their  associations  (e.g.,  SIDs,  registered  capabilities,  active service-flow parameters, remaining security context), regardless of whether or not the CPE is currently being served by the BS.

Prior to expiration of T30, the BS may attempt to verify connectivity to a CPE via periodic ranging. This can be facilitated by the BS sending an unsolicited RNG-CMD message with Ranging Status field set to “Re-range & Re-register” (see Table 44). Upon receiving said RNG-CMD, the CPE shall attempt to re- range with the BS, as well as send a REG-REQ with the current configuration of the CPE NMEA Location String IE (7.6.1.3.1.6) and Manufacturer-specific Antenna Model IE (7.7.7.3.4.8) to inform the BS of its current position and antenna information. Upon sending this REG-REQ to the BS, the CPE should use the signaling in 9.12.2 to re-populate the MIBs used to configure these IEs (see wranIfCpeAntennaGainTable and wranIfAntennaModel in 13.1) and update the configuration of these IEs by reading the information. If the CPE finds out that this information has changed, it shall re-initialize itself. If the BS does not receive either the RNG-REQ or the REG-REQ (with the location information) from the CPE in the allocated opportunity, the BS shall wait until T30 expires before de-registering the CPE.

If the CPE is currently being served by the BS, the BS can force the CPE to delete the pertinent information before expiration of T30 by the following:

a)	Send a DREG-CMD to CPE with Action Code = 0x04 (see Table 115) to shutdown the CPE. This is done if the BS detects that the CPE has moved outside the current coverage area of the BS and is not able to service it.

b)   Send a DREG-CMD to CPE with Action Code = 0x05 (see Table 115) to force CPE to reinitialize on the current operating channel. This is done if the CPE’s movement is beyond the movement threshold of ±25 m (see policy 8 in Table 234), but the CPE’s movement does not result in a new backup/candidate channel list upon query of the database service.

c)	Send a DREG-CMD to the CPE with Action Code = 0x01 and subsequently another DREG-CMD with Action Code = 0x03 (see Table 115) to temporarily disable the CPE’s transmission. This is done, to temporarily disable the CPE’s transmission when a CPE’s movement is within the movement threshold of ±25 m (see Policy 8 in Table 234), but the CPE’s movement does not results in a new backup/candidate channel list upon query of the database service. This avoids having to reinitialize the CPE.

For case a), T30 shall be cleared when the CPE is shutdown. For case b), the T30 shall be reset upon completion of re-registration. For case c), the T30 shall be reset upon sending the DREG-CMD to re-enable CPE.

If the SM (upon interrogating the SSA) detected that the CPE has moved, the BS shall request de- registration by sending a DREG-CMD message to the CPE set with the appropriate Action Code as mentioned above.

Registration is the process by which the CPE verifies its configuration with the BS/MR-BS. If the CPE supports a configuration that is set by the BS/MR-BS, it is allowed entry into the network and thus becomes manageable. To register with a BS/MR-BS, the CPE shall send a REG-REQ message to the BS/MR-BS. The REG-REQ message shall include a CPE NMEA Location string IE except for Mode I CPE.

During registration, the CPE’s NMEA Location String and various operational parameters are configured (see 7.7.7.3). The CPE sends its location data string (see 7.6.1.3.1.6) upon initial registration and re-registration. When the IP Address Allocation Information Element (see 7.7.7.3.4.11) is present in the REG-REQ message, the BS/MR-BS shall include this IP address allocation parameter in the REG-RSP message to command the CPE to use the indicated version of IP on the secondary management connection. The BS/MR-BS shall command the use of exactly one of the IP versions supported by the CPE.

The BS/MR-BS shall determine the location of the antenna of each associated CPE with the accuracy as specified in Table A.9 for the specific regulatory domain. The BS’s/MR-BS’s SM shall receive the generated NMEA string and validate its contents.

The BS’s/MR-BS’s SM shall provide the geolocation data to the database service. The BS/MR-BS shall refuse to serve the CPE if

The geographic location of the CPE except for Mode I CPE has not been successfully determined as indicated by a failed validation of the data in the NMEA string. Validation shall fail if

a) The NMEA string contains data that is outside the allowable range of values or;

b) The distance between the initializing CPE and the BS or other associated CPEs is outside the allowable range of values.

The database service has indicated that the CPE except for Mode I CPE cannot operate on the channel on which the WRAN network intends to operate.

In the first case, validation of the NMEA string fails and CPE initialization fails, in the second case, the CPE initialization fails on the current channel and shall proceed to the next channel on its available WRAN services list.

The BS/MR-BS shall respond with a REG-RSP message. The REG-RSP message shall include the Permanent Station ID (see Table 61), if CPE Privacy (see 8.7) is enabled. Figure 42 shows the procedure that shall be followed by the CPE to initiate registration.

Once the CPE has sent a REG-REQ to the BS/MR-BS, it shall wait for a REG-RSP to authorize it to forward traffic to the network. Figure 43 shows the waiting procedure that shall be followed by the CPE.

From encoding CPE operation capabilities in the REG-RSP (7.7.7.3.4.13), the CPE will operate as one of operating modes: a fixed subscriber CPE, a portable subscriber CPE (Mode I), a centralized scheduling R-CPE, or a distributed scheduling R-CPE (fixed or portable Mode II).

The BS/MR-BS shall perform the operations shown in Figure 44. Note that the Timer T13 represents the time allowed for a CPE, following receipt of a REG-RSP message, to send a TFTP-CPLT message to the BS/MR-BS, and its minimum time is specified in Table 272. In addition, the Timer T28 is the time allowed for the BS to complete the transmission of channel sets; its default value is specified in Table 272.

IEEE 802.22 CPEs and IEEE 802.22b CPEs are managed devices. Network entry is not considered complete until after the TFTP- CPLT/RSP (see 7.7.19). When the BS/MR-BS and CPE complete the TFTP-CPLT/RSP exchange, timer T30 is scheduled for the value set in CPE Registration Timer (7.7.7.3.5) IE. When T30 expires the BS/MR-BS and CPE shall delete all information pertaining to their associations (e.g., SIDs, registered capabilities, active service-flow parameters, remaining security context), regardless of whether or not the CPE is currently being served by the BS/MR-BS.

Prior to expiration of T30, the BS/MR-BS may attempt to verify connectivity to a CPE via periodic ranging. This can be facilitated by the BS/MR-BS sending an unsolicited RNG-CMD message with Ranging Status field set to “Re-range & Re-register” (see Table 44). Upon receiving said RNG-CMD, the CPE shall attempt to re-range with the BS/MR-BS, as well as send a REG-REQ with the current configuration of the CPE NMEA Location String IE (7.6.1.3.1.6) and Manufacturer-specific Antenna Model IE (7.7.7.3.4.8) to inform the BS/MR-BS of its current position and antenna information. Upon sending this REG-REQ to the BS/MR-BS, the CPE should use the signaling in 9.12.2 to re-populate the MIBs used to configure these IEs (see wranIfCpeAntennaGainTable and wranIfAntennaModel in 13.1) and update the configuration of these IEs by reading the information. If the CPE finds out that this information has changed, it shall re-initialize itself. If the BS/MR-BS does not receive either the RNG-REQ or the REG-REQ (with the location information) from the CPE in the allocated opportunity, the BS/MR-BS shall wait until T30 expires before de-registering the CPE.

If the CPE is currently being served by the BS/MR-BS, the BS/MR-BS can force the CPE to delete the pertinent information before expiration of T30 by the following:

a)	Send a DREG-CMD to CPE with Action Code = 0x04 (see Table 115) to shutdown the CPE. This is done if the BS/MR-BS detects that the CPE has moved outside the current coverage area of the BS/MR-BS and is not able to service it.

b)   Send a DREG-CMD to CPE with Action Code = 0x05 (see Table 115) to force CPE to reinitialize on the current operating channel. This is done if the CPE’s movement is beyond the movement threshold of ±25 m (see policy 8 in Table 234), but the CPE’s movement does not result in a new backup/candidate channel list upon query of the database service.

c)	Send a DREG-CMD to the CPE with Action Code = 0x01 and subsequently another DREG-CMD with Action Code = 0x03 (see Table 115) to temporarily disable the CPE’s transmission. This is done, to temporarily disable the CPE’s transmission when a CPE’s movement is within the movement threshold of ±25 m (see Policy 8 in Table 234), but the CPE’s movement does not results in a new backup/candidate channel list upon query of the database service. This avoids having to reinitialize the CPE.

For case a), T30 shall be cleared when the CPE is shutdown. For case b), the T30 shall be reset upon completion of re-registration. For case c), the T30 shall be reset upon sending the DREG-CMD to re-enable CPE.

If the SM (upon interrogating the SSA) detected that the CPE has moved, the BS/MR-BS shall request de-registration by sending a DREG-CMD message to the CPE set with the appropriate Action Code as mentioned above.

7.14.2.11.xx Registration (Distributed scheduling R-CPE)

To register with a distributed scheduling R-CPE, the CPE shall send a REG-REQ message to the distributed scheduling R-CPE. The REG-REQ message shall may include a CPE NMEA Location string IE.

During registration, the various operational parameters are configured (see 7.7.7.3). When the IP Address Allocation Information Element (see 7.7.7.3.4.11) is present in the REG-REQ message, the distributed scheduling R-CPE shall include this IP address allocation parameter in the REG-RSP message to command the CPE to use the indicated version of IP on the secondary management connection. The distributed scheduling R-CPE shall command the use of exactly one of the IP versions supported by the CPE.

The distributed scheduling R-CPE  may determine the location of the antenna of each associated CPE with the accuracy as specified in Table A.9 for the specific regulatory domain. The distributed scheduling R-CPE’s SM shall receive the validate its contents.

The distributed scheduling R-CPE’s SM shall may provide the geolocation data of the CPE tothe MR-BS. The MR-BS may refuse to serve the CPE if the geographic location of the CPE has not been successfully determined as indicated by a failed validation of the data in the NMEA string. Validation shall fail if

- The NMEA string contains data that is outside the allowable range of values or;

b) The distance between the initializing CPE and the associated distributed scheduling R-CPEs is outside the allowable range of values.

When a distributed scheduling R-CPE receives a REG-REQ message from the CPE, the distributed scheduling R-CPE sends a Container message containing the received REG-REQ message to the MR-BS. When the BS/MR-BS has successfully received the REG-REQ message by encoding the received Container message, it shall return a Container ACK message with a confirmation code for the REG-REQ to the distributed scheduling R-CPE. If the confirmation code for a certain management message is not “success”, the distributed scheduling R-CPE shall retransmit the indicated management message to the BS/MR-BS. After correctly receiving REG-REQ, the MR-BS responds with a REG-RSP message (see Table 106) to the CPE. The REG-RSP message shall include the Permanent Station ID (see Table 61), if CPE Privacy (see 8.7) is enabled. Figure 42 shows the procedure that shall be followed by the CPE to initiate registration.

Once the CPE has sent a REG-REQ to the distributed scheduling R-CPE, it shall wait for a REG-RSP to authorize it to forward traffic to the local network. Figure 43 shows the waiting procedure that shall be followed by the CPE.

The distributed scheduling R-CPE shall perform the operations shown in Figure 44. Note that the Timer T13 represents the time allowed for a CPE, following receipt of a REG-RSP message, to send a TFTP-CPLT message to the distributed scheduling R-CPE, and its minimum time is specified in Table 272. In addition, the Timer T28 is the time allowed for the MR-BS to complete the transmission of channel sets; its default value is specified in Table 272.

IEEE 802.22b CPEs are managed devices. Network entry is not considered complete until after the TFTP- CPLT/RSP (see 7.7.19). When the MR-BS and CPE through the distributed scheduling R-CPE complete the TFTP-CPLT/RSP exchange, timer T30 is scheduled for the value set in CPE Registration Timer (7.7.7.3.5) IE. When T30 expires the MR-BS, distributed scheduling R-CPE and CPE shall delete all information pertaining to their associations (e.g.,  SIDs, registered capabilities, active service-flow parameters, remaining security context), regardless of whether or not the CPE is currently being served by the distributed scheduling R-CPE.

Prior to expiration of T30, the distributed scheduling R-CPE may attempt to verify connectivity to a CPE via periodic ranging. This can be facilitated by the distributed scheduling R-CPE sending an unsolicited RNG-CMD message with Ranging Status field set to “Re-range & Re-register” (see Table 44). Upon receiving said RNG-CMD, the CPE shall attempt to re-range with the distributed scheduling R-CPE, as well as send a REG-REQ to the distributed scheduling R-CPE. The distributed scheduling R-CPE shall send a Container message containing the received REG-REQ message to the MR-BS. If the distributed scheduling R-CPE does not receive either the RNG-REQ or the REG-REQ (with the location information) from the CPE in the allocated opportunity, the distributed scheduling R-CPE shall wait until T30 expires before de-registering the CPE.

If the CPE is currently being served by the distributed scheduling R-CPE, the distributed scheduling R-CPE can force the CPE to delete the pertinent information before expiration of T30 by the following:

- Send a DREG-CMD to CPE with Action Code = 0x04 (see Table 115) to shutdown the CPE. This is done if the distributed scheduling R-CPE detects that the CPE has moved outside the current coverage area of the R-CPE and is not able to service it.

- Send a DREG-CMD to CPE with Action Code = 0x05 (see Table 115) to force CPE to reinitialize on the current operating channel. This is done if the CPE’s movement is beyond the movement threshold of ±25 m (see policy 8 in Table 234), but the CPE’s movement does not result in a new backup/candidate channel list upon query of the database service.

- Send a DREG-CMD to the CPE with Action Code = 0x01 and subsequently another DREG-CMD with Action Code = 0x03 (see Table 115) to temporarily disable the CPE’s transmission. This is done, to temporarily disable the CPE’s transmission when a CPE’s movement is within the movement threshold of ±25 m (see Policy 8 in Table 234), but the CPE’s movement does not results in a new backup/candidate channel list upon query of the database service. This avoids having to reinitialize the CPE.

For case a), T30 shall be cleared when the CPE is shutdown. For case b), the T30 shall be reset upon completion of re-registration. For case c), the T30 shall be reset upon sending the DREG-CMD to re-enable CPE.

If the SM (upon interrogating the SSA) detected that the CPE has moved, the distributed scheduling R-CPE shall request de-registration by sending a DREG-CMD message to the CPE set with the appropriate Action Code as mentioned above.

After the distributed scheduling R-CPE de-registers the CPE, the distributed scheduling R-CPE shall send Local Cell Update message (7.7.25) to the MR-BS.

Figure xx –CPE registration in a local cell

Figure xx – Wait for REG-REQ and Sending Extended REG-REQ at a distributed scheduling CPE

Figure xx – Wait for REG-RSP at a distributed scheduling CPE

Figure xx –Sending Local Cell Update REQ from a distributed scheduling CPE

Figure xx – Wait for Local Cell Update RSP at a distributed scheduling CPE

Figure xx –Local Cell Update at MR-BS

7.14.2.11.xx Registration (to MR-BS via Centralized Scheduling R-CPE)

To register with a MR-BS by relaying on a centralized scheduling R-CPE, the CPE shall send a REG-REQ message to the the centralized scheduling R-CPE. The REG-REQ message may include a CPE NMEA Location string IE. The centralized scheduling R-CPE shall send a Conainer message including the REQ-REQ message to the MR-BS. When the BS/MR-BS has successfully received the REG-REQ message by encoding the received Container message, it shall return a Container ACK message with a confirmation code for the REG-REQ to the centralized scheduling R-CPE. If the confirmation code for a certain management message is not “success”, the centralized scheduling R-CPE shall retransmit the indicated management message to the BS/MR-BS.

During registration, the CPE’s NMEA Location String and various operational parameters are configured (see 7.7.7.3). The CPE sends its location data string (see 7.6.1.3.1.6) upon initial registration and re-registration. When the IP Address Allocation Information Element (see 7.7.7.3.4.11) is present in the REG-REQ message, the MR-BS shall include this IP address allocation parameter in the REG-RSP message to command the CPE to use the indicated version of IP on the secondary management connection. The MR-BS shall command the use of exactly one of the IP versions supported by the CPE.

The MR-BS shall respond with a REG-RSP message. The REG-RSP message shall include the Permanent Station ID (see Table 61), if CPE Privacy (see 8.7) is enabled. Figure 42 shows the procedure that shall be followed by the CPE to initiate registration.

Once the CPE has sent a REG-REQ to the MR-BS, it shall wait for a REG-RSP to authorize it to forward traffic to the network. Figure 43 shows the waiting procedure that shall be followed by the CPE.

The MR-BS shall perform the operations shown in Figure 44. Note that the Timer T13 represents the time allowed for a CPE, following receipt of a REG-RSP message, to send a TFTP-CPLT message to the MR-BS, and its minimum time is specified in Table 272. In addition, the Timer T28 is the time allowed for the MR-BS to complete the transmission of channel sets; its default value is specified in Table 272.

IEEE 802.22b CPEs are managed devices. Network entry is not considered complete until after the TFTP- CPLT/RSP (see 7.7.19). When the MR-BS and CPE complete the TFTP-CPLT/RSP exchange, timer T30 is scheduled for the value set in CPE Registration Timer (7.7.7.3.5) IE. When T30 expires the MR-BS and CPE shall delete all information pertaining to their associations (e.g., SIDs, registered capabilities, active service-flow parameters, remaining security context), regardless of whether or not the CPE is currently being served by the MR-BS.

Prior to expiration of T30, the MR-BS may attempt to verify connectivity to a CPE via periodic ranging. This can be facilitated by the MR-BS sending an unsolicited RNG-CMD message with Ranging Status field set to “Re-range & Re-register” (see Table 44). Upon receiving said RNG-CMD, the CPE shall attempt to re-range with the centralized scheduling R-CPE, as well as send a REG-REQ to the centralized scheduling R-CPE. The centralized scheduling R-CPE sends a Container message containing the received REG-REQ message to the MR-BS. Upon sending this REG-REQ to the MR-BS, the CPE should use the signaling in 9.12.2 to re-populate the MIBs used to configure these IEs and update the configuration of these IEs by reading the information. If the CPE finds out that this information has changed, it shall re-initialize itself. If the MR-BS does not receive either the RNG-REQ or the REG-REQ (with the location information) from the CPE in the allocated opportunity, the MR-BS shall wait until T30 expires before de-registering the CPE.

If the CPE is currently being served by the MR-BS, the MR-BS can force the CPE to delete the pertinent information before expiration of T30 by the following:

- Send a DREG-CMD to CPE with Action Code = 0x04 (see Table 115) to shutdown the CPE. This is done if the MR-BS detects that the CPE has moved outside the current coverage area of the MR-BS and is not able to service it.

- Send a DREG-CMD to CPE with Action Code = 0x05 (see Table 115) to force CPE to reinitialize on the current operating channel. This is done if the CPE’s movement is beyond the movement threshold of ±25 m (see policy 8 in Table 234), but the CPE’s movement does not result in a new backup/candidate channel list upon query of the database service.

- Send a DREG-CMD to the CPE with Action Code = 0x01 and subsequently another DREG-CMD with Action Code = 0x03 (see Table 115) to temporarily disable the CPE’s transmission. This is done, to temporarily disable the CPE’s transmission when a CPE’s movement is within the movement threshold of ±25 m (see Policy 8 in Table 234), but the CPE’s movement does not results in a new backup/candidate channel list upon query of the database service. This avoids having to reinitialize the CPE.

For case a), T30 shall be cleared when the CPE is shutdown. For case b), the T30 shall be reset upon completion of re-registration. For case c), the T30 shall be reset upon sending the DREG-CMD to re-enable CPE.

If the SM (upon interrogating the SSA) detected that the CPE has moved, the MR-BS shall request de- registration by sending a DREG-CMD message to the CPE set with the appropriate Action Code as mentioned above.

Figure 42a – Wait for REG-REQ and Sending Container message including REG-REQ at a centralized scheduling CPE

Figure 42b – Wait for Container ACK at a centralized scheduling CPE

7.14.2.12 MR-BS transmits channel sets to CPE (Centralized mode)

The BS shall send the channel sets to the new CPE. The channel sets are described in 10.2.3. The channel sets that are sent to the initializing CPE are the backup channels and the candidate channels. The channel sets are sent in a DCD message, as described in 7.7.1 and in Table 24 to Table 26. The BS shall send DCD channel information elements 11 and 12. Table 26 describes information element 12 as the backup and candidate channel list. It is a prioritized list of the channels with the backup channel set higher in priority than the candidate channel set. The two sets are identified by sending information element 11, which provides the number of the higher prioritized backup channel set. Each channel in DCD information element 12 is characterized by both the channel number.

The MR-BS shall send the channel sets to the new CPE. The channel sets are described in 10.2.3. The channel sets that are sent to the initializing CPE are the backup channels and the candidate channels. The channel sets are sent in a DCD message, as described in 7.7.1 and in Table 24 to Table 26. The MR-BS shall send DCD channel information elements 11 and 12. Table 26 describes information element 12 as the backup and candidate channel list. It is a prioritized list of the channels with the backup channel set higher in priority than the candidate channel set. The two sets are identified by sending information element 11, which provides the number of the higher prioritized backup channel set. Each channel in DCD information element 12 is characterized by both the channel number.

The distributed scheduling R-CPE shall send the channel sets to the new CPE, which is registered with the distributed scheduling R-CPE. The channel sets are described in 10.2.3. The channel sets that are sent to the initializing CPE are the backup channels and the candidate channels. The channel sets are sent in a DCD message in a DRZ, as described in 7.7.1 and in Table 24 to Table 26. The distributed scheduling R-CPE shall send DCD channel information elements 11 and 12. Table 26 describes information element 12 as the backup and candidate channel list. It is a prioritized list of the channels with the backup channel set higher in priority than the candidate channel set. The two sets are identified by sending information element 11, which provides the number of the higher prioritized backup channel set. Each channel in DCD information element 12 is characterized by both the channel number.

7.14.2.13 Establish IP connectivity

The CPE shall invoke DHCP mechanism (IETF RFC 2131 [B20]) in order to obtain an IP address and any other parameters needed to establish IP connectivity. If the CPE has a configuration file, the DHCP response shall contain the name of a file that contains further configuration parameters.

Establishment of IP connectivity shall be performed on the CPE’s secondary management connection as shown in Figure 45.

In case where dynamic IP configuration is not preferred, the CPE shall obtain an IP address from its base station.

Figure 45  — Establishing IP connectivity

7.14.2.14 Establish time of day

The CPE and BS need to have the current date and time. This is required for time-stamping logged events for retrieval by the management system. This needs not be authenticated and needs to be accurate only to the nearest second. The current date and time may be obtained from a local time source or a remote service such as an NTP server.

The CPE and MR-BS need to have the current date and time. This is required for time-stamping logged events for retrieval by the management system. This needs not be authenticated and needs to be accurate only to the nearest second. The current date and time may be obtained from a local time source or a remote service such as an NTP server.

Figure 46  — Establishing time of day

7.14.2.15 Transfer operational parameters

The CPE shall download the CPE’s configuration file using TFTP on its own secondary management connection as shown in Figure 47. The CPE shall use an adaptive timeout for TFTP based on binary exponential backoff (IETF RFC 1123 [B19], IETF RFC 2349 [B21]).

When the configuration file download has completed successfully, the CPE shall notify the BS by transmitting the TFTP-CPLT message on the CPE’s primary management connection. Transmissions shall continue successfully until a TFTP-RSP message is received with response “OK” from the BS (see Figure

48 and Figure 49) or the CPE terminates retransmission due to retry exhaustion.

Upon sending a REG-RSP, the BS shall wait for a TFTP-CPLT. If the timer T13 (defined in Table 272) expires, the BS shall restart the registration process (REG-REQ/RSP) with the CPE (see Figure 48). Note that the Timer T26 refers to the time waited for TFTP-RSP. If T26 expires, then TFTP-CPLT is attempted until the maximum number of retries is exhausted. Upon the exhaustion, the CPE shall be deregistered (i.e., forced to reinitialize MAC) by sending a DREG-REQ with Action Code set to 0x05 to force itself to reattempt system access or 0x04 to shut itself down (see Figure 49).

The CPE shall download the CPE’s configuration file using TFTP on its own secondary management connection as shown in Figure 47. The CPE shall use an adaptive timeout for TFTP based on binary exponential backoff (IETF RFC 1123 [B19], IETF RFC 2349 [B21]).

When the configuration file download has completed successfully, the CPE shall notify the MR-BS directly or through the R-CPE by transmitting the TFTP-CPLT message on the CPE’s primary management connection. Transmissions shall continue successfully until a TFTP-RSP message is received with response “OK” from the MR-BS (see Figure 48 and Figure 49) or the CPE terminates retransmission due to retry exhaustion.

Upon sending a REG-RSP, the MR-BS shall wait for a TFTP-CPLT. If the timer T13 (defined in Table 272) expires, the MR-BS shall restart the registration process (REG-REQ/RSP) with the CPE (see Figure 48). Note that the Timer T26 refers to the time waited for TFTP-RSP. If T26 expires, then TFTP-CPLT is attempted until the maximum number of retries is exhausted. Upon the exhaustion, the CPE shall be deregistered (i.e., forced to reinitialize MAC) by sending a DREG-REQ with Action Code set to 0x05 to force itself to reattempt system access or 0x04 to shut itself down (see Figure 49)

Figure 47  — Transferring operational parameters

Figure x – Transferring operational parameters through R-CPE

7.14.2.16 Establish dynamic service flows

After the transfer of operational parameters for the CPE, the BS shall send DSA-REQ messages (Table 64) to the CPE to set up pre-provisioned service flows belonging to the CPE. The CPE responds with DSA- RSP messages. This is described further in 7.18.7.1.

After the transfer of operational parameters for the CPE, the MR-BS shall send DSA-REQ messages (Table 64) to the CPE to set up pre-provisioned service flows belonging to the CPE. The CPE responds with DSA- RSP messages. This is described further in 7.18.7.1.

7.14.2.17 Neighboring network discovery (outer network discovery)

Outer neighboring network discovery is to discover other nearby WRANs for self-coexistence.

After a CPE has registered with a WRAN BS, it shall perform neighboring network discovery in order to identify other nearby WRANs and enable efficient self-coexistence, if the CPE has not already done so. The neighboring network discovery involves listening to the medium for CBP packets or BS SCH transmitted by other WRAN BSs. This network discovery mechanism is described in 7.20.1.3.

After a CPE has registered with a WRAN MR-BS, it shall perform neighboring network discovery in order to identify other nearby WRANs and enable efficient self-coexistence, if the CPE has not already done so. The neighboring network discovery involves listening to the medium for CBP packets or MR-BS transmitted by other WRAN MR-BSs. This network discovery mechanism is described in 7.20.1.3.

7.6.1.2    MAC subheaders and special payloads

Six types of subheaders may be present. The per-PDU subheaders (i.e., Bandwidth Request, Extended Bandwidth Request, Fragmentation/Packing, Grant Management) may be inserted in MAC PDUs immediately following the generic MAC header. If indicated, the Bandwidth Request subheader and Extended Bandwidth Request subheader shall always follow the Generic MAC header. In the upstream, if both the Grant Management subheader and Fragmentation/Packing subheader are indicated, the Grant Management subheader shall come first. If both the Grant Management subheader and Bandwidth Request subheader are indicated, the Grant Management subheader shall come first.

There can be more than one Fragmentation/Packing subheader in a MAC PDU, all configured for packing. There can only be one Fragmentation/Packing subheader in the MAC PDU if configured for fragmentation. The Fragmentation/Packing subheader may be inserted before each MAC SDU if so indicated by setting Bit 1 in the Type field of the generic MAC Header and by setting the “Purpose bit” in Table 6 to 1.

When present, per-PDU subheaders shall always precede the first per-SDU subheader.

7.6.1.2.1    Bandwidth Request subheader

Bandwidth Request subheaders are transmitted by the CPE to the BS to request additional bandwidth for a connection. They shall be sent in a PDU by itself or in a PDU with other subheaders and/or data. (See Table 5.)

Table 5 — Bandwidth Request subheader format

| Syntax | Size | Notes |
| --- | --- | --- |
| BW_ Request_Subheader_Format() { |  |  |
| Type | 1 bit | Indicates the type of the bandwidth request adjustment 0: incremental 1: aggregate |
| BR | 20 bits | The number of bytes of upstream bandwidth requested by the CPE. The request shall not include any PHY overhead. |
| } |  |  |

7.6.1.2.1a   Extended Bandwidth Request subheader

Extended Bandwidth Request subheaders are transmitted by the centralized scheduling R-CPE to the BS/MR-BS to request additional bandwidth for a CRZ connection. They shall be sent in a PDU by itself or in a PDU with other subheaders and/or data. (See Table 5a.)

Table 5 a— Extended Bandwidth Request subheader format

| Syntax | Size | Notes |
| --- | --- | --- |
| BW_ Request_Subheader_Format() { |  |  |
| Number of BR CPEs ; n | 8 bits | The number of CPEs, which require bandwidth request |
| For (i=1; i<= n; i++){ | Variable |  |
| CPE MAC address | 48 bits | MAC address of CPE, which require bandwidth request. This is used in initial ranging |
| SID | 13 bits | SID of CPE, which require bandwidth request |
| Directions | 2 bits | 0: upstream 1: downstream 2: both of upstream and downstream |
| If(Directions =0 or 2) { |  |  |
| uFID | 8 bits | Flow ID for upstream |
| Type | 1 bit | Indicates the type of the bandwidth request adjustment 0: incremental 1: aggregate |
| BR | 20 bits | The number of bytes of upstream bandwidth requested by the CPE. The request shall not include any PHY overhead. |
| } |  |  |
| Else if (Directions = 1 or 2) { |  |  |
| dFID | 8 bits | Flow ID for downstream |
| Type | 1 bit | Indicates the type of the bandwidth request adjustment 0: incremental 1: aggregate |
| BR | 20 bits | The number of bytes of upstream bandwidth requested by the CPE. The request shall not include any PHY overhead. |
| } |  |  |
| } |  |  |
| } |  |  |

7.7.26    Container Message

A container message is used to convey management messages from the centralized scheduling R-CPE to the BS/MR-BS. (See Table xx.)

Table xx — Container message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Container_Message_Format() { |  |  |
| Management Message Type = xx | 8 bits |  |
| Number of Contained Messages ; n | 8 bits | The number of contained messages |
| For (i=1; i<= n; i++){ | Variable |  |
| Message Type = xx | 8 bits | RNG-req, CBC-req, REG-req,etc |
| SID | 13 bits | SID of CPE, which require a request |
| Information elements (Ies) | Variable | Table 42 |
| } |  |  |
| } |  |  |

7.7.26.1    Container ACK Message

A container ACK message is used to acknowledgement for a container message sent to the centralized scheduling R-CPE from the BS/MR-BS. (See Table xx.)

Table xx — Container ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Container_ACK Message_Format() { |  |  |
| Management Message Type = xx | 8 bits |  |
| Number of Contained Messages ; n | 8 bits | The number of contained messages |
| For (i=1; i<= n; i++){ | Variable |  |
| Message Type = xx | 8 bits | RNG-req, CBC-req, REG-req,etc |
| SID | 13 bits | SID of CPE, which require a request |
| Confirmation Code | 2 bits | 0: success 1: unknown message 2: failed 3: reserved |
| } |  |  |
| } |  |  |

7.7.11.1  CBC-REQ

The CPE CBC-REQ shall be transmitted by the CPE during initialization. A CPE shall generate CBC-REQ messages in the form shown in Table 105. Basic Capability Requests contain those CPE capabilities Ies that are necessary for effective communication with the CPE during the remainder of the initialization protocols. Only the following parameters shall be included in the Basic Capabilities Request (see 7.14.2.9), namely  the  Physical  Parameters  Supported  and  the Bandwidth  Allocation Supported.  Capabilities  for Construction and Transmission of MAC PDUs may be supported if needed.

Table 105 — CBC-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CBC-REQ_Message_Format() { |  |  |
| Management Message Type = 19 | 8 bits |  |
| Information elements (Ies) | Variable | See 7.7.11.3.1 and 7.7.11.3.2 |
| Timeout (T18) |  | Timeout for wating for CBC-RSP |
| } |  |  |

7.7.11.3.2.2.1   CPE Demodulator

This field indicates the different demodulator options supported by a CPE for the downstream reception.

Table 109 — CPE Demodulator information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 3 | 1 | For a particular mode being represented, see the corresponding index in Table 27 (DIUC values) | CBC-REQ, CBC-RSP |

7.7.11.3.2.2.2   CPE Modulator

This field indicates the different modulator options supported by a CPE for upstream transmission.

Table 110 — CPE Modulator information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 4 | 1 | For a particular mode being represented, see the corresponding index in Table 36 (UIUC values) | CBC-REQ, CBC-RSP |

7.7.11.3.2.2.3   Centralized Scheduling R-CPE Demodulator

This field indicates the different demodulator options supported by a centralized scheduling R-CPE for the downstream reception.

Table 109a — Centralized Scheduling R-CPE Demodulator information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| xx | 1 | For a particular mode being represented, see the corresponding index in Table 27 (DIUC values) | CBC-RSP |

7.7.11.3.2.2.4   Centralized Scheduling R-CPE Modulator

This field indicates the different modulator options supported by a centralized scheduling R-CPE for upstream transmission.

Table 110a — Centralized Scheduling R-CPE Modulator information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| xx | 1 | For a particular mode being represented, see the corresponding index in Table 36 (UIUC values) | CBC-RSP |

7.7.11.3.4   Relay CPE Mode

Table xx — Relay CPE Mode information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| xx | 1 | 0: No support Relay 1: Centralized Scheduling R-CPE Support 2: Distributed Scheduling R-CPE Support | CBC-REQ |

7.7.7.1    REG-REQ

The format of a REG-REQ message is shown in Table 45. This message shall be transmitted by CPEs at initialization phase.

The FID field carried in the MAC header of the PDU where this message is transmitted shall be the primary management FID for this CPE, which is assigned during the RNG-CMD message.

Table 45 — REG-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| REG-REQ_Message_Format() { |  |  |
| Management Message Type = 6 | 8 bits |  |
| Information elements (Ies) | Variable | 7.7.7.3 |
| Timeout (T13) |  | Timeout for wating for REG-RSP |
| } |  |  |

7.7.7.3    REG-REQ/RSP information elements

7.7.7.3.6    Local SID Group

Table xx — Local SID Group information element

| Syntax | Size | Notes | Scope |
| --- | --- | --- | --- |
| Local SID Group_IE(){ |  |  | REG-RSP |
| Element ID | 8 bits | 12 |  |
| Number of SIDs | 8 bits | Total number of SIDs assigned for a distributed scheduling R-CPE |  |
| SIDs | 13bits | Start SID; A group of SIDs will be allocated from SID. |  |
| } |  |  |  |

7.7.7.3.4.13 CPE Operational Capability

This field allows the CPE to signal to the BS that it is to be operated as a Fixed or Portable terminal.

Table 62 — CPE Operational Capability information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 16 | 1 | 0x00- Fixed (no relay) 0x01: Portable (no relay) 0x02: Centralized scheduling R-CPE (fixed only) 0x03: Distributed scheduling R-CPE (fixed) 0x04: Distributed scheduling R-CPE (portable Mode II) 0x04–0xFF: Reserved | REG- REQ/RSP |

7.7.25    Local Cell Update

7.7.25.1    Local Cell Update REQ

The format of a Local Cell Update request message is shown in Table xx. This message shall be transmitted by a distributed scheduling R-CPEs to the MR-BS at the change of local cell information.

Table xx — Local Cell Update REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Container_Message _Format() { |  |  |
| Management Message Type = xx | 8 bits |  |
| Number of Contained Messages ; n | 8 bits | The number of contained messages |
| For (i=1; i<= n; i++){ | Variable |  |
| Message Type = xx | 8 bits | Local Cell Update REQ |
| SID | 13 bits | SID of CPE, which require local cell update request |
| Information elements (IEs) | Variable | 7.7.7.3 |
| } |  |  |
| } |  |  |

7.7.25.1    Local Cell Update RSP

The format of a Local Cell Update response message is shown in Table xx. This message shall be transmitted by an MR-BS to a distributed scheduling R-CPEs for the confirmation of local cell update request.

Table xx — Local Cell Update RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| LocalCell_Update_RSP_Format() { |  |  |
| Management Message Type = xx | 8 bits |  |
| Number of CPEs ; n | 8 bits | The number of CPEs, which update information in a local cell |
| For (i=1; i<= n; i++){ | Variable |  |
| SID | 13 bits | SID |
| Information elements (IEs) | Variable | 7.7.7.3 |
| } |  |  |
| } |  |  |

7.7.25.1    Local Cell Update information elements

7.7.25.1.1    De-registration

Table xx —De-registration information element

| Element ID | Length | Value | Scope |
| --- | --- | --- | --- |
| xx | 1 bit | 0; De-registerd |  |